#include "mbed.h"

#include <cstdlib>

#include <cmath>

#include "stm32f4xx_hal.h"

// ================= MOTOR & CAN CONFIG =================

#define MOTOR_ID_1      11

#define MOTOR_ID_2      12

#define MOTOR_ID_3      13

#define MOTOR_ID_4      14

#define CAN_BAUD        1000000

#define REPLY_ID_1      (MOTOR_ID_1 + 0x100)

#define REPLY_ID_2      (MOTOR_ID_2 + 0x100)

#define REPLY_ID_3      (MOTOR_ID_3 + 0x100)

#define REPLY_ID_4      (MOTOR_ID_4 + 0x100)

DigitalOut led(LED1);

Serial pc(USBTX, USBRX, 115200);

CAN can(PA_11, PA_12, CAN_BAUD);

// LCD Config

#define I2C_SDA PB_9

#define I2C_SCL PB_8

I2C i2c_lcd(I2C_SDA, I2C_SCL);

uint8_t lcd_addr = 0x27;

#define LCD_BACKLIGHT 0x08

#define LCD_EN 0x04

#define LCD_RS 0x01

#define LCD_ADDR_1 0x27

#define LCD_ADDR_2 0x3F

PwmOut     m1_pwm(PB_14);

DigitalOut m1_dira(PC_8);

DigitalOut m1_dirb(PC_6);

PwmOut     m2_pwm(PB_13);

DigitalOut m2_dira(PB_15);

DigitalOut m2_dirb(PB_1);

// M3

DigitalOut m3_ina(PA_8);

DigitalOut m3_inb(PB_10);

PwmOut     m3_pwm(PA_9);

// M4

DigitalOut m4_ina(PA_13);

DigitalOut m4_inb(PA_14);

PwmOut     m4_pwm(PB_2);

// --- CONFIGURATION ---

const float MAX_VEL = 3.0f;

const float LINEAR_SPEED = MAX_VEL;

const float DISTANCE_1 = 7.0f;

const float DISTANCE_2 = 2.0f;

const float DISTANCE_3 = 11.0f;

const float TIME_1 = DISTANCE_1 / LINEAR_SPEED;

const float TIME_2 = DISTANCE_2 / LINEAR_SPEED;

const float TIME_3 = DISTANCE_3 / LINEAR_SPEED;

const float TIME_4 = 4.0;

// Ladder Configuration

const float LADDER_SPEED = 0.5f;

const float LADDER_TIME_20CM = 4.0f;   // Time to move 20cm

const float STABILIZE_TIME = 4.0f;     // Time to move forward

const float STABILIZE_VEL = 5.0f;

const float STABILIZE_BACK = 0.1f;

// Directions for M3/M4

const int DIR_UP_M3    = -1;

const int DIR_UP_M4    = 1;

const int DIR_DOWN_M3  = 1;

const int DIR_DOWN_M4  = -1;

const int NUM_MOTORS = 5;

const int NUM_PATTERNS = 7;

// ================= STATE ENUM =================

enum State {

DISABLED = 0,

MOVE_BACK_1 = 1,

CYL1_CYCLE = 2,

MOVE_BACK_2 = 3,

CYL2_CYCLE = 4,

MOVE_FOR_1 = 5,

MOVE_DC_TO_RELEASE = 6,

TURN_TO_LADDER = 7,

FIRST_LADDER = 8,

FINISHED = 9

};

volatile State current_state = DISABLED;

volatile int rx_count[4] = {0, 0, 0, 0};

CANMessage tx_msg, rx_msg;

float velocities[4] = {0.0f, 0.0f, 0.0f, 0.0f};

const float directions[NUM_PATTERNS][NUM_MOTORS] = {

{1.0f, -1.0f, -1.0f, 1.0f},   // 0:

{1.0f, 1.0f, -1.0f, -1.0f},   // 1: hoish

{-1.0f, 1.0f, 1.0f, -1.0f},   // 2:

{-1.0f, -1.0f, 1.0f, 1.0f},   // 3: Uragsh

{0.0f, 1.0f, 0.0f, -1.0f},    // 4: Diagonal

{-1.0f, -1.0f, -1.0f, -1.0f},  // 5: Turn Arc

{1.0f, 0.0f, -1.0f, 0.0f},    // 6: Diagonal

};

// ================= SENSORS =================

DigitalIn sens_start(PC_11, PullUp);

DigitalIn sens_motor_start(PA_5, PullUp);

DigitalIn sens_field_select(PC_3, PullUp);

DigitalIn sens_motor_stop(PB_0, PullUp);

DigitalIn box_sensor_1(PA_4, PullUp);

DigitalIn box_sensor_2(PC_2, PullUp);

DigitalIn box_sensor_3(PC_5, PullUp);

// Cylinders

DigitalOut cyl1(PC_7);

DigitalOut cyl2(PB_6);

DigitalOut cyl3(PA_7);

DigitalOut cyl4(PA_6);

DigitalOut* relays[4] = {&cyl1, &cyl2, &cyl3, &cyl4};

const bool ACTIVE_LOW = false;

#define RELAY_ON  1

#define RELAY_OFF 0

// ================= FUNCTION PROTOTYPES =================

void send_cmd(uint32_t id, float vel, float kp = 0, float kd = 1.0f);

void enter_mode(uint32_t id);

void safe_stop_all();

void on_rx();

void cyl_stop_all();

int read_active_low(DigitalIn& s);

int is_pressed(DigitalIn& s);

void set_motor(int dir, float duty);

void stopAllClimb();

// Movement Functions

void moveM3M4Up();

void moveM3Down();

void moveM4Down();

void moveCANForward();

void moveCANBackward();

// LCD Helpers

void wait_us_lcd(int us);

void write_i2c(uint8_t data);

void pulse_enable(uint8_t data);

void send_nibble(uint8_t nibble, uint8_t mode);

void send_command(uint8_t cmd);

void send_char(char c);

void lcd_print(const char* str);

void lcd_print_num(float num, int decimals);

void lcd_clear();

void lcd_set_cursor(uint8_t row, uint8_t col);

bool detect_lcd();

void lcd_init();

// ================= MAIN =================

int main() {

__HAL_RCC_PWR_CLK_ENABLE();

HAL_PWR_EnableBkUpAccess();

__HAL_RCC_LSE_CONFIG(RCC_LSE_OFF);

while(__HAL_RCC_GET_FLAG(RCC_FLAG_LSERDY) != RESET);

// Init Relays Off

for (int i = 0; i < 4; i++) {

relays[i]->write(ACTIVE_LOW ? 1 : 0);

}

// Init Climbing Motors

m3_pwm.period_ms(2); m3_pwm = 0.0f;

m4_pwm.period_ms(2); m4_pwm = 0.0f;

stopAllClimb();

// Init Grip Motor

m1_pwm.period(0.0001f);

m1_pwm = 0.0f;

m1_dira = 0; m1_dirb = 0;

wait_ms(500);

pc.printf("\r\n=== System Boot ===\r\n");

pc.printf("Dir M3 Up: %d, Dir M4 Up: %d\r\n", DIR_UP_M3, DIR_UP_M4);

if (!detect_lcd()) {

led = 1;

while(true) { led = !led; thread_sleep_for(200); }

}

lcd_init();

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("ABU Robocon 2026");

lcd_set_cursor(1, 0); lcd_print("Init CAN...");

int freq = can.frequency(CAN_BAUD);

if(freq == 0) {

lcd_set_cursor(1, 0); lcd_print("CAN FAILED!   ");

while(1) { led = !led; thread_sleep_for(100); }

}

can.attach(&on_rx, CAN::RxIrq);

can.filter(0x100, 0x700, CANStandard, 0);

enter_mode(MOTOR_ID_1); enter_mode(MOTOR_ID_2);

enter_mode(MOTOR_ID_3); enter_mode(MOTOR_ID_4);

safe_stop_all();

lcd_set_cursor(1, 0); lcd_print("Ready. Wait St");

Timer state_timer, debug_timer;

State last_state = DISABLED;

int ladder_sub_step = 0;

while(1) {

int st = read_active_low(sens_start);

int field_raw = sens_field_select.read();

const char* field_str = (field_raw == 0) ? "RED" : "BLU";

if (current_state != last_state) {

state_timer.reset();

state_timer.start();

debug_timer.reset();

last_state = current_state;

pc.printf("State: %d\r\n", current_state);

if (current_state == FIRST_LADDER) {

ladder_sub_step = 0;

pc.printf("Starting Custom Ladder Sequence...\r\n");

}

if(current_state != MOVE_BACK_1 && current_state != MOVE_BACK_2 && current_state != MOVE_FOR_1 && current_state != TURN_TO_LADDER && current_state != FIRST_LADDER) {

safe_stop_all();

set_motor(0, 0);

stopAllClimb();

}

}

// ================= STATE MACHINE =================

if (current_state == DISABLED) {

safe_stop_all(); cyl_stop_all(); set_motor(0, 0); stopAllClimb();

if (read_active_low(sens_start) == 0) {

thread_sleep_for(50);

if(read_active_low(sens_start) == 0) {

current_state = MOVE_BACK_1;

lcd_clear(); lcd_set_cursor(0, 0); lcd_print("1. MOVE BACK 1");

}

} else {

lcd_set_cursor(0, 0); lcd_print("St:"); lcd_print(st ? "1" : "0");

lcd_set_cursor(1, 0); lcd_print("Fld:"); lcd_print(field_str); lcd_print(" READY");

static Timer blink;

if(blink.read() > 0.5f) { blink.reset(); led = !led; }

}

}

else if (current_state == MOVE_BACK_1) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_1) { safe_stop_all(); wait_ms(300); current_state = CYL2_CYCLE; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (current_state == CYL2_CYCLE) {

cyl2.write(RELAY_ON); wait_ms(2000); cyl1.write(RELAY_ON); wait_ms(300);

current_state = MOVE_FOR_1; thread_sleep_for(50);

}

else if (current_state == MOVE_FOR_1) {

float t = state_timer.read();

static int turn_phase = 0;

if (turn_phase == 0) {

if (t < TIME_1*2) { for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[4][i]; // Diagonal

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);  send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);  send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();  lcd_set_cursor(1, 0); lcd_print("Turn Phase 1");

}

led = 1;

} else {  turn_phase = 1; state_timer.reset();

pc.printf("Switching to Turn Phase 2...\r\n");

}

}

else if (turn_phase == 1) {

if (t < TIME_1) {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * -1 * directions[5][i]; // Arc

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);  send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);  send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();

lcd_set_cursor(1, 0); lcd_print("Turn Phase 2");

}

led = 1;

} else {

safe_stop_all(); wait_ms(300); lcd_clear();

lcd_set_cursor(0, 0); lcd_print("7. GRIP RELEASE");

current_state = MOVE_DC_TO_RELEASE;

turn_phase = 0;

cyl2.write(RELAY_ON); wait_ms(2000);

}

}

thread_sleep_for(20);

}

else if (current_state == MOVE_DC_TO_RELEASE) {

int release_hit = is_pressed(sens_motor_stop);

set_motor(1, 0.3f);

lcd_set_cursor(0, 0); lcd_print("DC: Moving");

if (release_hit) {

m1_pwm = 0.0f; m1_dira = 1; m1_dirb = 1; wait_ms(50); m1_dira = 0; m1_dirb = 0; wait_ms(100);

current_state = TURN_TO_LADDER; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("RELEASE HIT");

}

if (state_timer.read() > 3.0f) { set_motor(0, 0); current_state = TURN_TO_LADDER; wait_ms(10000);}

}

else if (current_state == TURN_TO_LADDER) {

float t = state_timer.read();

static int turn_phase = 0;

if (turn_phase == 0) {

if (t < TIME_4) { for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * -1 * directions[5][i]; // Diagonal

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);  send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);  send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();  lcd_set_cursor(1, 0); lcd_print("Turn Phase 1");

}

led = 1;

} else {  turn_phase = 1; state_timer.reset();

pc.printf("Switching to Turn Phase 2...\r\n");

}

}

else if (turn_phase == 1) {

if (t < TIME_2*2) { for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[1][i]; // Uragsh

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);  send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);  send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();  lcd_set_cursor(1, 0); lcd_print("Turn Phase 1");

}

led = 1;

} else {  turn_phase = 2; state_timer.reset();

pc.printf("Switching to Turn Phase 2...\r\n");

}

}

else if (turn_phase == 2) {

if (t < TIME_3) {   for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * -1 * directions[6][i]; // Arc

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);  send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);  send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();

lcd_set_cursor(1, 0); lcd_print("Turn Phase 2");

}

led = 1;

} else {

safe_stop_all(); wait_ms(300); lcd_clear();

lcd_set_cursor(0, 0); lcd_print("7. GRIP RELEASE");

current_state = FIRST_LADDER;

turn_phase = 0;

}

}

thread_sleep_for(20);

}

// ================= CUSTOM LADDER LOGIC =================

else if (current_state == FIRST_LADDER) {

float t = state_timer.read();

if(debug_timer.read() > 1.0f) {

debug_timer.reset();

pc.printf("Step: %d, Time: %.2f\r\n", ladder_sub_step, t);

lcd_set_cursor(1, 0); lcd_print("Step: "); lcd_print_num(ladder_sub_step, 0);

}

// Step 0: M3 UP + M4 UP (Simultaneous)

if (ladder_sub_step == 0) {

stopAllClimb(); safe_stop_all();

if (t < LADDER_TIME_20CM) {

moveM3M4Up();

if(debug_timer.read() > 0.5f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("M3+M4 UP"); }

thread_sleep_for(20);

} else {

stopAllClimb(); safe_stop_all();

pc.printf("Step 0 Done. Stabilizing...\r\n");

wait_ms(100);

ladder_sub_step = 1;

state_timer.reset();

}

}

// Step 1: CAN Forward

else if (ladder_sub_step == 1) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); ladder_sub_step = 2; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

// Step 2: M3 DOWN

else if (ladder_sub_step == 2) {

stopAllClimb(); safe_stop_all();

if (t < LADDER_TIME_20CM) {

moveM3Down();

if(debug_timer.read() > 0.6f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("M3 DOWN"); }

thread_sleep_for(20);

} else {

stopAllClimb(); safe_stop_all();

pc.printf("Step 2 Done. Stabilizing...\r\n");

wait_ms(100);

ladder_sub_step = 3;

state_timer.reset();

}

}

// Step 3: CAN Forward

else if (ladder_sub_step == 3) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); ladder_sub_step = 4; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (ladder_sub_step == 4) {

stopAllClimb(); safe_stop_all();

float t = state_timer.read();

if (t >= TIME_4) { safe_stop_all(); wait_ms(300); ladder_sub_step = 6; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("2. CYL1 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[1][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.1f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

ladder_sub_step = 5;

}

thread_sleep_for(20);

}

// Step 4: M4 DOWN

else if (ladder_sub_step == 5) {

stopAllClimb(); safe_stop_all();

if (t < LADDER_TIME_20CM) {

moveM4Down();

if(debug_timer.read() > 0.6f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("M4 DOWN"); }

thread_sleep_for(20);

} else {

stopAllClimb(); safe_stop_all();

pc.printf("Step 4 Done. Sequence Complete!\r\n");

wait_ms(500);

ladder_sub_step = 6;

state_timer.reset();

}

}

// Step 3: CAN Forward

else if (ladder_sub_step == 6) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); ladder_sub_step = 7; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (ladder_sub_step == 7) {

stopAllClimb(); safe_stop_all();

if (t < TIME_4) {

moveCANBackward();

if(debug_timer.read() > 0.1f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("CAN FWD 1"); }

thread_sleep_for(20);

} else {

safe_stop_all();

pc.printf("Step 1 Done. M3 Down...\r\n");

wait_ms(100);

ladder_sub_step = 8;

state_timer.reset();

}

}

if (ladder_sub_step == 8) {

stopAllClimb(); safe_stop_all();

if (t < LADDER_TIME_20CM) {

moveM3M4Up();

if(debug_timer.read() > 0.5f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("M3+M4 UP"); }

thread_sleep_for(20);

} else {

stopAllClimb(); safe_stop_all();

pc.printf("Step 0 Done. Stabilizing...\r\n");

wait_ms(100);

ladder_sub_step = 9;

state_timer.reset();

}

}

else if (ladder_sub_step == 9) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); ladder_sub_step = 10; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (ladder_sub_step == 10) {

stopAllClimb(); safe_stop_all();

if (t < TIME_4) {

moveCANBackward();

if(debug_timer.read() > 0.1f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("CAN FWD 1"); }

thread_sleep_for(20);

} else {

safe_stop_all();

pc.printf("Step 1 Done. M3 Down...\r\n");

wait_ms(100);

ladder_sub_step = 11;

state_timer.reset();

}

}

// Step 2: M3 DOWN

else if (ladder_sub_step == 11) {

stopAllClimb(); safe_stop_all();

if (t < LADDER_TIME_20CM) {

moveM3Down();

if(debug_timer.read() > 0.6f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("M3 DOWN"); }

thread_sleep_for(20);

} else {

stopAllClimb(); safe_stop_all();

pc.printf("Step 2 Done. Stabilizing...\r\n");

wait_ms(100);

ladder_sub_step = 12;

state_timer.reset();

}

}

else if (ladder_sub_step == 12) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); ladder_sub_step = 13; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (ladder_sub_step == 13) {

stopAllClimb(); safe_stop_all();

if (t < TIME_4) {

moveCANBackward();

if(debug_timer.read() > 0.1f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("CAN FWD 1"); }

thread_sleep_for(20);

} else {

safe_stop_all();

pc.printf("Step 1 Done. M3 Down...\r\n");

wait_ms(100);

ladder_sub_step = 14;

state_timer.reset();

}

}

else if (ladder_sub_step == 14) {

stopAllClimb(); safe_stop_all();

if (t < LADDER_TIME_20CM) {

moveM4Down();

if(debug_timer.read() > 0.6f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("M4 DOWN"); }

thread_sleep_for(20);

} else {

stopAllClimb(); safe_stop_all();

pc.printf("Step 4 Done. Sequence Complete!\r\n");

wait_ms(500);

ladder_sub_step = 15;

state_timer.reset();

}

}

else if (ladder_sub_step == 15) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); current_state = FINISHED; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

}

// 9. FINISHED

else if (current_state == FINISHED) {

safe_stop_all();

cyl_stop_all();

set_motor(0, 0);

stopAllClimb();

led = 0;

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("SEQUENCE DONE");

lcd_set_cursor(1, 0); lcd_print("STOPPED");

thread_sleep_for(100);

}

thread_sleep_for(20);

}

}

// ================= MOVEMENT FUNCTIONS =================

// 1. BOTH M3 and M4 UP Together

void moveM3M4Up() {

if (DIR_UP_M3 > 0) { m3_ina = 1; m3_inb = 0; }

else { m3_ina = 0; m3_inb = 1; }

m3_pwm = 0.4f;

if (DIR_UP_M4 > 0) { m4_ina = 1; m4_inb = 0; }

else { m4_ina = 0; m4_inb = 1; }

m4_pwm = 0.5f;

}

// 2. M3 DOWN Only

void moveM3Down() {

m4_pwm = 0.0f; m4_ina = 0; m4_inb = 0;

wait_us(50);

if (DIR_DOWN_M3 > 0) { m3_ina = 1; m3_inb = 0; }

else { m3_ina = 0; m3_inb = 1; }

m3_pwm = 0.42f;

}

// 3. M4 DOWN Only

void moveM4Down() {

m3_pwm = 0.0f; m3_ina = 0; m3_inb = 0;

wait_us(50);

if (DIR_DOWN_M4 > 0) { m4_ina = 1; m4_inb = 0; }

else { m4_ina = 0; m4_inb = 1; }

m4_pwm = 0.5f;

}

// 4. CAN Forward

void moveCANForward() {

m3_pwm = 0.0f; m3_ina = 0; m3_inb = 0;

m4_pwm = 0.0f; m4_ina = 0; m4_inb = 0;

for(int i=0; i<NUM_MOTORS; i++) {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);

send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);

send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

}

}

void moveCANBackward() {

m3_pwm = 0.0f; m3_ina = 0; m3_inb = 0;

m4_pwm = 0.0f; m4_ina = 0; m4_inb = 0;

for(int i=0; i<NUM_MOTORS; i++) {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[1][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);

send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);

send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

}

}

void stopAllClimb() {

m3_pwm = 0.0f; m3_ina = 0; m3_inb = 0;

m4_pwm = 0.0f; m4_ina = 0; m4_inb = 0;

wait_ms(50);

}

void cyl_stop_all() {

cyl1.write(RELAY_OFF); cyl2.write(RELAY_OFF); cyl3.write(RELAY_OFF); cyl4.write(RELAY_OFF);

}

void set_motor(int dir, float duty) {

if (duty > 1.0f) duty = 1.0f;

if (duty < 0.0f) duty = 0.0f;

if (dir == 1) { m1_dira = 0; m1_dirb = 1; m1_pwm = duty; }

else if (dir == -1) { m1_dira = 1; m1_dirb = 0; m1_pwm = duty; }

else { m1_dira = 0; m1_dirb = 0; m1_pwm = 0.0f; }

}

int read_active_low(DigitalIn& s) { return (s.read() == 0) ? 1 : 0; }

int is_pressed(DigitalIn& s) { return (s.read() == 0) ? 1 : 0; }

void send_cmd(uint32_t id, float vel, float kp, float kd) {

if(vel < -30.0f) vel = -30.0f; if(vel > 30.0f) vel = 30.0f;

int p_int = 2048;

int v_int = (int)((vel + 30.0f) * 4095.0f / 60.0f);

int kp_int = (int)(kp * 4095.0f / 500.0f);

int kd_int = (int)(kd * 4095.0f / 5.0f);

int ff_int = 2048;

tx_msg.data[0] = (p_int >> 8) & 0xFF; tx_msg.data[1] = p_int & 0xFF;

tx_msg.data[2] = (v_int >> 4) & 0xFF; tx_msg.data[3] = ((v_int & 0x0F) << 4) | ((kp_int >> 8) & 0x0F);

tx_msg.data[4] = kp_int & 0xFF; tx_msg.data[5] = (kd_int >> 4) & 0xFF;

tx_msg.data[6] = ((kd_int & 0x0F) << 4) | ((ff_int >> 8) & 0x0F); tx_msg.data[7] = ff_int & 0xFF;

tx_msg.id = id; tx_msg.type = CANData; tx_msg.format = CANStandard; tx_msg.len = 8;

can.write(tx_msg);

}

void enter_mode(uint32_t id) {

tx_msg.id = id; tx_msg.type = CANData; tx_msg.format = CANStandard; tx_msg.len = 8;

for(int i = 0; i < 7; i++) tx_msg.data[i] = 0xFF; tx_msg.data[7] = 0xFC;

can.write(tx_msg); wait_ms(50);

}

void safe_stop_all() {

for(int k=0; k<3; k++) {

for(int i=0; i<NUM_MOTORS; i++) {

send_cmd(MOTOR_ID_1+i, 0.0f, 0.0f, 10.0f); wait_us(200);

}

wait_ms(10);

}

}

void on_rx() {

if(can.read(rx_msg)) {

if(rx_msg.id == REPLY_ID_1 && rx_msg.len >= 6) rx_count[0]++;

else if(rx_msg.id == REPLY_ID_2 && rx_msg.len >= 6) rx_count[1]++;

else if(rx_msg.id == REPLY_ID_3 && rx_msg.len >= 6) rx_count[2]++;

else if(rx_msg.id == REPLY_ID_4 && rx_msg.len >= 6) rx_count[3]++;

}

}

// ================= LCD FUNCTIONS =================

void wait_us_lcd(int us) { thread_sleep_for(us / 1000); }

void write_i2c(uint8_t data) { char cmd = data | LCD_BACKLIGHT; i2c_lcd.write(lcd_addr << 1, &cmd, 1); }

void pulse_enable(uint8_t data) { write_i2c(data | LCD_EN); wait_us_lcd(100); write_i2c(data & ~LCD_EN); wait_us_lcd(100); }

void send_nibble(uint8_t nibble, uint8_t mode) { uint8_t data = (nibble & 0xF0) | mode | LCD_BACKLIGHT; pulse_enable(data); }

void send_command(uint8_t cmd) { send_nibble(cmd & 0xF0, 0); send_nibble((cmd << 4) & 0xF0, 0); thread_sleep_for(2); }

void send_char(char c) { send_nibble(c & 0xF0, LCD_RS); send_nibble((c << 4) & 0xF0, LCD_RS); }

void lcd_print(const char* str) { while (*str) send_char(*str++); }

void lcd_print_num(float num, int decimals) { char buf[16]; sprintf(buf, "%.*f", decimals, num); lcd_print(buf); }

void lcd_clear() { send_command(0x01); thread_sleep_for(2); }

void lcd_set_cursor(uint8_t row, uint8_t col) { uint8_t addr = (row == 0) ? 0x80 + col : 0xC0 + col; send_command(addr); }

bool detect_lcd() {

char dummy = 0;

if (i2c_lcd.write(LCD_ADDR_1 << 1, &dummy, 0) == 0) { lcd_addr = LCD_ADDR_1; return true; }

if (i2c_lcd.write(LCD_ADDR_2 << 1, &dummy, 0) == 0) { lcd_addr = LCD_ADDR_2; return true; }

return false;

}

void lcd_init() {

thread_sleep_for(50);

write_i2c(0x03 | LCD_BACKLIGHT); wait_us_lcd(4500);

pulse_enable(0x03 | LCD_BACKLIGHT); wait_us_lcd(4500);

pulse_enable(0x03 | LCD_BACKLIGHT); wait_us_lcd(150);

pulse_enable(0x02 | LCD_BACKLIGHT);

send_command(0x28); send_command(0x0C); send_command(0x06); send_command(0x01);

thread_sleep_for(2);

}

// GIM8008 Quad Motor + Cylinders + Gripper Motor - Full Sequence

// NUCLEO_F446RE | MCP2551 | CAN 1Mbps | LCD I2C | Limit Switches

// Includes: Quad CAN Motors, 2 Solenoid Cylinders, 1 VNH5019 Gripper Motor

#include "mbed.h"

#include <cstdlib>

#include <cmath>

#include "stm32f4xx_hal.h"

// ================= MOTOR & CAN CONFIG =================

#define MOTOR_ID_1      11

#define MOTOR_ID_2      12

#define MOTOR_ID_3      13

#define MOTOR_ID_4      14

#define CAN_BAUD        1000000

#define REPLY_ID_1      (MOTOR_ID_1 + 0x100)

#define REPLY_ID_2      (MOTOR_ID_2 + 0x100)

#define REPLY_ID_3      (MOTOR_ID_3 + 0x100)

#define REPLY_ID_4      (MOTOR_ID_4 + 0x100)

DigitalOut led(LED1);

Serial pc(USBTX, USBRX, 115200);

CAN can(PA_11, PA_12, CAN_BAUD);

// ================= GRIPPER MOTOR HARDWARE (VNH5019) =================

// Limits: Active Low (0 = Pressed)

DigitalIn lim_grip(PC_4, PullUp);      // START/HOME Position

DigitalIn lim_release(PC_1, PullUp);   // END/FAR Position

DigitalIn action_btn(PA_10, PullUp);   // Single Control Button

// Motor Driver Pins

PwmOut     pwm(PB_14);

DigitalOut dira(PC_8);

DigitalOut dirb(PC_6);

// Gripper Config

const float MOVE_DUTY = 0.4f;      // Speed for moving between limits

const float SEARCH_DUTY = 0.2f;    // Slow speed for initial finding

// ================= QUAD MOTOR CONFIG =================

const float MAX_VEL = 3.0f;

const float WHEEL_RADIUS = 0.065f;

const float DISTANCE_1 = 0.80f;     // Updated to 0.8m as per LCD text

const float DISTANCE_2 = 0.50f;     // Updated to 0.5m

const float LINEAR_SPEED = MAX_VEL * WHEEL_RADIUS;

const float TIME_1 = DISTANCE_1 / LINEAR_SPEED;

const float TIME_2 = DISTANCE_2 / LINEAR_SPEED;

const int NUM_MOTORS = 4;

const int NUM_PATTERNS = 4;

// Fixed Enum Definition

enum State {

DISABLED = 0,

MOVE_BACK_1 = 1,

CYL1_CYCLE = 2,

MOVE_BACK_2 = 3,

CYL2_CYCLE = 4,

FINISHED = 5

};

volatile State current_state = DISABLED;

volatile int rx_count[4] = {0, 0, 0, 0};

CANMessage tx_msg, rx_msg;

float velocities[4] = {0.0f, 0.0f, 0.0f, 0.0f};

const int directions[NUM_PATTERNS][NUM_MOTORS] = {

{1, -1, -1, 1},   // 0: Uragsh (Fwd)

{1, 1, -1, -1},   // 1: Baruun (Right)

{-1, 1, 1, -1},   // 2: Hoish (Back)

{-1, -1, 1, 1}    // 3: Zvvn (Left)

};

// ================= SENSOR & LCD CONFIG =================

#define I2C_SDA PB_9

#define I2C_SCL PB_8

#define LCD_ADDR_1 0x27

#define LCD_ADDR_2 0x3F

DigitalIn sens_start(PC_0, PullUp);

DigitalIn sens_motor_start(PC_1, PullUp);

DigitalIn sens_field_select(PC_3, PullUp);

DigitalIn sens_motor_stop(PC_4, PullUp);

I2C i2c_lcd(I2C_SDA, I2C_SCL);

uint8_t lcd_addr = 0;

#define LCD_BACKLIGHT 0x08

#define LCD_EN 0x04

#define LCD_RS 0x01

// ================= CYLINDER HARDWARE =================

DigitalOut cyl1_ina(PA_9);

DigitalOut cyl1_inb(PC_7);

PwmOut     cyl1_pwm(PA_8); // Changed to PA_8 to avoid conflict with INA on PA_9 if needed, adjust if your HW is specific

DigitalOut cyl2_ina(PA_13);

DigitalOut cyl2_inb(PA_14);

PwmOut     cyl2_pwm(PB_2);

// ================= FUNCTION PROTOTYPES =================

void send_cmd(uint32_t id, float vel, float kp = 0, float kd = 1.0f);

void enter_mode(uint32_t id);

void safe_stop_all();

void on_rx();

void cyl_stop_all();

int read_active_low(DigitalIn& s);

void set_gripper_motor(int dir, float duty); // New function

// LCD Functions

void wait_us_lcd(int us);

void write_i2c(uint8_t data);

void pulse_enable(uint8_t data);

void send_nibble(uint8_t nibble, uint8_t mode);

void send_command(uint8_t cmd);

void send_char(char c);

void lcd_print(const char* str);

void lcd_print_num(float num, int decimals);

void lcd_clear();

void lcd_set_cursor(uint8_t row, uint8_t col);

bool detect_lcd();

void lcd_init();

// ================= MAIN =================

int main() {

// 1. Critical: Disable LSE for PB_8/PB_9 I2C

__HAL_RCC_PWR_CLK_ENABLE();

HAL_PWR_EnableBkUpAccess();

__HAL_RCC_LSE_CONFIG(RCC_LSE_OFF);

while(__HAL_RCC_GET_FLAG(RCC_FLAG_LSERDY) != RESET);

// 2. Init Cylinders Safe State

cyl1_ina = 0; cyl1_inb = 0; cyl1_pwm = 0.0f;

cyl2_ina = 0; cyl2_inb = 0; cyl2_pwm = 0.0f;

// 3. Init Gripper Motor Safe State

dira = 0; dirb = 0; pwm = 0.0f;

pc.printf("\r\n=== System Boot ===\r\n");

// 4. Init LCD

if (!detect_lcd()) {

led = 1;

while(true) { led = !led; thread_sleep_for(200); }

}

lcd_init();

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("ABU Robocon 2026");

lcd_set_cursor(1, 0); lcd_print("Init CAN...");

// 5. Initialize CAN

int freq = can.frequency(CAN_BAUD);

if(freq == 0) {

lcd_set_cursor(1, 0); lcd_print("CAN FAILED!   ");

while(1) { led = !led; thread_sleep_for(100); }

}

can.attach(&on_rx, CAN::RxIrq);

can.filter(0x100, 0x700, CANStandard, 0);

enter_mode(MOTOR_ID_1); enter_mode(MOTOR_ID_2);

enter_mode(MOTOR_ID_3); enter_mode(MOTOR_ID_4);

safe_stop_all();

lcd_set_cursor(1, 0); lcd_print("Ready. Wait St");

Timer state_timer, debug_timer, btn_timer;

State last_state = DISABLED;

int cyl_step = 0;

// Gripper State Variables

bool btn_last = 1;

bool gripper_moving = false;

int gripper_dir = 0; // 1=Out, -1=In

while(1) {

int st = read_active_low(sens_start);

int ms = read_active_low(sens_motor_start);

int sp = read_active_low(sens_motor_stop);

int field_raw = sens_field_select.read();

const char* field_str = (field_raw == 0) ? "RED" : "BLU";

// Read Limits

int grip_hit = read_active_low(lim_grip);

int release_hit = read_active_low(lim_release);

int btn_now = read_active_low(action_btn);

// Detect Main State Change

if (current_state != last_state) {

state_timer.reset();

state_timer.start();

last_state = current_state;

cyl_step = 0;

pc.printf("Main State: %d\r\n", current_state);

if(current_state != MOVE_BACK_1 && current_state != MOVE_BACK_2) {

safe_stop_all();

}

}

// --- GRIPPER MOTOR LOGIC (Independent of Main State) ---

// Runs continuously to allow gripping anytime

if (btn_now == 0 && btn_last == 1) { // Button Pressed (Edge Detect)

btn_timer.reset();

if (!gripper_moving) {

// Start Movement based on current position

if (grip_hit) {

// At Grip -> Move Out

gripper_dir = 1;

gripper_moving = true;

lcd_set_cursor(1, 0); lcd_print("Grip: Moving OUT");

} else if (release_hit) {

// At Release -> Move In

gripper_dir = -1;

gripper_moving = true;

lcd_set_cursor(1, 0); lcd_print("Grip: Moving IN ");

} else {

// Somewhere in middle -> Default to Out? Or toggle?

// Let's default to moving towards Release if not at limit

gripper_dir = 1;

gripper_moving = true;

}

} else {

// Already moving -> Stop (Toggle behavior)

gripper_moving = false;

set_gripper_motor(0, 0);

lcd_set_cursor(1, 0); lcd_print("Grip: STOPPED   ");

}

}

if (gripper_moving) {

if (gripper_dir == 1) {

// Moving Out (Release)

if (release_hit) {

gripper_moving = false;

set_gripper_motor(0, 0);

lcd_set_cursor(1, 0); lcd_print("Grip: AT RELEASE");

} else {

set_gripper_motor(1, MOVE_DUTY);

}

} else if (gripper_dir == -1) {

// Moving In (Grip)

if (grip_hit) {

gripper_moving = false;

set_gripper_motor(0, 0);

lcd_set_cursor(1, 0); lcd_print("Grip: AT HOME   ");

} else {

set_gripper_motor(-1, MOVE_DUTY);

}

}

}

btn_last = btn_now;

// End Gripper Logic

// --- MAIN STATE MACHINE ---

if (current_state == DISABLED) {

safe_stop_all();

cyl_stop_all();

if (st == 0) { // sens_start pressed

current_state = MOVE_BACK_1;

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("MOVE BACK 0.8m");

lcd_set_cursor(1, 0); lcd_print("Dist: 0.00/0.80m");

thread_sleep_for(100);

} else {

lcd_set_cursor(0, 0);

lcd_print("St:"); lcd_print(st ? "1" : "0");

lcd_print(" Ms:"); lcd_print(ms ? "1" : "0");

lcd_set_cursor(1, 0);

lcd_print("Fld:"); lcd_print(field_str); lcd_print(" READY   ");

static Timer blink;

if(blink.read() > 0.5f) { blink.reset(); led = !led; }

}

}

// 1. MOVE BACKWARD 0.8m

else if (current_state == MOVE_BACK_1) {

float t = state_timer.read();

float dist = t * LINEAR_SPEED;

if (t >= TIME_1) {

safe_stop_all();

wait_ms(300);

current_state = CYL1_CYCLE;

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("CYL 1: CYCLE");

lcd_set_cursor(1, 0); lcd_print("Starting...   ");

cyl_step = 0;

} else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[2][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);

send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);

send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();

lcd_set_cursor(1, 0);

lcd_print("Dist:"); lcd_print_num(dist, 2); lcd_print("/0.80m ");

}

led = 1;

}

thread_sleep_for(20);

}

// 2. CYLINDER 1 CYCLE

else if (current_state == CYL1_CYCLE) {

float t = state_timer.read();

if (cyl_step == 0) {

cyl1_ina = 1; cyl1_inb = 0; cyl1_pwm = 1.0f;

cyl2_ina = 0; cyl2_inb = 0; cyl2_pwm = 0.0f;

lcd_set_cursor(1, 0); lcd_print("C1 Extend     ");

if (t >= 2.0f) {

cyl_stop_all();

wait_ms(500);

current_state = MOVE_BACK_2; // Proceed to next move

state_timer.reset();

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("MOVE BACK 0.5m");

}

}

thread_sleep_for(50);

}

// 3. MOVE BACKWARD 0.5m

else if (current_state == MOVE_BACK_2) {

float t = state_timer.read();

float dist = t * LINEAR_SPEED;

if (t >= TIME_2) {

safe_stop_all();

wait_ms(300);

current_state = CYL2_CYCLE;

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("CYL 2: CYCLE");

lcd_set_cursor(1, 0); lcd_print("Starting...   ");

cyl_step = 0;

} else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[2][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);

send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);

send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();

lcd_set_cursor(1, 0);

lcd_print("Dist:"); lcd_print_num(dist, 2); lcd_print("/0.50m ");

}

led = 1;

}

thread_sleep_for(20);

}

// 4. CYLINDER 2 CYCLE

else if (current_state == CYL2_CYCLE) {

float t = state_timer.read();

if (cyl_step == 0) {

cyl1_ina = 0; cyl1_inb = 0; cyl1_pwm = 0.0f;

cyl2_ina = 1; cyl2_inb = 0; cyl2_pwm = 1.0f;

lcd_set_cursor(1, 0); lcd_print("C2 Extend     ");

if (t >= 2.0f) { cyl_step = 1; state_timer.reset(); }

}

else if (cyl_step == 1) {

cyl2_ina = 0; cyl2_inb = 1; cyl2_pwm = 1.0f;

lcd_set_cursor(1, 0); lcd_print("C2 Retract    ");

if (t >= 2.0f) {

cyl_stop_all();

safe_stop_all();

wait_ms(500);

current_state = FINISHED;

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("SEQUENCE DONE!");

lcd_set_cursor(1, 0); lcd_print("All Complete  ");

led = 1;

}

}

thread_sleep_for(50);

}

else if (current_state == FINISHED) {

safe_stop_all();

cyl_stop_all();

thread_sleep_for(1000);

}

thread_sleep_for(20);

}

}

// ================= IMPLEMENTATIONS =================

void set_gripper_motor(int dir, float duty) {

if (duty > 1.0f) duty = 1.0f;

if (duty < 0.0f) duty = 0.0f;

if (dir == 1) {

// Forward (Out to Release)

dira = 0;

dirb = 1;

pwm = duty;

} else if (dir == -1) {

// Reverse (In to Grip)

dira = 1;

dirb = 0;

pwm = duty;

} else {

// Stop

dira = 0;

dirb = 0;

pwm = 0.0f;

}

}

void send_cmd(uint32_t id, float vel, float kp, float kd) {

if(vel < -30.0f) vel = -30.0f;

if(vel > 30.0f) vel = 30.0f;

int p_int = 2048;

int v_int = (int)((vel + 30.0f) * 4095.0f / 60.0f);

int kp_int = (int)(kp * 4095.0f / 500.0f);

int kd_int = (int)(kd * 4095.0f / 5.0f);

int ff_int = 2048;

tx_msg.data[0] = (p_int >> 8) & 0xFF;

tx_msg.data[1] = p_int & 0xFF;

tx_msg.data[2] = (v_int >> 4) & 0xFF;

tx_msg.data[3] = ((v_int & 0x0F) << 4) | ((kp_int >> 8) & 0x0F);

tx_msg.data[4] = kp_int & 0xFF;

tx_msg.data[5] = (kd_int >> 4) & 0xFF;

tx_msg.data[6] = ((kd_int & 0x0F) << 4) | ((ff_int >> 8) & 0x0F);

tx_msg.data[7] = ff_int & 0xFF;

tx_msg.id = id;

tx_msg.type = CANData;

tx_msg.format = CANStandard;

tx_msg.len = 8;

can.write(tx_msg);

}

void enter_mode(uint32_t id) {

tx_msg.id = id; tx_msg.type = CANData; tx_msg.format = CANStandard; tx_msg.len = 8;

for(int i = 0; i < 7; i++) tx_msg.data[i] = 0xFF;

tx_msg.data[7] = 0xFC;

can.write(tx_msg);

wait_ms(50);

}

void safe_stop_all() {

for(int k=0; k<3; k++) {

for(int i=0; i<NUM_MOTORS; i++) {

send_cmd(MOTOR_ID_1+i, 0.0f, 0.0f, 10.0f);

wait_us(200);

}

wait_ms(10);

}

}

void on_rx() {

if(can.read(rx_msg)) {

if(rx_msg.id == REPLY_ID_1 && rx_msg.len >= 6) rx_count[0]++;

else if(rx_msg.id == REPLY_ID_2 && rx_msg.len >= 6) rx_count[1]++;

else if(rx_msg.id == REPLY_ID_3 && rx_msg.len >= 6) rx_count[2]++;

else if(rx_msg.id == REPLY_ID_4 && rx_msg.len >= 6) rx_count[3]++;

}

}

void cyl_stop_all() {

cyl1_ina = 0; cyl1_inb = 0; cyl1_pwm = 0.0f;

cyl2_ina = 0; cyl2_inb = 0; cyl2_pwm = 0.0f;

}

int read_active_low(DigitalIn& s) {

return (s.read() == 0) ? 1 : 0;

}

// LCD Functions

void wait_us_lcd(int us) { thread_sleep_for(us / 1000); }

void write_i2c(uint8_t data) { char cmd = data | LCD_BACKLIGHT; i2c_lcd.write(lcd_addr << 1, &cmd, 1); }

void pulse_enable(uint8_t data) { write_i2c(data | LCD_EN); wait_us_lcd(100); write_i2c(data & ~LCD_EN); wait_us_lcd(100); }

void send_nibble(uint8_t nibble, uint8_t mode) { uint8_t data = (nibble & 0xF0) | mode | LCD_BACKLIGHT; pulse_enable(data); }

void send_command(uint8_t cmd) { send_nibble(cmd & 0xF0, 0); send_nibble((cmd << 4) & 0xF0, 0); thread_sleep_for(2); }

void send_char(char c) { send_nibble(c & 0xF0, LCD_RS); send_nibble((c << 4) & 0xF0, LCD_RS); }

void lcd_print(const char* str) { while (*str) send_char(*str++); }

void lcd_print_num(float num, int decimals) { char buf[16]; sprintf(buf, "%.*f", decimals, num); lcd_print(buf); }

void lcd_clear() { send_command(0x01); thread_sleep_for(2); }

void lcd_set_cursor(uint8_t row, uint8_t col) { uint8_t addr = (row == 0) ? 0x80 + col : 0xC0 + col; send_command(addr); }

bool detect_lcd() { char dummy = 0; if (i2c_lcd.write(LCD_ADDR_1 << 1, &dummy, 0) == 0) { lcd_addr = LCD_ADDR_1; return true; } if (i2c_lcd.write(LCD_ADDR_2 << 1, &dummy, 0) == 0) { lcd_addr = LCD_ADDR_2; return true; } return false; }

void lcd_init() { thread_sleep_for(50); write_i2c(0x03 | LCD_BACKLIGHT); wait_us_lcd(4500); pulse_enable(0x03 | LCD_BACKLIGHT); wait_us_lcd(4500); pulse_enable(0x03 | LCD_BACKLIGHT); wait_us_lcd(150); pulse_enable(0x02 | LCD_BACKLIGHT); send_command(0x28); send_command(0x0C); send_command(0x06); send_command(0x01); thread_sleep_for(2); }

// GIM8008 Quad Motor + Cylinders - Custom Ladder Sequence

// Sequence: M3+M4 UP -> CAN Fwd -> M3 DOWN -> CAN Fwd -> M4 DOWN -> FINISH

#include "mbed.h"

#include <cstdio>

#include <cstdlib>

#include <cmath>

#include "stm32f4xx_hal.h"

// ================= MOTOR & CAN CONFIG =================

#define MOTOR_ID_1      11

#define MOTOR_ID_2      12

#define MOTOR_ID_3      13

#define MOTOR_ID_4      14

#define CAN_BAUD        1000000

#define REPLY_ID_1      (MOTOR_ID_1 + 0x100)

#define REPLY_ID_2      (MOTOR_ID_2 + 0x100)

#define REPLY_ID_3      (MOTOR_ID_3 + 0x100)

#define REPLY_ID_4      (MOTOR_ID_4 + 0x100)

DigitalOut led(LED1);

Serial pc(USBTX, USBRX, 115200);

CAN can(PA_11, PA_12, CAN_BAUD);

// DC Motor Limit Sensors

DigitalIn lim_grip(PC_1, PullUp);

DigitalIn lim_release(PC_4, PullUp);

// LCD Config

#define I2C_SDA PB_9

#define I2C_SCL PB_8

I2C i2c_lcd(I2C_SDA, I2C_SCL);

uint8_t lcd_addr = 0x27;

#define LCD_BACKLIGHT 0x08

#define LCD_EN 0x04

#define LCD_RS 0x01

#define LCD_ADDR_1 0x27

#define LCD_ADDR_2 0x3F

// ============================================================================

// MOTOR 1: GRIP / BOX HANDLING (VNH5019)

// ============================================================================

PwmOut     m1_pwm(PB_14);

DigitalOut m1_dira(PC_8);

DigitalOut m1_dirb(PC_6);

// ============================================================================

// ORGOGCH MOTORS (CLIMBING M3, M4)

// ============================================================================

// M3

DigitalOut m3_ina(PA_8);

DigitalOut m3_inb(PB_10);

PwmOut     m3_pwm(PA_9);

// M4

DigitalOut m4_ina(PA_13);

DigitalOut m4_inb(PA_14);

PwmOut     m4_pwm(PB_2);

// --- CONFIGURATION ---

const float MAX_VEL = 3.0f;

const float LINEAR_SPEED = MAX_VEL;

const float DISTANCE_1 = 5.0f;

const float DISTANCE_2 = 2.0f;

const float DISTANCE_3 = 10.0f;

const float TIME_1 = DISTANCE_1 / LINEAR_SPEED;

const float TIME_2 = DISTANCE_2 / LINEAR_SPEED;

const float TIME_3 = DISTANCE_3 / LINEAR_SPEED;

const float TIME_4 = TIME_2 / 4;

// Ladder Configuration

const float LADDER_SPEED = 0.5f;

const float LADDER_TIME_20CM = 4.0f;   // Time to move 20cm

const float STABILIZE_TIME = 4.0f;     // Time to move forward

const float STABILIZE_VEL = 5.0f;

const float STABILIZE_BACK = 0.1f;

// Directions for M3/M4

const int DIR_UP_M3    = -1;

const int DIR_UP_M4    = 1;

const int DIR_DOWN_M3  = 1;

const int DIR_DOWN_M4  = -1;

const int NUM_MOTORS = 4;

const int NUM_PATTERNS = 6;

// ================= STATE ENUM =================

enum State {

DISABLED = 0,

MOVE_BACK_1 = 1,

CYL1_CYCLE = 2,

MOVE_BACK_2 = 3,

CYL2_CYCLE = 4,

MOVE_FOR_1 = 5,

MOVE_DC_TO_RELEASE = 6,

TURN_TO_LADDER = 7,

FIRST_LADDER = 8,

FINISHED = 9

};

volatile State current_state = DISABLED;

volatile int rx_count[4] = {0, 0, 0, 0};

CANMessage tx_msg, rx_msg;

float velocities[4] = {0.0f, 0.0f, 0.0f, 0.0f};

const float directions[NUM_PATTERNS][NUM_MOTORS] = {

{1.0f, -1.0f, -1.0f, 1.0f},   // 0:

{1.0f, 1.0f, -1.0f, -1.0f},   // 1: hoish

{-1.0f, 1.0f, 1.0f, -1.0f},   // 2:

{-1.0f, -1.0f, 1.0f, 1.0f},   // 3: Uragsh

{0.0f, 1.0f, 0.0f, -1.0f},    // 4: Diagonal

{-1.0f, -1.0f, -1.0f, -1.0f}  // 5: Turn Arc

};

// ================= SENSORS =================

DigitalIn sens_start(PC_11, PullUp);

DigitalIn sens_motor_start(PC_4, PullUp);

DigitalIn sens_field_select(PC_3, PullUp);

DigitalIn sens_motor_stop(PC_1, PullUp);

DigitalIn box_sensor_1(PA_5, PullUp);

DigitalIn box_sensor_2(PA_4, PullUp);

DigitalIn box_sensor_3(PB_1, PullUp);

// Cylinders

DigitalOut cyl1(PC_7);

DigitalOut cyl2(PB_6);

DigitalOut cyl3(PA_7);

DigitalOut cyl4(PA_6);

DigitalOut* relays[4] = {&cyl1, &cyl2, &cyl3, &cyl4};

const bool ACTIVE_LOW = false;

#define RELAY_ON  1

#define RELAY_OFF 0

// ================= FUNCTION PROTOTYPES =================

void send_cmd(uint32_t id, float vel, float kp = 0, float kd = 1.0f);

void enter_mode(uint32_t id);

void safe_stop_all();

void on_rx();

void cyl_stop_all();

int read_active_low(DigitalIn& s);

int is_pressed(DigitalIn& s);

void set_motor(int dir, float duty);

void stopAllClimb();

// Movement Functions

void moveM3M4Up();

void moveM3Down();

void moveM4Down();

void moveCANForward();

void moveCANBackward();

// LCD Helpers

void wait_us_lcd(int us);

void write_i2c(uint8_t data);

void pulse_enable(uint8_t data);

void send_nibble(uint8_t nibble, uint8_t mode);

void send_command(uint8_t cmd);

void send_char(char c);

void lcd_print(const char* str);

void lcd_print_num(float num, int decimals);

void lcd_clear();

void lcd_set_cursor(uint8_t row, uint8_t col);

bool detect_lcd();

void lcd_init();

// ================= MAIN =================

int main() {

__HAL_RCC_PWR_CLK_ENABLE();

HAL_PWR_EnableBkUpAccess();

__HAL_RCC_LSE_CONFIG(RCC_LSE_OFF);

while(__HAL_RCC_GET_FLAG(RCC_FLAG_LSERDY) != RESET);

// Init Relays Off

for (int i = 0; i < 4; i++) {

relays[i]->write(ACTIVE_LOW ? 1 : 0);

}

// Init Climbing Motors

m3_pwm.period_ms(2); m3_pwm = 0.0f;

m4_pwm.period_ms(2); m4_pwm = 0.0f;

stopAllClimb();

// Init Grip Motor

m1_pwm.period(0.0001f);

m1_pwm = 0.0f;

m1_dira = 0; m1_dirb = 0;

wait_ms(500);

pc.printf("\r\n=== System Boot ===\r\n");

pc.printf("Dir M3 Up: %d, Dir M4 Up: %d\r\n", DIR_UP_M3, DIR_UP_M4);

if (!detect_lcd()) {

led = 1;

while(true) { led = !led; thread_sleep_for(200); }

}

lcd_init();

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("ABU Robocon 2026");

lcd_set_cursor(1, 0); lcd_print("Init CAN...");

int freq = can.frequency(CAN_BAUD);

if(freq == 0) {

lcd_set_cursor(1, 0); lcd_print("CAN FAILED!   ");

while(1) { led = !led; thread_sleep_for(100); }

}

can.attach(&on_rx, CAN::RxIrq);

can.filter(0x100, 0x700, CANStandard, 0);

enter_mode(MOTOR_ID_1); enter_mode(MOTOR_ID_2);

enter_mode(MOTOR_ID_3); enter_mode(MOTOR_ID_4);

safe_stop_all();

lcd_set_cursor(1, 0); lcd_print("Ready. Wait St");

Timer state_timer, debug_timer;

State last_state = DISABLED;

int ladder_sub_step = 0;

while(1) {

int st = read_active_low(sens_start);

int field_raw = sens_field_select.read();

const char* field_str = (field_raw == 0) ? "RED" : "BLU";

if (current_state != last_state) {

state_timer.reset();

state_timer.start();

debug_timer.reset();

last_state = current_state;

pc.printf("State: %d\r\n", current_state);

if (current_state == FIRST_LADDER) {

ladder_sub_step = 0;

pc.printf("Starting Custom Ladder Sequence...\r\n");

}

if(current_state != MOVE_BACK_1 && current_state != MOVE_BACK_2 && current_state != MOVE_FOR_1 && current_state != TURN_TO_LADDER && current_state != FIRST_LADDER) {

safe_stop_all();

set_motor(0, 0);

stopAllClimb();

}

}

// ================= STATE MACHINE =================

if (current_state == DISABLED) {

safe_stop_all(); cyl_stop_all(); set_motor(0, 0); stopAllClimb();

if (read_active_low(sens_start) == 0) {

thread_sleep_for(50);

if(read_active_low(sens_start) == 0) {

current_state = MOVE_BACK_1;

lcd_clear(); lcd_set_cursor(0, 0); lcd_print("1. MOVE BACK 1");

}

} else {

lcd_set_cursor(0, 0); lcd_print("St:"); lcd_print(st ? "1" : "0");

lcd_set_cursor(1, 0); lcd_print("Fld:"); lcd_print(field_str); lcd_print(" READY");

static Timer blink;

if(blink.read() > 0.5f) { blink.reset(); led = !led; }

}

}

else if (current_state == MOVE_BACK_1) {

float t = state_timer.read();

if (t >= TIME_1) { safe_stop_all(); wait_ms(300); current_state = CYL1_CYCLE; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("2. CYL1 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (current_state == CYL1_CYCLE) {

cyl1.write(RELAY_ON); wait_ms(2000); cyl1.write(RELAY_OFF); wait_ms(300);

current_state = MOVE_BACK_2; thread_sleep_for(50);

}

else if (current_state == MOVE_BACK_2) {

float t = state_timer.read();

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); current_state = CYL2_CYCLE; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (current_state == CYL2_CYCLE) {

cyl2.write(RELAY_ON); wait_ms(2000); cyl2.write(RELAY_OFF); wait_ms(300);

current_state = MOVE_FOR_1; thread_sleep_for(50);

}

else if (current_state == MOVE_FOR_1) {

float t = state_timer.read();

if (t >= 6.0) { safe_stop_all(); wait_ms(300); lcd_clear(); lcd_set_cursor(0, 0); lcd_print("6. GRIP CYCLE"); current_state = MOVE_DC_TO_RELEASE; }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[4][i];

for(int i=0; i<4; i++) { send_cmd(MOTOR_ID_1+i, velocities[i]); wait_us(100); }

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Turning..."); } led = 1;

}

thread_sleep_for(20);

}

else if (current_state == MOVE_DC_TO_RELEASE) {

cyl1.write(RELAY_ON); wait_ms(1000);

int release_hit = is_pressed(lim_release);

set_motor(-1, 0.3f);

lcd_set_cursor(0, 0); lcd_print("DC: Moving");

if (release_hit) {

m1_pwm = 0.0f; m1_dira = 1; m1_dirb = 1; wait_ms(50); m1_dira = 0; m1_dirb = 0; wait_ms(100);

current_state = TURN_TO_LADDER; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("RELEASE HIT");

}

wait_ms(10000);

if (state_timer.read() > 6.0f) { set_motor(0, 0); current_state = FINISHED; }

cyl1.write(RELAY_OFF); thread_sleep_for(50);

}

else if (current_state == TURN_TO_LADDER) {

float t = state_timer.read();

if (t >= TIME_2) { safe_stop_all(); current_state = FINISHED; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("8. CLIMB START"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[0][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Approaching..."); } led = 1;

}

thread_sleep_for(20);

}

// 9. FINISHED

else if (current_state == FINISHED) {

safe_stop_all();

cyl_stop_all();

set_motor(0, 0);

stopAllClimb();

led = 0;

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("SEQUENCE DONE");

lcd_set_cursor(1, 0); lcd_print("STOPPED");

thread_sleep_for(100);

}

thread_sleep_for(20);

}

}

// ================= MOVEMENT FUNCTIONS =================

void cyl_stop_all() {

cyl1.write(RELAY_OFF); cyl2.write(RELAY_OFF); cyl3.write(RELAY_OFF); cyl4.write(RELAY_OFF);

}

void set_motor(int dir, float duty) {

if (duty > 1.0f) duty = 1.0f;

if (duty < 0.0f) duty = 0.0f;

if (dir == 1) { m1_dira = 0; m1_dirb = 1; m1_pwm = duty; }

else if (dir == -1) { m1_dira = 1; m1_dirb = 0; m1_pwm = duty; }

else { m1_dira = 0; m1_dirb = 0; m1_pwm = 0.0f; }

}

int read_active_low(DigitalIn& s) { return (s.read() == 0) ? 1 : 0; }

int is_pressed(DigitalIn& s) { return (s.read() == 0) ? 1 : 0; }

void send_cmd(uint32_t id, float vel, float kp, float kd) {

if(vel < -30.0f) vel = -30.0f; if(vel > 30.0f) vel = 30.0f;

int p_int = 2048;

int v_int = (int)((vel + 30.0f) * 4095.0f / 60.0f);

int kp_int = (int)(kp * 4095.0f / 500.0f);

int kd_int = (int)(kd * 4095.0f / 5.0f);

int ff_int = 2048;

tx_msg.data[0] = (p_int >> 8) & 0xFF; tx_msg.data[1] = p_int & 0xFF;

tx_msg.data[2] = (v_int >> 4) & 0xFF; tx_msg.data[3] = ((v_int & 0x0F) << 4) | ((kp_int >> 8) & 0x0F);

tx_msg.data[4] = kp_int & 0xFF; tx_msg.data[5] = (kd_int >> 4) & 0xFF;

tx_msg.data[6] = ((kd_int & 0x0F) << 4) | ((ff_int >> 8) & 0x0F); tx_msg.data[7] = ff_int & 0xFF;

tx_msg.id = id; tx_msg.type = CANData; tx_msg.format = CANStandard; tx_msg.len = 8;

can.write(tx_msg);

}

void enter_mode(uint32_t id) {

tx_msg.id = id; tx_msg.type = CANData; tx_msg.format = CANStandard; tx_msg.len = 8;

for(int i = 0; i < 7; i++) tx_msg.data[i] = 0xFF; tx_msg.data[7] = 0xFC;

can.write(tx_msg); wait_ms(50);

}

void safe_stop_all() {

for(int k=0; k<3; k++) {

for(int i=0; i<NUM_MOTORS; i++) {

send_cmd(MOTOR_ID_1+i, 0.0f, 0.0f, 10.0f); wait_us(200);

}

wait_ms(10);

}

}

void on_rx() {

if(can.read(rx_msg)) {

if(rx_msg.id == REPLY_ID_1 && rx_msg.len >= 6) rx_count[0]++;

else if(rx_msg.id == REPLY_ID_2 && rx_msg.len >= 6) rx_count[1]++;

else if(rx_msg.id == REPLY_ID_3 && rx_msg.len >= 6) rx_count[2]++;

else if(rx_msg.id == REPLY_ID_4 && rx_msg.len >= 6) rx_count[3]++;

}

}

// ================= LCD FUNCTIONS =================

void wait_us_lcd(int us) { thread_sleep_for(us / 1000); }

void write_i2c(uint8_t data) { char cmd = data | LCD_BACKLIGHT; i2c_lcd.write(lcd_addr << 1, &cmd, 1); }

void pulse_enable(uint8_t data) { write_i2c(data | LCD_EN); wait_us_lcd(100); write_i2c(data & ~LCD_EN); wait_us_lcd(100); }

void send_nibble(uint8_t nibble, uint8_t mode) { uint8_t data = (nibble & 0xF0) | mode | LCD_BACKLIGHT; pulse_enable(data); }

void send_command(uint8_t cmd) { send_nibble(cmd & 0xF0, 0); send_nibble((cmd << 4) & 0xF0, 0); thread_sleep_for(2); }

void send_char(char c) { send_nibble(c & 0xF0, LCD_RS); send_nibble((c << 4) & 0xF0, LCD_RS); }

void lcd_print(const char* str) { while (*str) send_char(*str++); }

void lcd_print_num(float num, int decimals) { char buf[16]; sprintf(buf, "%.*f", decimals, num); lcd_print(buf); }

void lcd_clear() { send_command(0x01); thread_sleep_for(2); }

void lcd_set_cursor(uint8_t row, uint8_t col) { uint8_t addr = (row == 0) ? 0x80 + col : 0xC0 + col; send_command(addr); }

bool detect_lcd() {

char dummy = 0;

if (i2c_lcd.write(LCD_ADDR_1 << 1, &dummy, 0) == 0) { lcd_addr = LCD_ADDR_1; return true; }

if (i2c_lcd.write(LCD_ADDR_2 << 1, &dummy, 0) == 0) { lcd_addr = LCD_ADDR_2; return true; }

return false;

}

void lcd_init() {

thread_sleep_for(50);

write_i2c(0x03 | LCD_BACKLIGHT); wait_us_lcd(4500);

pulse_enable(0x03 | LCD_BACKLIGHT); wait_us_lcd(4500);

pulse_enable(0x03 | LCD_BACKLIGHT); wait_us_lcd(150);

pulse_enable(0x02 | LCD_BACKLIGHT);

send_command(0x28); send_command(0x0C); send_command(0x06); send_command(0x01);

thread_sleep_for(2);

}

/* =============================================================================

R2_Main.cpp  —  ABU Robocon 2026 "Kung Fu Quest" — FULL AUTO + FOREST PATH FIRST

Platform    : NUCLEO_F446RE  mbed OS5  ARMC6

FLOW (PATH INPUT FIRST):

boot

└─► main loop (per round):

STEP 1  collect_forest_path()  ← IR remote sends path FIRST

Your exact KEY_MAP + debug prints

1-12 = block | press-count: 1=straight 2=R 3+=L

Confirm = lock | Ok = done | Clear = reset | B = skip

STEP 2  op_mode()              ← operator picks run mode

btn_a scroll, IR confirm:

0 NORMAL      full sequence (spear → forest)

1 RETRY MC    skip spear, go straight to forest

2 RETRY ARN   arena (no forest path used)

3 TEST SENS   sensor diagnostics

4 TEST ACT    actuator/cylinder tests

STEP 3  dispatch()             ← runs selected mode using stored path

Cases 1-6: spear pickup at rack

Case 7   : forest entry (50cm + climb onto first block)

Case 8   : forest navigation (executes IR-stored path)

Case 9   : release + done

STEP 4  wait for IR            ← start next round (new path)

============================================================================= */

#include "mbed.h"

#include <cmath>

#include <cstdlib>

#include <cstdio>

#include <cstdarg>

#include <cstring>

#define DEBUG 1

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 1 — CONFIGURATION                                              ║

// ╚══════════════════════════════════════════════════════════════════════════╝

float S1_K = 43.057f, S1_OFF = 0.442f;

float S2_K = 43.057f, S2_OFF = 0.442f;

float S1_SPEAR_CM = 30.0f;

float S2_RACK_CM  = 30.0f;

#define TICKS_PER_REV  133.0f

#define WHEEL_DIAM_M   0.220f

#define WHEEL_CIRC_M   (3.14159f * WHEEL_DIAM_M)

#define TICKS_PER_M    (TICKS_PER_REV / WHEEL_CIRC_M)

#define MOTOR_DIR_SIGN (-1)

float APPROACH_METRES = 0.70f;

float APPROACH_SPD    = 3.0f;

float APPROACH_KP     = 0.08f;

int   APPROACH_CREEP_TICKS = 8;

float APPROACH_CREEP_SPD   = 0.35f;

float BACK_SPD = 3.0f;

float STRAFE_SPD = 0.6f;

float FWD_SPEAR_SPD = 1.5f;

float ROT_DUTY = 0.5f;

uint32_t DIAG_MS = 4000;

uint32_t TURN_45_MS = 1000;

float    MOVE_SPD = 3.0f;

uint32_t CYL_ARM_MS = 1000;

uint32_t GRIP_SETTLE_MS = 500;

uint32_t CYL2_EXTEND_MS = 600;

uint32_t CYL1_RETRACT_MS = 800;

uint32_t CYL4_MS = 1000;

uint32_t ROT_TIMEOUT_MS = 4000;

uint32_t HOLD_MS = 10000;

uint32_t SWEEP_MAX_MS = 2500;

float    RAMP_STEP = 0.05f;

// ── FOREST CONFIG ─────────────────────────────────────────────────────────

#define MAX_FOREST_PATH 16

#define MAX_FOREST_BOXES 12

float    FOREST_STEP_M     = 0.50f;

float    FOREST_DRIVE_SPD  = 1.5f;

uint32_t CLIMB_FRONT_MS    = 1000;

uint32_t CLIMB_BACK_MS     = 1000;

uint32_t CLIMB_DRIVE_MS    = 1200;

uint32_t TURN_FOREST_MS    = 800;

float    BOX_GRIP_MOVE_M   = 1.0f;

float    BOX_GRIP_SPD      = 2.0f;

#define  BOX_GRIP_KP           0.08f

#define  BOX_GRIP_CREEP_TICKS  8

#define  BOX_GRIP_CREEP_SPD    0.35f

#define  CAN_GRIP              10

int  forest_path_blocks[MAX_FOREST_PATH];

int  forest_path_presses[MAX_FOREST_PATH];

int  forest_path_len = 0;

bool forest_path_confirmed = false;

int  forest_box_blocks[MAX_FOREST_BOXES];

int  forest_box_actions[MAX_FOREST_BOXES];

int  forest_box_count = 0;

bool forest_boxes_confirmed = false;

// ── YOUR EXACT KEY_MAP (unchanged) ────────────────────────────────────────

typedef struct { uint8_t code; const char* label; } KeyMap;

const KeyMap KEY_MAP[] = {

{0x00, "10"}, {0x01, "11"}, {0x02, "12"},

{0x04, "7"},  {0x05, "8"},  {0x06, "9"},

{0x08, "4"},  {0x09, "5"},  {0x0A, "6"},

{0x0D, "2"},  {0x0C, "1"},  {0x0E, "3"},

{0x15, "Ok"}, {0xFF, "0"},  {0x11, "Confirm"},

{0x14, "Last mood"}, {0x16, "Next mood"}, {0x19, "Clear"}

};

#define KEY_COUNT (sizeof(KEY_MAP)/sizeof(KEY_MAP[0]))

#define PATH_MODE 0

#define BOX_MODE  1

// Block height/action lookups

float ir_get_height(int block) {

if (block == 2 || block == 4 || block == 10 || block == 12) return 20.0f;

if (block == 6 || block == 8) return 60.0f;

if (block >= 1 && block <= 12) return 40.0f;

return 0.0f;

}

int ir_get_box_action(int block) {

if (block==1||block==3||block==5||block==6||block==7||block==8) return 1;

if (block==4||block==9||block==10||block==12) return -1;

return 0;

}

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 2 — HARDWARE                                                    ║

// ╚══════════════════════════════════════════════════════════════════════════╝

CAN        can(PA_11, PA_12, 1000000);

Serial     pc(USBTX, USBRX, 115200);

DigitalOut led(LED1);

AnalogIn   sharp1(PC_0);

AnalogIn   sharp2(PA_4);

DigitalOut cyl_arm(PC_7);

DigitalOut cyl_grip(PB_6);

DigitalOut cyl_up(PA_6);

DigitalOut cyl_dir(PA_7);

DigitalOut cyl5_a(PA_8),  cyl5_b(PB_10);

DigitalOut cyl6_a(PA_13), cyl6_b(PA_14);

PwmOut     rot_pwm(PB_14);

DigitalOut rot_dira(PC_6), rot_dirb(PC_8);

DigitalIn  sw_home(PC_1), sw_90(PA_0);

DigitalIn  zone_sw(PC_3, PullUp);

DigitalIn  btn_a(PA_10, PullUp);

DigitalIn  btn_b(PB_3,  PullUp);

DigitalIn  ir_sens(PA_9, PullUp);  // IR_GO button

I2C        i2c_lcd(PB_9, PB_8);

uint8_t    lcd_addr = 0x27;

// ✅ FIX: IR Remote (NEC) - use PB_0 (NOT PA_10 which is btn_a!)

#define IR_PIN PA_10

DigitalIn  ir_pin(IR_PIN, PullUp);

Timer      ir_timer;

Timer      press_timer;  // For multi-press detection

static int forest_step_idx = 0;

static int forest_sub_step = 0;  // 0=climb, 1=move, 2=box, 3=turn

static bool forest_init_done = false;

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 3 — MODE CONSTANTS                                              ║

// ╚══════════════════════════════════════════════════════════════════════════╝

#define ZONE_RED 0

#define ZONE_BLUE 1

#define MODE_NORMAL 0

#define MODE_RETRY_MC 1

#define MODE_RETRY_ARN 2

#define MODE_TEST_SENS 3

#define MODE_TEST_ACT 4

#define NUM_MODES 5

#define S4_FWD       0

#define S4_CYL2_EXT  1

#define S4_CYL1_RET  2

int g_zone = ZONE_RED;

int g_mode = MODE_NORMAL;

struct ForestState {

int   block;

float height;

int   direction;

} forest_state;

void forest_state_init(int zone) {

forest_state.block     = (zone == ZONE_RED) ? 2 : 11;

forest_state.height    = 0.0f;

forest_state.direction = 0;

}

void cyl5_extend()  { cyl5_a = 1; cyl5_b = 0; }

void cyl5_retract() { cyl5_a = 0; cyl5_b = 1; }

void cyl6_extend()  { cyl6_a = 1; cyl6_b = 0; }

void cyl6_retract() { cyl6_a = 0; cyl6_b = 1; }

void cyl56_safe()   { cyl5_a = 0; cyl5_b = 0; cyl6_a = 0; cyl6_b = 0; }

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 4 — CAN / MOTOR ENGINE                                          ║

// ╚══════════════════════════════════════════════════════════════════════════╝

#define CAN_FL 11

#define CAN_BL 12

#define CAN_BR 13

#define CAN_FR 14

#define NUM_W  4

const float MOTOR_DIR[5][4] = {

{ 1.0f,-1.0f,-1.0f, 1.0f},

{-1.0f,-1.0f, 1.0f, 1.0f},

{-1.0f, 1.0f, 1.0f,-1.0f},

{ 1.0f, 1.0f,-1.0f,-1.0f},

{ 1.0f, 0.0f,-1.0f, 0.0f},

};

CANMessage tx_msg, rx_msg;

volatile int32_t  enc_left = 32000, enc_right = 32000;

int32_t old_stop = 32000;

volatile uint16_t last_pos[5] = {2048,2048,2048,2048,2048};

volatile bool     first_rx[5] = {true,true,true,true,true};

volatile uint32_t last_rx_ms[5] = {0};

float vel_l = 0.f, vel_r = 0.f;

#define CORR_GAIN 16

#define CORR_CAP 0.4f

void can_send_vel(uint32_t id, float vel) {

if(vel<-30.f) vel=-30.f; if(vel>30.f) vel=30.f;

int p=1, v=(int)((vel+30.f)*4095.f/60.f);

int kdi=(int)(1.0f*4095.f/5.f), ff=2048;

tx_msg.data[0]=(p>>8)&0xFF; tx_msg.data[1]=p&0xFF;

tx_msg.data[2]=(v>>4)&0xFF; tx_msg.data[3]=((v&0xF)<<4);

tx_msg.data[4]=0;

tx_msg.data[5]=(kdi>>4)&0xFF;

tx_msg.data[6]=((kdi&0xF)<<4)|((ff>>8)&0xF);

tx_msg.data[7]=ff&0xFF;

tx_msg.id=id; tx_msg.type=CANData; tx_msg.format=CANStandard; tx_msg.len=8;

can.write(tx_msg); wait_us(100);

}

void can10_send_vel(float vel) { can_send_vel(CAN_GRIP, vel); }

void motors_drive(int row, float speed) {

for(int i=0;i<NUM_W;i++){ can_send_vel(CAN_FL+i, speed*MOTOR_DIR[row][i]); wait_us(100); }

}

void motors_stop() {

for(int k=0;k<3;k++){

for(int i=0;i<NUM_W;i++){ can_send_vel(CAN_FL+i,0.f); wait_us(200); }

wait_us(5000);

}

vel_l=vel_r=0.f;

}

void can_enter_mode(uint32_t id) {

tx_msg.id=id; tx_msg.type=CANData; tx_msg.format=CANStandard; tx_msg.len=8;

for(int i=0;i<7;i++) tx_msg.data[i]=0xFF; tx_msg.data[7]=0xFC;

can.write(tx_msg); thread_sleep_for(50);

}

void linear_forward(float spd) {

float c=0.f;

if(enc_right>enc_left){ c=(float)(enc_right-enc_left)/CORR_GAIN/100.f; if(c>CORR_CAP) c=CORR_CAP; vel_r=spd-c; vel_l=spd+c; }

else if(enc_left>enc_right){ c=(float)(enc_left-enc_right)/CORR_GAIN/100.f; if(c>CORR_CAP) c=CORR_CAP; vel_l=spd-c; vel_r=spd+c; }

else { vel_l=vel_r=spd; }

if(vel_l<0.1f) vel_l=0.1f; if(vel_l>30.f) vel_l=30.f;

if(vel_r<0.1f) vel_r=0.1f; if(vel_r>30.f) vel_r=30.f;

}

void drive_fwd_corrected(float cmd) {

linear_forward(cmd);

can_send_vel(CAN_FL,-vel_l); wait_us(100);

can_send_vel(CAN_BL,-vel_l); wait_us(100);

can_send_vel(CAN_BR, vel_r); wait_us(100);

can_send_vel(CAN_FR, vel_r); wait_us(100);

}

void drive_diagonal_BR(float spd){ can_send_vel(CAN_FL,0.f);wait_us(100); can_send_vel(CAN_BL,spd);wait_us(100); can_send_vel(CAN_BR,0.f);wait_us(100); can_send_vel(CAN_FR,-spd);wait_us(100); }

void drive_diagonal_BL(float spd){ can_send_vel(CAN_FL,spd);wait_us(100); can_send_vel(CAN_BL,0.f);wait_us(100); can_send_vel(CAN_BR,-spd);wait_us(100); can_send_vel(CAN_FR,0.f);wait_us(100); }

void drive_spin(float spd){ can_send_vel(CAN_FL,spd);wait_us(100); can_send_vel(CAN_BL,spd);wait_us(100); can_send_vel(CAN_BR,spd);wait_us(100); can_send_vel(CAN_FR,spd);wait_us(100); }

void on_can_rx() {

if(!can.read(rx_msg) || rx_msg.len<6) return;

uint32_t now = us_ticker_read()/1000;

for(int i=0;i<5;i++){

uint32_t expected_id = (i<NUM_W) ? (CAN_FL+i+0x100) : (CAN_GRIP+0x100);

if(rx_msg.id != expected_id) continue;

last_rx_ms[i]=now;

uint16_t pos = ((uint16_t)rx_msg.data[1]<<4) | (rx_msg.data[2]>>4);

if(pos>4095) continue;

if(first_rx[i]){ last_pos[i]=pos; first_rx[i]=false; continue; }

int16_t d = (int16_t)pos - (int16_t)last_pos[i];

if(d>2048) d-=4096; if(d<-2048) d+=4096;

last_pos[i]=pos;

if(i==0||i==1) enc_left += (int32_t)d;

if(i==2||i==3) enc_right += (int32_t)d;

}

}

void encoders_reset() {

__disable_irq();

enc_left=enc_right=old_stop=32000;

for(int i=0;i<NUM_W;i++){ last_pos[i]=2048; first_rx[i]=true; }

__enable_irq();

}

bool move_forward_metres(float distance_m, float max_speed) {

int32_t target_ticks = (int32_t)(distance_m*TICKS_PER_M)*MOTOR_DIR_SIGN;

encoders_reset();

uint32_t t0 = us_ticker_read()/1000, last_print=0;

int32_t last_enc=0, t_stall=t0;

#ifdef DEBUG == 1

pc.printf("[MOVE] FWD %.4fm target=%+ld\r\n", (double)distance_m, (long)target_ticks);

#endif

while(true){

while(can.read(rx_msg)) on_can_rx();

int32_t enc_avg = (enc_left+enc_right)/2;

int32_t error = target_ticks - enc_avg;

bool reached = (target_ticks<0) ? (enc_avg<=target_ticks) : (enc_avg>=target_ticks);

if(reached){ motors_stop(); pc.printf("[MOVE] DONE enc=%+ld\r\n",(long)enc_avg); return true; }

uint32_t now = us_ticker_read()/1000;

if(now-t0 > 8000){ motors_stop(); pc.printf("[MOVE] TIMEOUT\r\n"); return false; }

if(abs(enc_avg-last_enc)>2){ last_enc=enc_avg; t_stall=now; }

else if(now-t_stall>1500){ motors_stop(); for(int i=0;i<NUM_W;i++) can_enter_mode(CAN_FL+i); thread_sleep_for(300); t_stall=us_ticker_read()/1000; last_enc=enc_avg; }

float cmd; int abs_err=abs((int)error);

if(abs_err<=APPROACH_CREEP_TICKS) cmd=APPROACH_CREEP_SPD;

else { cmd=APPROACH_KP*(float)abs_err; if(cmd>max_speed) cmd=max_speed; if(cmd<0.8f) cmd=0.8f; }

if(error<0) cmd=-cmd;

drive_fwd_corrected(cmd);

thread_sleep_for(10);

}

}

bool move_can10_metres(float distance_m, float max_speed) {

__disable_irq(); enc_left=32000; last_pos[4]=2048; first_rx[4]=true; __enable_irq();

int32_t target_ticks = (int32_t)(fabsf(distance_m)*TICKS_PER_M)*MOTOR_DIR_SIGN;

if(distance_m<0) target_ticks=-target_ticks;

uint32_t t0=us_ticker_read()/1000; int32_t last_enc=0, t_stall=t0;

pc.printf("  [GRIP] %.2fm target=%+ld\r\n", (double)distance_m, (long)target_ticks);

while(true){

while(can.read(rx_msg)){

if(rx_msg.len<6 || rx_msg.id!=(CAN_GRIP+0x100)) continue;

uint16_t pos = ((uint16_t)rx_msg.data[1]<<4)|(rx_msg.data[2]>>4);

if(pos>4095) continue;

if(first_rx[4]){ last_pos[4]=pos; first_rx[4]=false; continue; }

int16_t d = (int16_t)pos-(int16_t)last_pos[4];

if(d>2048) d-=4096; if(d<-2048) d+=4096;

last_pos[4]=pos; enc_left += (int32_t)d;

}

int32_t enc_avg = enc_left-32000;

int32_t error = target_ticks-enc_avg;

bool reached = (target_ticks<0) ? (enc_avg<=target_ticks) : (enc_avg>=target_ticks);

if(reached){ can10_send_vel(0.f); pc.printf("  [GRIP] DONE\r\n"); return true; }

if(us_ticker_read()/1000-t0 > 5000){ can10_send_vel(0.f); pc.printf("  [GRIP] TIMEOUT\r\n"); return false; }

if(abs(enc_avg-last_enc)>2){ last_enc=enc_avg; t_stall=us_ticker_read()/1000; }

else if(us_ticker_read()/1000-t_stall>1500){ can10_send_vel(0.f); can_enter_mode(CAN_GRIP); thread_sleep_for(300); t_stall=us_ticker_read()/1000; last_enc=enc_avg; }

float cmd; int abs_err=abs((int)error);

if(abs_err<=BOX_GRIP_CREEP_TICKS) cmd=BOX_GRIP_CREEP_SPD;

else { cmd=BOX_GRIP_KP*(float)abs_err; if(cmd>max_speed) cmd=max_speed; if(cmd<0.8f) cmd=0.8f; }

if(error<0) cmd=-cmd;

can10_send_vel(cmd);

thread_sleep_for(10);

}

}

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 5 — SENSORS                                                     ║

// ╚══════════════════════════════════════════════════════════════════════════╝

float read_cm(AnalogIn& s, float K, float off) {

float v = (float)s.read_u16()/65535.f*3.3f;

if(v<0.35f) return 999.f; if(v>2.90f) return 10.f;

float d = K/(v-off);

if(d<10.f) d=10.f; if(d>150.f) return 999.f;

return d;

}

bool rack_in_range(float d){ return (d<S2_RACK_CM && d>1.f); }

bool spear_detected(float d){ return (d<S1_SPEAR_CM && d>1.f); }

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 6 — LCD                                                         ║

// ╚══════════════════════════════════════════════════════════════════════════╝

void lcd_write(uint8_t d){ char c=(char)(d|0x08); i2c_lcd.write(lcd_addr<<1,&c,1); }

void lcd_pulse(uint8_t d){ lcd_write(d|0x04); wait_us(1); lcd_write(d&~0x04); wait_us(50); }

void lcd_nibble(uint8_t n,uint8_t rs){ lcd_pulse((n&0xF0)|rs|0x08); }

void lcd_cmd(uint8_t c){ lcd_nibble(c&0xF0,0); lcd_nibble((c<<4)&0xF0,0); wait_us(37); }

void lcd_cmd_slow(uint8_t c){ lcd_nibble(c&0xF0,0); lcd_nibble((c<<4)&0xF0,0); wait_us(1600); }

void lcd_char(char c){ lcd_nibble(c&0xF0,1); lcd_nibble((c<<4)&0xF0,1); wait_us(37); }

void lcd_str(const char* s){ while(*s) lcd_char(*s++); }

void lcd_clr(){ lcd_cmd_slow(0x01); }

void lcd_pos(uint8_t r,uint8_t c){ lcd_cmd((r==0)?(0x80+c):(0xC0+c)); }

void lprintf(uint8_t row,uint8_t col,const char* fmt,...) {

char buf[17]={0}; va_list a; va_start(a,fmt); vsnprintf(buf,17,fmt,a); va_end(a);

int l=strlen(buf); while(l<16) buf[l++]=' '; buf[16]=0;

lcd_pos(row,col); lcd_str(buf);

}

bool lcd_detect(){ char d=0; if(i2c_lcd.write(0x27<<1,&d,0)==0){lcd_addr=0x27;return true;} if(i2c_lcd.write(0x3F<<1,&d,0)==0){lcd_addr=0x3F;return true;} return false; }

void lcd_init(){

thread_sleep_for(50);

for(int i=0;i<3;i++){ lcd_pulse(0x03|0x08); wait_us(4500); }

lcd_pulse(0x02|0x08);

lcd_cmd(0x28); lcd_cmd(0x0C); lcd_cmd(0x06); lcd_cmd_slow(0x01);

}

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 8 — IR REMOTE NEC DECODER (YOUR EXACT CODE + DEBUG)            ║

// ╚══════════════════════════════════════════════════════════════════════════╝

#define NEC_LEADER_MIN  8000

#define NEC_LEADER_MAX  10000

#define NEC_ONE_MIN     1500

int wait_for_level(int level, int timeout_us) {

ir_timer.reset(); ir_timer.start();

while (ir_pin.read() != level) {

if (ir_timer.read_us() > timeout_us) { ir_timer.stop(); return -1; }

}

int t = ir_timer.read_us(); ir_timer.stop();

return t;

}

uint32_t receive_nec() {

if (wait_for_level(0, 100000) < 0) return 0;

int low = wait_for_level(1, 15000);

if (low < NEC_LEADER_MIN || low > NEC_LEADER_MAX) return 0;

int high = wait_for_level(0, 6000);

if (high < 3000 || high > 6000) return 0;

uint32_t data = 0;

for (int i = 0; i < 32; i++) {

if (wait_for_level(1, 1000) < 0) return 0;

int space = wait_for_level(0, 2500);

if (space < 0) return 0;

if (space > NEC_ONE_MIN) data |= (1u << i);

}

wait_for_level(1, 1000);

return data;

}

uint8_t last_cmd = 0xFF;

int press_count = 0;

// ✅ YOUR EXACT decode_label + DEBUG PRINTS + Mbed OS 5 FIX

const char* decode_label(uint32_t frame, int* out_block) {

uint8_t cmd = (frame >> 16) & 0xFF;

uint8_t cmd_inv = (frame >> 24) & 0xFF;

if ((uint8_t)(cmd ^ cmd_inv) != 0xFF) {

pc.printf("[IR DEBUG] Checksum fail: cmd=0x%02X inv=0x%02X\r\n", cmd, cmd_inv);

return nullptr;

}

// ✅ FIX: Use read_us() for Mbed OS 5 compatibility (500ms = 500000µs)

if (cmd == last_cmd && press_timer.read_us() < 500000) {

press_count++;

} else {

press_count = 1;

last_cmd = cmd;

}

press_timer.reset();

// 🔍 DEBUG: Print raw code for EVERY button press

pc.printf("[IR DEBUG] Frame=0x%08lX | Cmd=0x%02X | Inv=0x%02X | Presses=%d\r\n",

(unsigned long)frame, cmd, cmd_inv, press_count);

// Look up in YOUR KEY_MAP

for (int i = 0; i < KEY_COUNT; i++) {

if (KEY_MAP[i].code == cmd) {

if (out_block) {

int val = atoi(KEY_MAP[i].label);

*out_block = (val >= 1 && val <= 12) ? val : -1;

}

pc.printf("[IR DEBUG] ✓ Matched: '%s' → Block=%d\r\n", KEY_MAP[i].label, *out_block);

return KEY_MAP[i].label;

}

}

pc.printf("[IR DEBUG] ✗ Unknown code: 0x%02X (add to KEY_MAP[])\r\n", cmd);

return nullptr;

}

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 8.5 — FOREST PATH INPUT (YOUR EXACT LOGIC + GLOBAL STORAGE)    ║

// ╚══════════════════════════════════════════════════════════════════════════╝

struct ForestRobot {

int   current_block;

float current_height_cm;

int   direction;

} forest_robot;

void forest_state_init_v1(int zone) {

forest_robot.current_block     = (zone == ZONE_RED) ? 2 : 11;

forest_robot.current_height_cm = 0.0f;

forest_robot.direction         = 0;

}

// ✅ YOUR EXACT collect_forest_path() + DEBUG + GLOBAL STORAGE

bool collect_forest_path() {

pc.printf("\r\n================================\n");

pc.printf("  IR PATH NAVIGATION (FIRST)\n");

pc.printf("================================\n");

pc.printf("Remote Labels → Blocks:\n");

pc.printf("  1→Block1  2→Block2  3→Block3\n");

pc.printf("  4→Block4  5→Block5  6→Block6\n");

pc.printf("  7→Block7  8→Block8  9→Block9\n");

pc.printf("  10→Block10  11→Block11  12→Block12\n");

pc.printf("\nPress Count:\n");

pc.printf("  1x = Climb to height\n");

pc.printf("  2x = Climb + Turn Right\n");

pc.printf("  3x = Climb + Turn Left\n");

pc.printf("\nCommands:\n");

pc.printf("  Clear   = Clear path\n");

pc.printf("  Confirm = Lock path\n");

pc.printf("  Ok      = Execute path\n");

pc.printf("  btn_b   = Skip path (for test modes)\n");

pc.printf("================================\n\n");

// Reset path/box storage

forest_path_len = 0; forest_path_confirmed = false;

forest_box_count = 0; forest_boxes_confirmed = false;

int input_mode = PATH_MODE;

press_timer.reset(); press_timer.start();

last_cmd = 0xFF; press_count = 0;

lprintf(0,0,"FOREST PATH IN  ");

lprintf(1,0,"IR remote, B=sk ");

uint32_t last_blink = us_ticker_read()/1000;

int b_prev = 1;

while(true) {

// btn_b skips path collection entirely

int b = btn_b.read();

if(b_prev == 1 && b == 0) {

b_prev = b; thread_sleep_for(50);

pc.printf("[FOREST] btn_b → skip path collection\r\n");

lprintf(0,0,"PATH SKIPPED    ");

lprintf(1,0,"No forest run   ");

thread_sleep_for(800);

return false;

}

b_prev = b;

// LED heartbeat

uint32_t now = us_ticker_read()/1000;

if(now - last_blink > 500) { last_blink = now; led = !led; }

// Try to receive IR frame

uint32_t frame = receive_nec();

if(frame == 0) { thread_sleep_for(10); continue; }

// === PRINT RAW DATA (YOUR DEBUG) ===

uint8_t addr    = frame & 0xFF;

uint8_t addr_inv= (frame >> 8) & 0xFF;

uint8_t cmd     = (frame >> 16) & 0xFF;

uint8_t cmd_inv = (frame >> 24) & 0xFF;

pc.printf("[RAW] 0x%08lX | A:0x%02X/~0x%02X C:0x%02X/~0x%02X\n",

frame, addr, addr_inv, cmd, cmd_inv);

int block = -1;

const char* label = decode_label(frame, &block);

if(!label) continue;

// ✅ YOUR DEBUG PRINT

pc.printf("[DECODED] Label:\"%s\" Block:%d Presses:%d\n",

label, block, press_count);

// === Handle Commands (YOUR EXACT LOGIC) ===

if (strcmp(label, "Ok") == 0) {

if(!forest_path_confirmed) {

lprintf(1,0,"Confirm first!  ");

pc.printf("[FOREST] cannot Ok — path not confirmed\r\n");

continue;

}

lprintf(0,0,"PATH SAVED      ");

lprintf(1,0,"Pick mode now   ");

pc.printf("[FOREST] === PATH SAVED (%d steps), going to op_mode ===\r\n",

forest_path_len);

led=1; thread_sleep_for(500); led=0;

return true;

}

if (strcmp(label, "Clear") == 0) {

pc.printf("[CMD] Clear → Reset path\n");

forest_path_len = 0;

forest_path_confirmed = false;

pc.printf("[PATH] CLEARED\n");

for (int i = 0; i < 3; i++) { led = !led; wait_ms(150); }

led = 0;

lprintf(1,0,"PATH cleared    ");

continue;

}

if (strcmp(label, "Confirm") == 0) {

pc.printf("[CMD] Confirm → Lock path\n");

if (forest_path_len == 0) {

pc.printf("[WARN] Path empty!\n");

lprintf(1,0,"PATH empty!     ");

continue;

}

forest_path_confirmed = true;

pc.printf("\n✓ PATH CONFIRMED (%d steps)\n", forest_path_len);

pc.printf("Press 'Ok' to finish, 'Clear' to reset\n\n");

led = 1; wait_ms(400); led = 0;

lprintf(0,0,"PATH OK %d steps ", forest_path_len);

lprintf(1,0,"Press Ok to GO  ");

continue;

}

// === Add numeric blocks to path (YOUR EXACT LOGIC) ===

if (block >= 1 && block <= 12) {

if(input_mode==PATH_MODE) {

if(forest_path_len>=MAX_FOREST_PATH){

lprintf(1,0,"PATH FULL!      ");

continue;

}

forest_path_blocks[forest_path_len]  = block;

forest_path_presses[forest_path_len] = press_count;

forest_path_len++;

pc.printf("[INPUT] Block %d | Press #%d\n", block, press_count);

pc.printf("[PATH] Added:%d(%dx) | Full:", block, press_count);

for (int i = 0; i < forest_path_len; i++) {

pc.printf(" %d(%dx)", forest_path_blocks[i], forest_path_presses[i]);

}

pc.printf("\n");

lprintf(0,0,"PATH len=%d     ", forest_path_len);

lprintf(1,0,"+B%d x%d        ", block, press_count);

// LED blinks = press count

for (int i = 0; i < press_count && i < 4; i++) {

led = 1; wait_ms(80); led = 0; wait_ms(80);

}

} else {

// BOX mode (optional)

int act = ir_get_box_action(block);

if(act==0){ lprintf(1,0,"No box at B%d   ", block); continue; }

bool dup=false;

for(int i=0;i<forest_box_count;i++) if(forest_box_blocks[i]==block) dup=true;

if(dup){ lprintf(1,0,"B%d already in  ", block); continue; }

if(forest_box_count>=MAX_FOREST_BOXES){ lprintf(1,0,"BOX list full   "); continue; }

forest_box_blocks[forest_box_count] = block;

forest_box_actions[forest_box_count]= act;

forest_box_count++;

lprintf(1,0,"+Box B%d %s     ", block, (act==1)?"UP":"DN");

pc.printf("[FOREST] +box B%d %s (n=%d)\r\n",

block, (act==1)?"UP":"DN", forest_box_count);

}

} else {

pc.printf("[INFO] Label \"%s\" not a block (1-12)\n", label);

}

thread_sleep_for(250);  // Debounce

}

}

// ── Forest Movement Helpers ────────────────

void forest_turn_right() {

pc.printf("[FOREST] turn RIGHT (strafe)\r\n");

lprintf(1,0,"Turn RIGHT      ");

motors_drive(0, STRAFE_SPD);

thread_sleep_for(TURN_FOREST_MS);

motors_stop();

forest_robot.direction = (forest_robot.direction + 1) % 4;

thread_sleep_for(150);

}

void forest_turn_left() {

pc.printf("[FOREST] turn LEFT (strafe)\r\n");

lprintf(1,0,"Turn LEFT       ");

motors_drive(2, STRAFE_SPD);

thread_sleep_for(TURN_FOREST_MS);

motors_stop();

forest_robot.direction = (forest_robot.direction + 3) % 4;

thread_sleep_for(150);

}

void forest_adjust_height(float target_cm) {

float diff = target_cm - forest_robot.current_height_cm;

if(fabsf(diff) < 5.0f) {

pc.printf("[FOREST] same height %.0fcm — no climb\r\n", (double)target_cm);

return;

}

}

void forest_grab_box(int direction) {

pc.printf("[FOREST] GRAB BOX dir=%+d\r\n", direction);

lprintf(0,0,"GRAB BOX %s     ", (direction>0)?"UP":"DN");

lprintf(1,0,"CAN10 extend... ");

if(!move_can10_metres(BOX_GRIP_MOVE_M * (float)direction, BOX_GRIP_SPD)) {

pc.printf("[FOREST] CAN10 extend FAIL\r\n");

lprintf(1,0,"GRIP FAIL ext   ");

return;

}

lprintf(1,0,"CYL2 close      ");

cyl_grip = 1;

thread_sleep_for(GRIP_SETTLE_MS);

lprintf(1,0,"CAN10 retract.. ");

if(!move_can10_metres(-BOX_GRIP_MOVE_M * (float)direction, BOX_GRIP_SPD)) {

pc.printf("[FOREST] CAN10 retract FAIL\r\n");

lprintf(1,0,"GRIP FAIL ret   ");

return;

}

lprintf(1,0,"BOX GRABBED OK  ");

pc.printf("[FOREST] BOX OK\r\n");

thread_sleep_for(200);

}

bool forest_should_grab_box(int block) {

int act = ir_get_box_action(block);

if(act == 0) return false;

if(forest_boxes_confirmed && forest_box_count > 0) {

for(int i=0;i<forest_box_count;i++)

if(forest_box_blocks[i] == block) return true;

return false;

}

return true;

}

// ── Lookup: is a box registered for this block? ──────────────────────────

bool forest_block_has_box(int block, int* out_action) {

for(int i = 0; i < forest_box_count; i++) {

if(forest_box_blocks[i] == block) {

if(out_action) *out_action = forest_box_actions[i];

return true;

}

}

return false;

}

// ── forest_move_to_block: climb sequence for one block (CYL5+CYL6 walk) ──

//   UP-group blocks (1,2,3,5,6,7,8,11): climb UP onto the block

//   DOWN-group blocks (4,9,10,12)     : climb DOWN onto the block

void forest_move_to_block(int block) {

bool is_up_group   = (block == 1 || block == 2 || block == 3 ||

block == 5 || block == 6 || block == 7 ||

block == 8 || block == 11);

bool is_down_group = (block == 4 || block == 9 ||

block == 10 || block == 12);

// ── Inline 40cm move using your working Case-2 pattern ──

#define FWD_40CM() do { \

encoders_reset(); \

uint32_t _t0 = us_ticker_read()/1000; \

int32_t _target = (int32_t)(0.40f * TICKS_PER_M) * MOTOR_DIR_SIGN; \

while(true) { \

while(can.read(rx_msg)) on_can_rx(); \

int32_t _enc = ((enc_left+enc_right)/2) - 32000; \

bool _done = (_target<0) ? (_enc<=_target) : (_enc>=_target); \

if(_done || (us_ticker_read()/1000 - _t0 > 2000)) { motors_stop(); break; } \

drive_fwd_corrected(FOREST_DRIVE_SPD); \

thread_sleep_for(10); \

} \

} while(0)

// ───────────────────────────────────────────────────────

if(is_up_group) {

pc.printf("[MOVE] Sequence UP to Block %d\r\n", block);

pc.printf("  -> Cyl5+Cyl6 EXTEND\r\n");

lprintf(1,0,"C5+C6 EXTEND    ");

cyl5_extend(); cyl6_extend();

thread_sleep_for(CLIMB_FRONT_MS);

pc.printf("  -> FWD 40cm\r\n");

lprintf(1,0,"FWD 40cm...     ");

FWD_40CM();  // ← CHANGED

pc.printf("  -> Cyl5 RETRACT\r\n");

lprintf(1,0,"C5 RETRACT      ");

cyl5_retract();

thread_sleep_for(CLIMB_FRONT_MS);

pc.printf("  -> FWD 40cm\r\n");

lprintf(1,0,"FWD 40cm...     ");

FWD_40CM();  // ← CHANGED

pc.printf("  -> Cyl6 RETRACT\r\n");

lprintf(1,0,"C6 RETRACT      ");

cyl6_retract();

thread_sleep_for(CLIMB_BACK_MS);

pc.printf("  -> FWD 40cm\r\n");

lprintf(1,0,"FWD 40cm...     ");

FWD_40CM();  // ← CHANGED

} else if(is_down_group) {

pc.printf("[MOVE] Sequence DOWN to Block %d\r\n", block);

pc.printf("  -> Cyl5+Cyl6 RETRACT\r\n");

lprintf(1,0,"C5+C6 RETRACT   ");

cyl5_retract(); cyl6_retract();

thread_sleep_for(CLIMB_FRONT_MS);

pc.printf("  -> FWD 40cm\r\n");

lprintf(1,0,"FWD 40cm...     ");

FWD_40CM();  // ← CHANGED

pc.printf("  -> Cyl5 EXTEND\r\n");

lprintf(1,0,"C5 EXTEND       ");

cyl5_extend();

thread_sleep_for(CLIMB_FRONT_MS);

pc.printf("  -> FWD 40cm\r\n");

lprintf(1,0,"FWD 40cm...     ");

FWD_40CM();  // ← CHANGED

pc.printf("  -> Cyl6 EXTEND\r\n");

lprintf(1,0,"C6 EXTEND       ");

cyl6_extend();

thread_sleep_for(CLIMB_BACK_MS);

pc.printf("  -> FWD 40cm\r\n");

lprintf(1,0,"FWD 40cm...     ");

FWD_40CM();  // ← CHANGED

} else {

pc.printf("[MOVE] Block %d not in up/down group — FWD 40cm\r\n", block);

lprintf(1,0,"FWD 40cm...     ");

FWD_40CM();  // ← CHANGED

}

}

// ── forest_box_action: CYL3/CAN10/CYL4 grab sequence at current block ────

//   Works only when a box is registered for the block (caller decides).

void forest_box_action(int block) {

pc.printf("[BOX] Executing sequence for Block %d\r\n", block);

bool is_up_group   = (block == 1 || block == 2 || block == 3 ||

block == 5 || block == 6 || block == 7 ||

block == 8 || block == 11);

bool is_down_group = (block == 4 || block == 9 ||

block == 10 || block == 12);

// ── Inline 50cm CAN10 move using encoder feedback ──

#define CAN10_50CM(_dir) do { \

__disable_irq(); \

enc_left = 32000; last_pos[4] = 2048; first_rx[4] = true; \

__enable_irq(); \

int32_t _target = (int32_t)(0.50f * TICKS_PER_M) * MOTOR_DIR_SIGN * (_dir); \

uint32_t _t0 = us_ticker_read()/1000; \

while(true) { \

while(can.read(rx_msg)) { \

if(rx_msg.len<6 || rx_msg.id!=(CAN_GRIP+0x100)) continue; \

uint16_t _pos = ((uint16_t)rx_msg.data[1]<<4)|(rx_msg.data[2]>>4); \

if(_pos>4095) continue; \

if(first_rx[4]){ last_pos[4]=_pos; first_rx[4]=false; continue; } \

int16_t _d = (int16_t)_pos - (int16_t)last_pos[4]; \

if(_d>2048) _d-=4096; if(_d<-2048) _d+=4096; \

last_pos[4]=_pos; enc_left += (int32_t)_d; \

} \

int32_t _enc = enc_left - 32000; \

bool _done = (_target<0) ? (_enc<=_target) : (_enc>=_target); \

if(_done || (us_ticker_read()/1000 - _t0 > 3000)) { can10_send_vel(0.f); break; } \

can10_send_vel(BOX_GRIP_SPD * (_dir)); \

thread_sleep_for(10); \

} \

} while(0)

// ───────────────────────────────────────────────────────

if(is_up_group) {

pc.printf("[BOX] UP SEQUENCE\r\n");

pc.printf("  -> CYL3 UP\r\n");

cyl_up = 1;

thread_sleep_for(500);

pc.printf("  -> CAN10 +50 cm\r\n");

lprintf(1,0,"CAN10 +50cm...  ");

CAN10_50CM(1);  // ← CHANGED

lprintf(1,0,"OK              ");

pc.printf("  -> CYL4 UP\r\n");

cyl_dir = 1;

thread_sleep_for(500);

pc.printf("  -> CAN10 -50 cm\r\n");

lprintf(1,0,"CAN10 -50cm...  ");

CAN10_50CM(-1);  // ← CHANGED

lprintf(1,0,"OK              ");

pc.printf("  -> CYL4 DOWN\r\n");

cyl_dir = 0;

thread_sleep_for(500);

cyl_up = 0;

} else if(is_down_group) {

pc.printf("[BOX] DOWN SEQUENCE\r\n");

pc.printf("  -> CYL3 DOWN\r\n");

cyl_up = 0;

thread_sleep_for(500);

pc.printf("  -> CAN10 +50 cm\r\n");

lprintf(1,0,"CAN10 +50cm...  ");

CAN10_50CM(1);  // ← CHANGED

lprintf(1,0,"OK              ");

pc.printf("  -> CYL4 UP\r\n");

cyl_dir = 1;

thread_sleep_for(500);

pc.printf("  -> CAN10 -50 cm\r\n");

lprintf(1,0,"CAN10 -50cm...  ");

CAN10_50CM(-1);  // ← CHANGED

lprintf(1,0,"OK              ");

pc.printf("  -> CYL4 DOWN\r\n");

cyl_dir = 0;

thread_sleep_for(500);

cyl_up = 1;

} else {

pc.printf("[BOX] No action defined for block %d\r\n", block);

}

}

void forest_execute_step(int idx) {

int   block   = forest_path_blocks[idx];

int   presses = forest_path_presses[idx];

float h_target = ir_get_height(block);

int   box_act  = ir_get_box_action(block);

pc.printf("\r\n[FOREST STEP %d/%d] B%d presses=%d h=%.0fcm box_act=%+d\r\n",

idx+1, forest_path_len, block, presses, (double)h_target, box_act);

lprintf(0,0,"S%d/%d B%d H%.0f", idx+1, forest_path_len, block, (double)h_target);

forest_adjust_height(h_target);

if(box_act != 0 && forest_should_grab_box(block)) {

forest_grab_box(box_act);

}

if      (presses == 2) forest_turn_right();

else if (presses >= 3) forest_turn_left();

forest_robot.current_block = block;

pc.printf("[FOREST STEP %d] DONE block=%d h=%.0f dir=%d\r\n",

idx+1, forest_robot.current_block,

(double)forest_robot.current_height_cm, forest_robot.direction);

thread_sleep_for(250);

}

void forest_run_path(int zone) {

pc.printf("\r\n[FOREST] === RUN PATH zone=%s len=%d ===\r\n",

(zone==ZONE_RED)?"RED":"BLUE", forest_path_len);

if(!forest_path_confirmed || forest_path_len == 0) {

pc.printf("[FOREST] no path — skipping\r\n");

lprintf(0,0,"NO FOREST PATH  ");

lprintf(1,0,"Skipping...     ");

thread_sleep_for(800);

return;

}

pc.printf("[FOREST] path: ");

for(int i=0;i<forest_path_len;i++)

pc.printf("B%d(x%d) ", forest_path_blocks[i], forest_path_presses[i]);

pc.printf("\r\n");

forest_state_init(zone);

for(int i=0; i<forest_path_len; i++) {

forest_execute_step(i);

led = !led;

}

motors_stop();

cyl56_safe();

pc.printf("[FOREST] === PATH COMPLETE ===\r\n");

lprintf(0,0,"FOREST DONE     ");

lprintf(1,0,"All steps OK    ");

thread_sleep_for(1000);

}

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 9 — OPERATION MODE                                              ║

// ╚══════════════════════════════════════════════════════════════════════════╝

int btn_a_prev=1, btn_b_prev=1, ir_prev=1;

uint32_t btn_a_last_press=0;

bool btn_a_edge() {

int n=btn_a.read();

uint32_t now=us_ticker_read()/1000;

bool e=(btn_a_prev==1 && n==0);

if(e && (now-btn_a_last_press)<150) e=false;

if(e){ btn_a_last_press=now; pc.printf("[DBG] btn_a\r\n"); }

btn_a_prev=n;

return e;

}

bool btn_b_edge() {

int n=btn_b.read();

bool e=(btn_b_prev==1 && n==0);

btn_b_prev=n;

if(e){ pc.printf("[DBG] btn_b\r\n"); thread_sleep_for(50); }

return e;

}

bool ir_edge() {

int n=ir_sens.read();

bool e=(ir_prev==1 && n==0);

ir_prev=n;

if(e) thread_sleep_for(30);

return e;

}

const char* MODE_ROW0[NUM_MODES] = {"0:NORMAL RUN    ","1:RETRY MC      ","2:RETRY ARENA   ","3:TEST SENSORS  ","4:TEST ACTUATORS"};

const char* MODE_ROW1_RED[NUM_MODES] = {">RED FULL+FOREST",">RED->FOREST    ",">RED->ARENA KFS ",">IR=ENTER B=EXIT",">IR=ENTER B=EXIT"};

const char* MODE_ROW1_BLUE[NUM_MODES]= {">BLU FULL+FOREST",">BLU->FOREST    ",">BLU->ARENA KFS ",">IR=ENTER B=EXIT",">IR=ENTER B=EXIT"};

void test_sensors() {

pc.printf("[TEST_SENS]\r\n"); int page=0;

const char* PN[]={"PG0:SHARP  ","PG1:ROT    ","PG2:INPUTS "};

lprintf(0,0,"%s",PN[page]); lprintf(1,0,"A=page B=exit  "); thread_sleep_for(500);

while(true) {

if(btn_b_edge()){ lcd_clr(); return; }

if(btn_a_edge()){ page=(page+1)%3; lprintf(0,0,"%s",PN[page]); }

float d1=read_cm(sharp1,S1_K,S1_OFF), d2=read_cm(sharp2,S2_K,S2_OFF);

const char* s1z=spear_detected(d1)?"SPEAR":"EMPTY", *s2z=rack_in_range(d2)?"RACK!":"far";

switch(page){

case 0: lprintf(0,0,"S1:%5.1fcm %-5s",d1,s1z); lprintf(1,0,"S2:%5.1fcm %-5s",d2,s2z); break;

case 1: lprintf(0,0,"PA0:%d PA1:%d    ",sw_90.read(),sw_home.read()); lprintf(1,0,"C5a:%d C6a:%d    ",cyl5_a.read(),cyl6_a.read()); break;

case 2: lprintf(0,0,"ZN:%-4s IR:%d    ",(zone_sw.read()==0)?"RED":"BLUE",ir_sens.read()); lprintf(1,0,"A=page B=exit  "); break;

}

thread_sleep_for(400);

}

}

void test_actuators() {

const char* AN[]={"C1 DOWN","C1 UP","C2 CLOSE","C2 OPEN","C4 UP","C4 DOWN","C5 EXT","C5 RET","C6 EXT","C6 RET","ROT HOME","ROT 90","CAN10 +1m","CAN10 -1m"};

int act=0; lprintf(0,0,"%s",AN[act]); lprintf(1,0,"IR=fire B=exit ");

while(true) {

if(btn_b_edge()){ motors_stop(); cyl56_safe(); lcd_clr(); return; }

if(btn_a_edge()){ act=(act+1)%14; lprintf(0,0,"%s",AN[act]); }

if(ir_edge()) {

lprintf(1,0,"FIRING...       ");

switch(act){

case 0: cyl_arm=1; thread_sleep_for(CYL_ARM_MS); break;

case 1: cyl_arm=0; thread_sleep_for(CYL_ARM_MS); break;

case 2: cyl_grip=1; thread_sleep_for(GRIP_SETTLE_MS); break;

case 3: cyl_grip=0; thread_sleep_for(GRIP_SETTLE_MS); break;

case 4: cyl_dir=1; thread_sleep_for(CYL4_MS); break;

case 5: cyl_dir=0; thread_sleep_for(CYL4_MS); break;

case 6: cyl5_extend(); thread_sleep_for(CLIMB_FRONT_MS); break;

case 7: cyl5_retract(); thread_sleep_for(CLIMB_FRONT_MS); break;

case 8: cyl6_extend(); thread_sleep_for(CLIMB_BACK_MS); break;

case 9: cyl6_retract(); thread_sleep_for(CLIMB_BACK_MS); break;

case 12: move_can10_metres( BOX_GRIP_MOVE_M, BOX_GRIP_SPD); break;

case 13: move_can10_metres(-BOX_GRIP_MOVE_M, BOX_GRIP_SPD); break;

}

lprintf(1,0,"DONE            ");

}

thread_sleep_for(80); led=!led;

}

}

void op_mode() {

g_mode = MODE_NORMAL;

g_zone = (zone_sw.read()==0) ? ZONE_RED : ZONE_BLUE;

pc.printf("[OP] zone=%s\r\n",(g_zone==ZONE_RED)?"RED":"BLUE");

lprintf(0,0,"%s",MODE_ROW0[g_mode]);

lprintf(1,0,"%s",(g_zone==ZONE_RED)?MODE_ROW1_RED[g_mode]:MODE_ROW1_BLUE[g_mode]);

while(true) {

g_zone = (zone_sw.read()==0) ? ZONE_RED : ZONE_BLUE;

if(btn_a_edge()){

g_mode=(g_mode+1)%NUM_MODES;

lprintf(0,0,"%s",MODE_ROW0[g_mode]);

lprintf(1,0,"%s",(g_zone==ZONE_RED)?MODE_ROW1_RED[g_mode]:MODE_ROW1_BLUE[g_mode]);

}

lprintf(1,0,"%s",(g_zone==ZONE_RED)?MODE_ROW1_RED[g_mode]:MODE_ROW1_BLUE[g_mode]);

if(ir_edge()){

if(g_mode==MODE_TEST_SENS){ thread_sleep_for(300); test_sensors(); g_mode=MODE_NORMAL; continue; }

if(g_mode==MODE_TEST_ACT) { thread_sleep_for(300); test_actuators(); g_mode=MODE_NORMAL; continue; }

break;

}

if(btn_b_edge()){ g_mode=MODE_NORMAL; continue; }

led=!led; thread_sleep_for(50);

}

while(true){

const char* zn=(g_zone==ZONE_RED)?"RED ":"BLU";

lprintf(0,0,"OK? %s M%d       ",zn,g_mode);

lprintf(1,0,"IR=GO  B=CANCEL ");

if(btn_b_edge()){ lprintf(0,0,"CANCELLED       "); thread_sleep_for(600); return; }

if(ir_edge()){ lprintf(0,0,"STARTING...     "); thread_sleep_for(300); led=1; return; }

led=!led; thread_sleep_for(100);

}

}

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 10 — STATE NAMES                                                ║

// ╚══════════════════════════════════════════════════════════════════════════╝

#define S7_ENTRY  0

#define S7_CLIMB  1

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 11A — RED SEQUENCE (UPDATED)                                    ║

// ╚══════════════════════════════════════════════════════════════════════════╝

void run_normal_red() {

pc.printf("\r\n[RUN] RED: FWD->LEFT->FWD->diag-RIGHT-BACK->spin-CCW\r\n");

const char* SN[] = {

"?       ","INIT    ","APPROACH","SWEEP   ",

"PICKUP  ","BACK    ","ROT_90  ","CYL14   ","HOLD    "

};

int sub4=S4_FWD, sub5=0, sub8=0;

uint32_t sub_t0=0;

int8_t  mode=0;

bool    op=true, first=true;

float   spd=0.f;

int     conf=0, ptick=0;

uint32_t t0=us_ticker_read()/1000;

auto sms=[&]()->uint32_t{ return us_ticker_read()/1000-t0; };

auto enter=[&](int8_t s){

const char* from=(mode>=0&&mode<=8)?SN[mode]:"?";

const char* to  =(s   >=0&&s   <=8)?SN[s]   :"?";

pc.printf("\r\n[STATE-RED] %s -> %s  EL:%ld ER:%ld t:%lus\r\n",

from,to,(long)enc_left,(long)enc_right,(unsigned long)sms()/1000);

mode=s; t0=us_ticker_read()/1000; conf=0; first=true; spd=0.f;

motors_stop(); encoders_reset();

lprintf(0,0,"R:%-14s",to);

lprintf(1,0,"                ");

};

encoders_reset();

enter(1);

while(op){

float d1=read_cm(sharp1,S1_K,S1_OFF);

float d2=read_cm(sharp2,S2_K,S2_OFF);

int32_t target_ticks = 0;  // ← Add this line

switch(mode){

case 1:{   // INIT

if(first){

first=false;

lprintf(1,0,"C1DN C2OPN C4=0 ");

cyl_arm=1; cyl_grip=0; cyl_dir=0;

thread_sleep_for(CYL_ARM_MS);

lprintf(1,0,"Rot->HOME RED   ");

lprintf(1,0,"INIT OK HOME=%d ",sw_home.read());

thread_sleep_for(300); enter(2);

}

break;

}

case 2: {   // APPROACH — METRE-BASED FORWARD (encoder feedback, simple structure)

if (first) {

first = false;

t0 = us_ticker_read() / 1000;

encoders_reset();

// Convert target metres → encoder ticks with direction sign

target_ticks = (int32_t)(APPROACH_METRES * TICKS_PER_M) * MOTOR_DIR_SIGN;

pc.printf("[CASE 2] APPROACH: %.3fm = %+ld ticks\r\n",

(double)APPROACH_METRES, (long)target_ticks);

lprintf(1, 0, "FWD %.3fm...    ", (double)APPROACH_METRES);

}

// Read current encoder position (average of left+right wheels)

int32_t enc_avg = (enc_left + enc_right) / 2;

int32_t error = target_ticks - enc_avg;

float current_metres = (float)enc_avg / TICKS_PER_M * (float)MOTOR_DIR_SIGN;

// ── Stop: encoder reached/passed target (exact tick, no deadband) ──

bool reached = (target_ticks < 0) ? (enc_avg <= target_ticks) : (enc_avg >= target_ticks);

if (reached) {

motors_stop();

float overshoot_m = (float)(enc_avg - target_ticks) / TICKS_PER_M;

pc.printf("[CASE 2] DONE: %.3f m (err: %.3f mm)\r\n",

(double)fabsf(current_metres), (double)(overshoot_m * 1000.f));

lprintf(1, 0, "APPROACH OK     ");

thread_sleep_for(100);

enter(3);  // → Proceed to Case 3

break;

}

// ── Safety timeout (fallback if encoder fails) ───────────────────

uint32_t el = sms();

if (el > 8000) {  // 8 second hard limit

motors_stop();

pc.printf("[FAULT] Approach timeout at %.3f m\r\n", (double)fabsf(current_metres));

lprintf(1, 0, "TIMEOUT!        ");

op = false;

break;

}

// ── Speed control: simple P-loop + ramp limit ───────────────────

static float spd = 0.f;  // Static to maintain across loops

float cmd = 0.08f * (float)abs((int)error);  // P-gain = 0.08

if (cmd > APPROACH_SPD) cmd = APPROACH_SPD;   // Max speed cap

if (cmd < 0.8f && abs((int)error) > 20) cmd = 0.8f;  // Min to overcome friction

if (error < 0) cmd = -cmd;  // Reverse if overshoot

// Apply forward motion with straightening correction

drive_fwd_corrected(cmd);

// ── Live LCD/Serial feedback (every ~100ms) ─────────────────────

static uint32_t last_print = 0;

if (el - last_print >= 100) {

last_print = el;

lprintf(1, 0, "%.3f/%.3fm  ", (double)fabsf(current_metres), (double)APPROACH_METRES);

// Optional serial debug:

// pc.printf("  enc=%+ld err=%+ld spd=%.2f m=%.3f\r\n",

//           (long)enc_avg, (long)error, (double)cmd, (double)fabsf(current_metres));

}

break;

}

case 3:{   // SWEEP — LEFT (row 2)  ← RED

if(first) {

first = false;

pc.printf("[SWEEP] strafe LEFT\r\n");

lprintf(1,0,"SWEEP LEFT...   ");

}

// Print S1 every 500ms

if(sms() % 500 < 15) {

pc.printf("[SWEEP] t=%lums S1=%.1fcm thresh=%.1fcm\r\n",

(unsigned long)sms(), (double)d1, (double)S1_SPEAR_CM);

}

// Stop and go to PICKUP if anything within 30cm

if(d1 < S1_SPEAR_CM && d1 > 1.f) {

motors_stop();

pc.printf("[SWEEP] detected! S1=%.1fcm — stop and enter PICKUP\r\n",

(double)d1);

lprintf(1,0,"DET %.1fcm      ", d1);

thread_sleep_for(100);

sub4 = S4_FWD;

enter(4);

break;

}

// Keep going left

motors_drive(2, STRAFE_SPD);   // row 2 = LEFT

lprintf(1,0,"S1:%.1fcm       ", d1);

break;

}

case 4:{   // PICKUP

switch(sub4){

case S4_FWD:{

if(first){ first=false; sub_t0=us_ticker_read()/1000; motors_stop(); encoders_reset(); lprintf(1,0,"FWD 1600ms...   "); }

uint32_t el=us_ticker_read()/1000-sub_t0;

if(el>=1600){ motors_stop(); sub4=S4_CYL2_EXT; sub_t0=us_ticker_read()/1000; first=true; break; }

spd+=RAMP_STEP; if(spd>FWD_SPEAR_SPD) spd=FWD_SPEAR_SPD;

drive_fwd_corrected(spd);

lprintf(1,0,"FWD %lu/1600ms  ",(unsigned long)el); break;

}

case S4_CYL2_EXT:{

if(first){ first=false; sub_t0=us_ticker_read()/1000; cyl_grip=1; lprintf(1,0,"CYL2 extend...  "); }

uint32_t el=us_ticker_read()/1000-sub_t0;

lprintf(1,0,"CYL2 %lu/%lums  ",(unsigned long)el,(unsigned long)CYL2_EXTEND_MS);

if(el>=CYL2_EXTEND_MS){ sub4=S4_CYL1_RET; sub_t0=us_ticker_read()/1000; first=true; } break;

}

case S4_CYL1_RET:{

if(first){ first=false; sub_t0=us_ticker_read()/1000; cyl_arm=0; lprintf(1,0,"CYL1 retract... "); }

uint32_t el=us_ticker_read()/1000-sub_t0;

lprintf(1,0,"CYL1 %lu/%lums  ",(unsigned long)el,(unsigned long)CYL1_RETRACT_MS);

if(el>=CYL1_RETRACT_MS){ pc.printf("[PICKUP-RED] done C1=%d C2=%d\r\n",cyl_arm.read(),cyl_grip.read()); lprintf(1,0,"PICKUP OK RED!  "); thread_sleep_for(200); enter(5); } break;

}

default: pc.printf("[FAULT-RED] bad sub4=%d\r\n",sub4); op=false; break;

}

break;

}

case 5:{   // BACK — diagonal RIGHT-back (sub0) + spin CCW (sub1)  ← RED

switch(sub5){

case 0:{

if(first){ first=false; pc.printf("[BACK-RED sub0] diag RIGHT-back %lums\r\n",(unsigned long)DIAG_MS); lprintf(1,0,"DIAG R-BACK RED "); t0=us_ticker_read()/1000; }

if(sms()>DIAG_MS+3000){ pc.printf("[FAULT-RED] diag timeout\r\n"); op=false; break; }

if(sms()>=DIAG_MS){ motors_stop(); lprintf(1,0,"DIAG done RED   "); thread_sleep_for(80); sub5=1; t0=us_ticker_read()/1000; first=true; break; }

drive_diagonal_BR(MOVE_SPD);   // +spd = diagonal RIGHT-back  ← RED

lprintf(1,0,"DIAGR t:%lu/%lu ",(unsigned long)sms(),(unsigned long)DIAG_MS);

break;

}

case 1:{

if(first){ first=false; pc.printf("[BACK-RED sub1] spin CCW %lums\r\n",(unsigned long)TURN_45_MS); lprintf(1,0,"SPIN CCW RED    "); t0=us_ticker_read()/1000; }

if(sms()>TURN_45_MS+500){ pc.printf("[FAULT-RED] spin timeout\r\n"); op=false; break; }

if(sms()>=TURN_45_MS){ motors_stop(); lprintf(1,0,"SPIN done RED   "); thread_sleep_for(200); sub5=0; enter(6); break; }

drive_spin(MOVE_SPD*2);   // +spd = CCW  ← RED

lprintf(1,0,"CCW t:%lu/%lu   ",(unsigned long)sms(),(unsigned long)TURN_45_MS);

break;

}

default: pc.printf("[FAULT-RED] bad sub5=%d\r\n",sub5); op=false; break;

}

break;

}

case 6:{   // ROT_90 → then enter FOREST

if(first) {

first=false;

// ── TRANSITION TO FOREST OPERATIONS ──────────────────────

// Initialize forest state and start path execution

forest_state_init(ZONE_RED);  // Set start block/height for RED

forest_step_idx = 0;

forest_sub_step = 0;

forest_init_done = false;

// Check if we have a confirmed path to execute

if(!forest_path_confirmed || forest_path_len == 0) {

pc.printf("[RED-FOREST] No path confirmed — skipping forest section\r\n");

lprintf(0,0,"NO FOREST PATH  ");

lprintf(1,0,"Skip to DONE    ");

thread_sleep_for(500);

enter(9);  // Skip to DONE

} else {

pc.printf("[RED-FOREST] Starting forest nav: %d steps\r\n", forest_path_len);

lprintf(0,0,"FOREST NAV RED  ");

lprintf(1,0,"Step 1/%d...    ", forest_path_len);

enter(7);  // → Enter FOREST_STEP case

}

}

break;

}

case 7: {   // FOREST PATH EXECUTION — recall stored IR path

static int forest_step = 0;

if(first) {

first = false;

forest_step = 0;

pc.printf("\r\n[FOREST] === EXECUTE STORED PATH (%d steps) ===\r\n",

forest_path_len);

// Print full path so the operator can verify

pc.printf("  Path: ");

for(int i = 0; i < forest_path_len; i++) {

pc.printf("%d", forest_path_blocks[i]);

if(forest_path_presses[i] == 2)

{

pc.printf("R");//rot cmd

}

else if(forest_path_presses[i] >= 3)

{

pc.printf("L");

}

pc.printf(" ");

}

pc.printf("\r\n");

// Print registered boxes

pc.printf("  Boxes registered: ");

if(forest_box_count == 0) pc.printf("(none)");

for(int i = 0; i < forest_box_count; i++) {

pc.printf("B%d(%s) ", forest_box_blocks[i],

(forest_box_actions[i] == 1) ? "UP" : "DN");

}

pc.printf("\r\n\r\n");

lprintf(0,0,"FOREST RUN      ");

lprintf(1,0,"%d steps queued ", forest_path_len);

thread_sleep_for(400);

}

// ── Per-step execution ───────────────────────────────────────────

if(forest_step < forest_path_len) {

int block   = forest_path_blocks[forest_step];

int presses = forest_path_presses[forest_step];

int box_act = 0;

bool has_box = forest_block_has_box(block, &box_act);

pc.printf("\r\n[STEP %d/%d] Block=%d  presses=%d  box=%s\r\n",

forest_step + 1, forest_path_len,

block, presses, has_box ? "YES" : "no");

lprintf(0,0,"S%d/%d B%-2d %s",

forest_step + 1, forest_path_len, block,

has_box ? "+BOX" : "    ");

// ── 1. Always: move to the block ─────────────────────────────

pc.printf("  [1] forest_move_to_block(%d)\r\n", block);

lprintf(1,0,"MOVE -> B%d      ", block);

forest_move_to_block(block);

// ── 2. If box registered: grab it, then move past ────────────

if(has_box) {

pc.printf("  [2] forest_box_action(%d)  (act=%+d)\r\n",

block, box_act);

lprintf(1,0,"BOX action B%d   ", block);

forest_box_action(block);

pc.printf("  [3] forest_move_to_block(%d)  (continue past)\r\n",

block);

lprintf(1,0,"MOVE past B%d    ", block);

forest_move_to_block(block);

}

if(presses == 2) {

pc.printf("  [4] TURN RIGHT  (presses=2)\r\n");

lprintf(1,0,"TURN RIGHT      ");

forest_turn_right();

} else if(presses >= 3) {

pc.printf("  [4] TURN LEFT   (presses=%d)\r\n", presses);

lprintf(1,0,"TURN LEFT       ");

forest_turn_left();

} else {

pc.printf("  [4] STRAIGHT    (presses=1)\r\n");

}

forest_state.block = block;

forest_step++;

thread_sleep_for(200);

} else {

// ── All steps done ────────────────────────────────────────────

pc.printf("\r\n[FOREST] === ALL %d STEPS COMPLETE ===\r\n",

forest_path_len);

lprintf(0,0,"FOREST DONE     ");

lprintf(1,0,"All steps OK    ");

forest_step = 0;        // reset for next run

motors_stop();

cyl56_safe();

thread_sleep_for(500);

enter(9);                // → DONE state

}

break;

}

case 8:{   // FOREST NAVIGATE — execute IR path

if(first){

first=false;

pc.printf("[RED-FOREST] NAV: %d steps\r\n", forest_path_len);

forest_run_path(ZONE_RED);

thread_sleep_for(500);

enter(9);  // → DONE

}

break;

}

case 9:{   // DONE

cyl_grip=0; cyl_dir=0; cyl56_safe();

lprintf(0,0,"DONE RED        ");

lprintf(1,0,"Sequence OK     ");

pc.printf("[RED] === SEQUENCE COMPLETE ===\r\n");

op=false;

break;

}

default: op=false; break;

}

ptick++;

if(ptick>=50){

ptick=0;

const char* sn=(mode>=1&&mode<=8)?SN[mode]:"?";

pc.printf("[RED %-8s %2lus] S1:%.1f EL:%ld ER:%ld spd:%.2f C1:%d C2:%d C4:%d s4:%d s7:%d\r\n",

sn,(unsigned long)sms()/1000,(double)d1,(long)enc_left,(long)enc_right,

(double)spd,cyl_arm.read(),cyl_grip.read(),cyl_dir.read(),sub4);

}

thread_sleep_for(10);

}

motors_stop();

pc.printf("[RUN] run_normal_red complete\r\n");

}

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 11B — BLUE SEQUENCE                                             ║

// ╚══════════════════════════════════════════════════════════════════════════╝

void run_normal_blue() {

pc.printf("\r\n[RUN-BLU] FWD->RIGHT->FWD->BACK->ROT90 → FOREST PATH\r\n");

const char* SN[]={"?","INIT","APPROACH","SWEEP","PICKUP","BACK","ROT_90","FOREST_E","FOREST_N","DONE"};

int sub4=S4_FWD, sub5=0;

uint32_t sub_t0=0;

int8_t  mode=0;

bool    op=true, first=true;

float   spd=0.f;

int     ptick=0;

uint32_t t0=us_ticker_read()/1000;

auto sms=[&]()->uint32_t{ return us_ticker_read()/1000-t0; };

auto enter=[&](int8_t s){

const char* from=(mode>=0&&mode<=9)?SN[mode]:"?";

const char* to  =(s   >=0&&s   <=9)?SN[s]   :"?";

pc.printf("\r\n[BLU] %s -> %s\r\n",from,to);

mode=s; t0=us_ticker_read()/1000; first=true; spd=0.f;

motors_stop(); encoders_reset();

lprintf(0,0,"B:%-14s",to); lprintf(1,0,"                ");

};

encoders_reset();

enter(1);

while(op) {

float d1=read_cm(sharp1,S1_K,S1_OFF);

switch(mode){

case 1:{

if(first){

first=false;

cyl_arm=1; cyl_grip=0; cyl_dir=0; cyl56_safe();

thread_sleep_for(CYL_ARM_MS);

thread_sleep_for(300); enter(2);

}

break;

}

case 2:{

if(first){ first=false; bool ok=move_forward_metres(APPROACH_METRES,APPROACH_SPD); if(ok) enter(3); else op=false; }

break;

}

case 3:{

if(first){ first=false; lprintf(1,0,"SWEEP RIGHT...  "); }

if(d1<S1_SPEAR_CM && d1>1.f){ motors_stop(); thread_sleep_for(100); sub4=S4_FWD; enter(4); break; }

if(sms()>SWEEP_MAX_MS){ motors_stop(); op=false; break; }

motors_drive(0, STRAFE_SPD);

break;

}

case 4:{

switch(sub4){

case S4_FWD:{

if(first){ first=false; sub_t0=us_ticker_read()/1000; encoders_reset(); }

if(us_ticker_read()/1000-sub_t0>=1800){ motors_stop(); sub4=S4_CYL2_EXT; sub_t0=us_ticker_read()/1000; first=true; break; }

spd+=RAMP_STEP; if(spd>FWD_SPEAR_SPD) spd=FWD_SPEAR_SPD;

drive_fwd_corrected(spd); break;

}

case S4_CYL2_EXT:{

if(first){ first=false; sub_t0=us_ticker_read()/1000; cyl_grip=1; }

if(us_ticker_read()/1000-sub_t0>=CYL2_EXTEND_MS){ sub4=S4_CYL1_RET; sub_t0=us_ticker_read()/1000; first=true; } break;

}

case S4_CYL1_RET:{

if(first){ first=false; sub_t0=us_ticker_read()/1000; cyl_arm=0; }

if(us_ticker_read()/1000-sub_t0>=CYL1_RETRACT_MS){ thread_sleep_for(200); enter(5); } break;

}

}

break;

}

case 5:{

switch(sub5){

case 0:{

if(first){ first=false; t0=us_ticker_read()/1000; }

if(sms()>=DIAG_MS){ motors_stop(); sub5=1; t0=us_ticker_read()/1000; first=true; break; }

drive_diagonal_BL(MOVE_SPD*1.5f); break;

}

case 1:{

if(first){ first=false; t0=us_ticker_read()/1000; }

if(sms()>=TURN_45_MS){ motors_stop(); sub5=0; enter(6); break; }

drive_spin(-MOVE_SPD*2); break;

}

}

break;

}

case 6:{

switch(sub5){

case 0:{

if(first){ first=false; t0=us_ticker_read()/1000; }

if(sms()>=DIAG_MS){ motors_stop(); sub5=1; t0=us_ticker_read()/1000; first=true; break; }

drive_diagonal_BL(MOVE_SPD*1.5f); break;

}

case 1:{

if(first){ first=false; t0=us_ticker_read()/1000; }

if(sms()>=TURN_45_MS){ motors_stop(); sub5=0; enter(6); break; }

drive_spin(-MOVE_SPD*2); break;

}

}

break;

}

case 7: {   // FOREST PATH EXECUTION — recall stored IR path

static int forest_step = 0;

if(first) {

first = false;

forest_step = 0;

pc.printf("\r\n[FOREST] === EXECUTE STORED PATH (%d steps) ===\r\n",

forest_path_len);

// Print full path so the operator can verify

pc.printf("  Path: ");

for(int i = 0; i < forest_path_len; i++) {

pc.printf("%d", forest_path_blocks[i]);

if(forest_path_presses[i] == 2) pc.printf("R");

else if(forest_path_presses[i] >= 3) pc.printf("L");

pc.printf(" ");

}

pc.printf("\r\n");

// Print registered boxes

pc.printf("  Boxes registered: ");

if(forest_box_count == 0) pc.printf("(none)");

for(int i = 0; i < forest_box_count; i++) {

pc.printf("B%d(%s) ", forest_box_blocks[i], (forest_box_actions[i] == 1) ? "UP" : "DN");

}

pc.printf("\r\n\r\n");

lprintf(0,0,"FOREST RUN      ");

lprintf(1,0,"%d steps queued ", forest_path_len);

thread_sleep_for(400);

}

// ── Per-step execution ───────────────────────────────────────────

if(forest_step < forest_path_len) {

int block   = forest_path_blocks[forest_step];

int presses = forest_path_presses[forest_step];

int box_act = 0;

bool has_box = forest_block_has_box(block, &box_act);

pc.printf("\r\n[STEP %d/%d] Block=%d  presses=%d  box=%s\r\n",

forest_step + 1, forest_path_len,

block, presses, has_box ? "YES" : "no");

lprintf(0,0,"S%d/%d B%-2d %s",  forest_step + 1, forest_path_len, block,  has_box ? "+BOX" : "    ");

// ── 1. Always: move to the block ─────────────────────────────

pc.printf("  [1] forest_move_to_block(%d)\r\n", block);

lprintf(1,0,"MOVE -> B%d      ", block);

forest_move_to_block(block);

// ── 2. If box registered: grab it, then move past ────────────

if(has_box) {

pc.printf("  [2] forest_box_action(%d)  (act=%+d)\r\n",

block, box_act);

lprintf(1,0,"BOX action B%d   ", block);

forest_box_action(block);

pc.printf("  [3] forest_move_to_block(%d)  (continue past)\r\n",

block);

lprintf(1,0,"MOVE past B%d    ", block);

forest_move_to_block(block);

}

if(presses == 2) {

pc.printf("  [4] TURN RIGHT  (presses=2)\r\n");

lprintf(1,0,"TURN RIGHT      ");

forest_turn_right();

} else if(presses >= 3) {

pc.printf("  [4] TURN LEFT   (presses=%d)\r\n", presses);

lprintf(1,0,"TURN LEFT       ");

forest_turn_left();

} else {

pc.printf("  [4] STRAIGHT    (presses=1)\r\n");

}

forest_state.block = block;

forest_step++;

thread_sleep_for(200);

} else {

// ── All steps done ────────────────────────────────────────────

pc.printf("\r\n[FOREST] === ALL %d STEPS COMPLETE ===\r\n",

forest_path_len);

lprintf(0,0,"FOREST DONE     ");

lprintf(1,0,"All steps OK    ");

forest_step = 0;        // reset for next run

motors_stop();

cyl56_safe();

thread_sleep_for(500);

enter(9);                // → DONE state

}

break;

}

case 8:{

if(first){

first=false;

forest_run_path(ZONE_BLUE);

thread_sleep_for(500);

enter(9);

}

break;

}

case 9:{

cyl_grip=0; cyl_dir=0; cyl56_safe();

lprintf(0,0,"DONE BLUE       ");

lprintf(1,0,"Sequence OK     ");

pc.printf("[BLU] === SEQUENCE COMPLETE ===\r\n");

op=false;

break;

}

default: op=false; break;

}

ptick++;

if(ptick>=50){

ptick=0;

const char* sn=(mode>=1&&mode<=9)?SN[mode]:"?";

pc.printf("[BLU %-8s %2lus] H:%.0f B:%d\r\n",

sn,(unsigned long)sms()/1000,

(double)forest_robot.current_height_cm,forest_robot.current_block);

}

thread_sleep_for(10);

}

motors_stop(); cyl56_safe();

}

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 12 — DISPATCH                                                   ║

// ╚══════════════════════════════════════════════════════════════════════════╝

void run_retry_mc(int zone) {

lprintf(0,0,"RETRY MC        ");

lprintf(1,0,"%s direct forest",(zone==ZONE_RED)?"R":"B");

pc.printf("[RETRY_MC] zone=%s — direct forest using stored path\r\n",

(zone==ZONE_RED)?"RED":"BLUE");

forest_run_path(zone);

}

void run_retry_arn(int zone) {

lprintf(0,0,"RETRY ARENA     ");

lprintf(1,0,"%s->TTT         ",(zone==ZONE_RED)?"R":"B");

pc.printf("[RETRY_ARN] zone=%s — direct to arena (no forest path used)\r\n",

(zone==ZONE_RED)?"RED":"BLUE");

}

void dispatch(int zone, int mode) {

pc.printf("[DISPATCH] zone=%s mode=%d (path already loaded: %s len=%d)\r\n",

(zone==ZONE_RED)?"RED":"BLUE", mode,

forest_path_confirmed?"YES":"NO", forest_path_len);

if(zone==ZONE_RED){

switch(mode){

case MODE_NORMAL:    run_normal_red();          break;

case MODE_RETRY_MC:  run_retry_mc(ZONE_RED);    break;

case MODE_RETRY_ARN: run_retry_arn(ZONE_RED);   break;

default:             run_normal_red();          break;

}

} else {

switch(mode){

case MODE_NORMAL:    run_normal_blue();         break;

case MODE_RETRY_MC:  run_retry_mc(ZONE_BLUE);   break;

case MODE_RETRY_ARN: run_retry_arn(ZONE_BLUE);  break;

default:             run_normal_blue();         break;

}

}

}

// ╔══════════════════════════════════════════════════════════════════════════╗

// ║  SECTION 13 — MAIN                                                       ║

// ╚══════════════════════════════════════════════════════════════════════════╝

int main() {

cyl_arm=0; cyl_grip=0; cyl_dir=0;

cyl56_safe();

rot_pwm.period(0.0001f);

bool lcd_ok=lcd_detect();

if(lcd_ok) lcd_init();

else pc.printf("[WARN] LCD missing\r\n");

thread_sleep_for(50);

pc.printf("\r\n=== R2 Robocon 2026 — PATH INPUT FIRST ===\r\n");

pc.printf("ZONE PC3=%d (%s)\r\n", zone_sw.read(), (zone_sw.read()==0)?"RED":"BLUE");

pc.printf("FLOW: IR PATH INPUT → op_mode → cases 1-6 (spear) → cases 7-9 (forest)\r\n");

if(can.frequency(1000000)==0){

pc.printf("[FATAL] CAN init failed\r\n");

while(1){ led=!led; thread_sleep_for(100); }

}

can.attach(&on_can_rx, CAN::RxIrq);

can.filter(0x100, 0x700, CANStandard, 0);

for(int i=0;i<NUM_W;i++){ can_enter_mode(CAN_FL+i); pc.printf("[OK] motor ID%d\r\n",CAN_FL+i); }

can_enter_mode(CAN_GRIP);

pc.printf("[OK] motor ID%d (grip)\r\n",CAN_GRIP);

motors_stop();

lprintf(0,0,"R2 Robocon 2026 ");

lprintf(1,0,"%s Ready        ",(zone_sw.read()==0)?"RED":"BLU");

thread_sleep_for(1000);

// ── MAIN GAME LOOP ───────────────────────────────────────────────────

while(true){

// ── STEP 1: IR PATH INPUT (RUNS FIRST) ───────────────────────────

pc.printf("\r\n========================================\r\n");

pc.printf("[FLOW] STEP 1: IR PATH INPUT\r\n");

pc.printf("========================================\r\n");

lprintf(0,0,"STEP 1: IR PATH ");

lprintf(1,0,"Use remote      ");

thread_sleep_for(500);

bool got_path = collect_forest_path();

if(!got_path) {

pc.printf("[FLOW] Path skipped (btn_b) — run will skip forest section\r\n");

lprintf(0,0,"PATH SKIPPED    ");

lprintf(1,0,"No forest run   ");

thread_sleep_for(800);

} else {

pc.printf("[FLOW] Path locked: %d steps confirmed=%d\r\n",

forest_path_len, forest_path_confirmed);

lprintf(0,0,"PATH READY      ");

lprintf(1,0,"%d steps stored ", forest_path_len);

thread_sleep_for(800);

}

// ── STEP 2: MODE SELECTION ───────────────────────────────────────

pc.printf("\r\n[FLOW] STEP 2: MODE SELECTION\r\n");

op_mode();

// ── STEP 3: EXECUTE ──────────────────────────────────────────────

pc.printf("\r\n[FLOW] STEP 3: EXECUTE  zone=%s mode=%d\r\n",

(g_zone==ZONE_RED)?"RED":"BLUE", g_mode);

dispatch(g_zone, g_mode);

// ── STEP 4: WAIT FOR NEXT ROUND ──────────────────────────────────

pc.printf("\r\n[FLOW] STEP 4: RUN COMPLETE — IR for next round\r\n\r\n");

lprintf(0,0,"RUN COMPLETE    ");

lprintf(1,0,"IR=new path     ");

while(ir_sens.read()==1){ led=!led; thread_sleep_for(300); }

thread_sleep_for(500);

}

}