Keyesir to ir remote first code

#include "mbed.h"

#define IR_PIN      PA_10

#define NEC_LEADER_MIN  8000

#define NEC_LEADER_MAX  10000

#define NEC_ONE_MIN     1500

DigitalIn  ir_pin(IR_PIN, PullUp);

DigitalOut led(LED1);

Serial     pc(USBTX, USBRX, 115200);

Timer      timer;

// All known keys

typedef struct {

uint8_t     code;

const char* label;

} KeyMap;

const KeyMap KEY_MAP[] = {

{ 0x00, "1" },

{ 0x01, "2" },

{ 0x02, "3" },

{ 0x04, "4" },

{ 0x05, "5" },

{ 0x06, "6" },

{ 0x08, "7" },

{ 0x09, "8" },

{ 0x0A, "9" },

// --- discover these by pressing each button and reading Unknown key: 0xXX ---

{ 0x0D, "0" },      // replace 0xFF with real code

{ 0x0C, "*" },      // replace 0xFF with real code

{ 0x0E, "#" },      // replace 0xFF with real code

{ 0x11, "UP" },     // replace 0xFF with real code

{ 0x19, "DOWN" },   // replace 0xFF with real code

{ 0x14, "LEFT" },   // replace 0xFF with real code

{ 0x16, "RIGHT" },  // replace 0xFF with real code

{ 0x15, "OK" },     // replace 0xFF with real code

};

const int KEY_COUNT = sizeof(KEY_MAP) / sizeof(KEY_MAP[0]);

int wait_for(int state, int timeout_us) {

timer.reset();

timer.start();

while (ir_pin.read() != state) {

if (timer.read_us() > timeout_us) { timer.stop(); return -1; }

}

int t = timer.read_us();

timer.stop();

return t;

}

uint32_t receive_nec() {

if (wait_for(0, 100000) < 0) return 0;

int leader_low = wait_for(1, 15000);

if (leader_low < NEC_LEADER_MIN || leader_low > NEC_LEADER_MAX) return 0;

int leader_high = wait_for(0, 6000);

if (leader_high < 3000 || leader_high > 6000) return 0;

uint32_t data = 0;

for (int i = 0; i < 32; i++) {

if (wait_for(1, 1000) < 0) return 0;

int space = wait_for(0, 2500);

if (space < 0) return 0;

if (space > NEC_ONE_MIN) data |= (1u << i);

}

wait_for(1, 1000);

return data;

}

int main() {

pc.printf("IR Remote ready - press any button\r\n");

while (1) {

uint32_t frame = receive_nec();

if (frame == 0) continue;

uint8_t cmd     = (frame >> 16) & 0xFF;

uint8_t cmd_inv = (frame >> 24) & 0xFF;

if ((uint8_t)(cmd ^ cmd_inv) != 0xFF) continue; // bad frame

// Look up in table

bool found = false;

for (int k = 0; k < KEY_COUNT; k++) {

if (cmd == KEY_MAP[k].code) {

pc.printf("%s\r\n", KEY_MAP[k].label);

led = !led;

found = true;

break;

}

}

// DISCOVERY MODE: prints unknown codes so you can fill in the table

if (!found) {

pc.printf("Unknown key: 0x%02X  <-- add this to KEY_MAP\r\n", cmd);

}

wait_ms(200);

}

}

Path as a block code

#include "mbed.h"

// === Hardware ===

#define IR_PIN      PA_10

DigitalIn  ir_pin(IR_PIN, PullUp);

DigitalOut led(LED1);

Serial     pc(USBTX, USBRX, 115200);

Timer      timer;

// === Path Configuration ===

#define MAX_PATH_LENGTH 16

int path[MAX_PATH_LENGTH];

int path_len = 0;

bool path_ready = false;

// === IR NEC Timing ===

#define NEC_LEADER_MIN  8000

#define NEC_LEADER_MAX  10000

#define NEC_ONE_MIN     1500

// === Button Map: NEC command byte → block number or action ===

// ⚠️ UPDATE THESE WITH YOUR REMOTE'S ACTUAL CODES (see instructions below)

typedef struct {

uint8_t code;   // NEC command byte (lower 8 bits of 32-bit code)

char    label;  // '0'-'9' for blocks, or '*', '#', 'O' for OK

} KeyMap;

const KeyMap KEY_MAP[] = {

{0x00, '1'}, {0x01, '2'}, {0x02, '3'},

{0x04, '4'}, {0x05, '5'}, {0x06, '6'},

{0x08, '7'}, {0x09, '8'}, {0x0A, '9'},

{0x0D, '0'}, {0x0C, '*'}, {0x0E, '#'},

{0x15, 'O'}, // 'O' = OK/Execute

{0xFF,  0}   // Sentinel

};

// === IR Receive (Blocking, NEC Protocol) ===

int wait_for_level(int level, int timeout_us) {

timer.reset();

timer.start();

while (ir_pin.read() != level) {

if (timer.read_us() > timeout_us) { timer.stop(); return -1; }

}

int t = timer.read_us();

timer.stop();

return t;

}

uint32_t receive_nec_frame() {

// Wait for start pulse (active low)

if (wait_for_level(0, 100000) < 0) return 0;

// Leader pulse: 9ms low

int leader_low = wait_for_level(1, 15000);

if (leader_low < NEC_LEADER_MIN || leader_low > NEC_LEADER_MAX) return 0;

// Leader space: 4.5ms high

int leader_high = wait_for_level(0, 6000);

if (leader_high < 3000 || leader_high > 6000) return 0;

// Decode 32 bits

uint32_t data = 0;

for (int i = 0; i < 32; i++) {

if (wait_for_level(1, 1000) < 0) return 0;

int space = wait_for_level(0, 2500);

if (space < 0) return 0;

if (space > NEC_ONE_MIN) data |= (1u << i);

}

wait_for_level(1, 1000); // Final pulse

return data;

}

// === Decode NEC frame to button ===

char decode_button(uint32_t frame) {

uint8_t cmd     = (frame >> 16) & 0xFF;

uint8_t cmd_inv = (frame >> 24) & 0xFF;

// Validate NEC checksum

if ((uint8_t)(cmd ^ cmd_inv) != 0xFF) return 0;

// Look up in map

for (int i = 0; KEY_MAP[i].code != 0xFF; i++) {

if (KEY_MAP[i].code == cmd) return KEY_MAP[i].label;

}

return 0; // Unknown

}

// === Path Management ===

void add_to_path(char block) {

if (path_len < MAX_PATH_LENGTH && block >= '0' && block <= '9') {

path[path_len++] = block - '0';

pc.printf("Added: %c | Path: ", block);

for (int i = 0; i < path_len; i++) pc.printf("%d ", path[i]);

pc.printf("\n");

led = 1; wait_ms(100); led = 0; // Quick blink

}

}

void clear_path() {

path_len = 0;

path_ready = false;

pc.printf("Path cleared\n");

// Double blink

for(int i=0; i<2; i++) { led=1; wait_ms(100); led=0; wait_ms(100); }

}

void confirm_path() {

if (path_len > 0) {

path_ready = true;

pc.printf("✓ Path confirmed: ");

for (int i = 0; i < path_len; i++) pc.printf("%d ", path[i]);

pc.printf("\nPress OK to execute\n");

// Long blink

led = 1; wait_ms(300); led = 0;

} else {

pc.printf("Path empty! Add blocks first\n");

}

}

void execute_path() {

if (!path_ready || path_len == 0) {

pc.printf("No path to execute\n");

return;

}

pc.printf("\n=== EXECUTING PATH ===\n");

pc.printf("Blocks: ");

for (int i = 0; i < path_len; i++) pc.printf("%d ", path[i]);

pc.printf("\n");

// === ROBOT CONTROL SECTION ===

// Replace this loop with your actual robot movement code

for (int i = 0; i < path_len; i++) {

int block = path[i];

pc.printf("→ Moving to block %d\n", block);

// TODO: Insert your robot navigation code here

// Example:

//   robot.go_to_block(block);

//   while (!robot.at_target()) { /* wait */ }

led = 1; wait_ms(200); led = 0; // Visual feedback per block

wait_ms(500); // Simulate movement time

}

pc.printf("=== PATH COMPLETE ===\n\n");

path_ready = false; // Reset for next path

}

// === Main Loop ===

int main() {

pc.printf("=== IR Path Input System ===\n");

pc.printf("Controls:\n");

pc.printf("  0-9 : Add block to path\n");

pc.printf("  *   : Clear path\n");

pc.printf("  #   : Confirm path\n");

pc.printf("  OK  : Execute path\n\n");

// Startup blink

for(int i=0; i<3; i++) { led=!led; wait_ms(200); }

led = 0;

while (true) {

uint32_t frame = receive_nec_frame();

if (frame == 0) continue;

char btn = decode_button(frame);

if (btn == 0) continue; // Unknown button

// Handle commands

if (btn >= '0' && btn <= '9') {

add_to_path(btn);

} else if (btn == '*') {

clear_path();

} else if (btn == '#') {

confirm_path();

} else if (btn == 'O') { // OK button

execute_path();

}

wait_ms(200); // Debounce

}

}

Path go up and down

#include "mbed.h"

// === Hardware Declarations ===

#define IR_PIN      PA_10

DigitalIn  ir_pin(IR_PIN, PullUp);

DigitalOut led(LED1);

Serial     pc(USBTX, USBRX);

Timer      timer;

// === Path Configuration ===

#define MAX_PATH_LENGTH 16

int path[MAX_PATH_LENGTH];

int path_len = 0;

bool path_ready = false;

// === IR NEC Timing ===

#define NEC_LEADER_MIN  8000

#define NEC_LEADER_MAX  10000

#define NEC_ONE_MIN     1500

// === Block Definitions (12 blocks + gate) ===

struct Block {

int id;

float x, y;      // Position (mm)

float height;    // Height in cm

};

const Block BLOCKS[13] = {

{0, 0, 0, 0},       // Index 0 unused

{1,   0, 100, 40},  // Row 1: 40cm

{2, 100, 100, 20},  // Row 1: 20cm (GATE)

{3, 200, 100, 40},  // Row 1: 40cm

{4,   0, 200, 20},  // Row 2: 20cm

{5, 100, 200, 40},  // Row 2: 40cm

{6, 200, 200, 60},  // Row 2: 60cm

{7,   0, 300, 40},  // Row 3: 40cm

{8, 100, 300, 60},  // Row 3: 60cm

{9, 200, 300, 40},  // Row 3: 40cm

{10,  0, 400, 20},  // Row 4: 20cm

{11,100, 400, 40},  // Row 4: 40cm

{12,200, 400, 20}   // Row 4: 20cm

};

// === Robot State ===

struct Robot {

int current_block = 2;      // Start at gate (block 2)

float current_height = 20;  // Block 2 height

};

Robot robot;

// === Button Map: NEC command byte → action ===

typedef struct {

uint8_t code;

char label;

} KeyMap;

const KeyMap KEY_MAP[] = {

{0x00, '1'}, {0x01, '2'}, {0x02, '3'},

{0x04, '4'}, {0x05, '5'}, {0x06, '6'},

{0x08, '7'}, {0x09, '8'}, {0x0A, '9'},

{0x0D, '0'}, {0x0C, '*'}, {0x0E, '#'},

{0x15, 'O'}, // OK/Execute

{0xFF, 0}    // Sentinel

};

// === IR Receive Functions ===

int wait_for_level(int level, int timeout_us) {

timer.reset();

timer.start();

while (ir_pin.read() != level) {

if (timer.read_us() > timeout_us) { timer.stop(); return -1; }

}

int t = timer.read_us();

timer.stop();

return t;

}

uint32_t receive_nec_frame() {

if (wait_for_level(0, 100000) < 0) return 0;

int leader_low = wait_for_level(1, 15000);

if (leader_low < NEC_LEADER_MIN || leader_low > NEC_LEADER_MAX) return 0;

int leader_high = wait_for_level(0, 6000);

if (leader_high < 3000 || leader_high > 6000) return 0;

uint32_t data = 0;

for (int i = 0; i < 32; i++) {

if (wait_for_level(1, 1000) < 0) return 0;

int space = wait_for_level(0, 2500);

if (space < 0) return 0;

if (space > NEC_ONE_MIN) data |= (1u << i);

}

wait_for_level(1, 1000);

return data;

}

char decode_button(uint32_t frame) {

uint8_t cmd = (frame >> 16) & 0xFF;

uint8_t cmd_inv = (frame >> 24) & 0xFF;

if ((uint8_t)(cmd ^ cmd_inv) != 0xFF) return 0;

for (int i = 0; KEY_MAP[i].code != 0xFF; i++) {

if (KEY_MAP[i].code == cmd) return KEY_MAP[i].label;

}

return 0;

}

// === Height Transition Check ===

bool can_transition(int from_id, int to_id) {

if (from_id < 1 || from_id > 12 || to_id < 1 || to_id > 12) return false;

float h1 = BLOCKS[from_id].height;

float h2 = BLOCKS[to_id].height;

float diff = h2 - h1;

// Robot capabilities: can climb up 30cm, down 40cm max per move

if (diff > 0) return diff <= 30;   // Up

else return (-diff) <= 40;          // Down

}

// === Movement Primitives ===

void climb_to_height(float target_h) {

float diff = target_h - robot.current_height;

if (diff > 0) {

pc.printf("  ↑ Climbing UP %.1f cm\n", diff);

// TODO: Activate lift/ramp mechanism UP

} else if (diff < 0) {

pc.printf("  ↓ Climbing DOWN %.1f cm\n", -diff);

// TODO: Activate lift/ramp mechanism DOWN

}

robot.current_height = target_h;

Thread::wait(200 + (int)(abs(diff) * 5)); // Simulate climb time

}

void move_horizontal(float x, float y) {

pc.printf("  → Moving to (%.0f, %.0f) mm\n", x, y);

// TODO: Your 2D navigation code (motors, encoders, etc.)

Thread::wait(500); // Simulate horizontal movement

}

void go_to_block(int block_num) {

if (block_num < 1 || block_num > 12) return;

const Block& target = BLOCKS[block_num];

// Step 1: Adjust height FIRST (critical for safety)

if (target.height != robot.current_height) {

climb_to_height(target.height);

}

// Step 2: Move horizontally on same height plane

move_horizontal(target.x, target.y);

// Step 3: Update robot state

robot.current_block = block_num;

pc.printf("  ✓ Arrived Block %d (h=%.1f cm)\n", block_num, target.height);

}

// === Path Management ===

void add_to_path(char block_char) {

if (path_len >= MAX_PATH_LENGTH) {

pc.printf("Path full!\n");

return;

}

int block = block_char - '0';

if (block < 1 || block > 12) {

pc.printf("Invalid block: %c\n", block_char);

return;

}

path[path_len++] = block;

pc.printf("Added %d | Path: ", block);

for (int i = 0; i < path_len; i++) pc.printf("%d ", path[i]);

pc.printf("\n");

led = 1; Thread::wait(100); led = 0;

}

void clear_path() {

path_len = 0;

path_ready = false;

pc.printf("Path cleared\n");

for(int i=0; i<2; i++) { led=1; Thread::wait(100); led=0; Thread::wait(100); }

}

void confirm_path() {

if (path_len == 0) {

pc.printf("Path empty!\n");

return;

}

// Validate entire path

int current = robot.current_block;

bool valid = true;

for (int i = 0; i < path_len; i++) {

if (!can_transition(current, path[i])) {

pc.printf("ERROR: Cannot go %d→%d (height)\n", current, path[i]);

valid = false;

break;

}

current = path[i];

}

if (valid) {

path_ready = true;

pc.printf("✓ Path confirmed: ");

for (int i = 0; i < path_len; i++) pc.printf("%d ", path[i]);

pc.printf("\nPress OK to execute\n");

led = 1; Thread::wait(300); led = 0;

}

}

void execute_path() {

if (!path_ready || path_len == 0) {

pc.printf("No valid path to execute\n");

return;

}

pc.printf("\n=== EXECUTING PATH ===\n");

pc.printf("Start: Block %d (h=%.1f cm)\n", robot.current_block, robot.current_height);

for (int i = 0; i < path_len; i++) {

int target = path[i];

pc.printf("\nStep %d: %d → %d\n", i+1, robot.current_block, target);

go_to_block(target);

led = !led; // Visual feedback per block

}

pc.printf("\n=== PATH COMPLETE ===\n\n");

path_ready = false;

}

// === Main ===

int main() {

pc.baud(115200);

pc.printf("=== 3D Grid IR Navigation ===\n");

pc.printf("Heights: 2,4,10,12=20cm | 1,3,5,7,9,11=40cm | 6,8=60cm\n");

pc.printf("Start: Gate (Block 2, 20cm)\n\n");

pc.printf("Controls: 0-9=add block | *=clear |#=confirm |OK=execute\n\n");

// Startup test blink

for(int i=0; i<3; i++) { led=!led; Thread::wait(200); }

led = 0;

while (true) {

uint32_t frame = receive_nec_frame();

if (frame == 0) continue;

char btn = decode_button(frame);

if (btn == 0) continue;

if (btn >= '0' && btn <= '9') {

add_to_path(btn);

} else if (btn == '*') {

clear_path();

} else if (btn == '#') {

confirm_path();

} else if (btn == 'O') {

execute_path();

}

Thread::wait(200); // Debounce

}

}

Box that change direction

#include "mbed.h"

// === Hardware ===

#define IR_PIN      PA_10

DigitalIn  ir_pin(IR_PIN, PullUp);

DigitalOut led(LED1);

Serial     pc(USBTX, USBRX, 115200);

Timer      timer;

Timer      press_timer;

// === YOUR EXACT KEY_MAP (unchanged) ===

typedef struct {

uint8_t     code;

const char* label;

} KeyMap;

const KeyMap KEY_MAP[] = {

{0x00, "10"}, {0x01, "11"}, {0x02, "12"},

{0x04, "7"},  {0x05, "8"},  {0x06, "9"},

{0x08, "4"},  {0x09, "5"},  {0x0A, "6"},

{0x0D, "2"},  {0x0C, "1"},  {0x0E, "3"},

{0x15, "Ok"}, {0xFF, "0"},  {0x11, "Confirm"},

{0x14, "Last mood"}, {0x16, "Next mood"}, {0x19, "Clear"}

};

const int KEY_COUNT = sizeof(KEY_MAP) / sizeof(KEY_MAP[0]);

// === Block Heights ===

float get_height(int block) {

if (block == 2 || block == 4 || block == 10 || block == 12) return 20.0f;

if (block == 6 || block == 8) return 60.0f;

if (block >= 1 && block <= 12) return 40.0f;

return 0.0f;

}

// === Robot State ===

struct Robot {

int block = 2;           // Start at gate (block 2)

float height = 20.0f;

int direction = 0;       // 0=N, 1=E, 2=S, 3=W

} robot;

// === Path Storage ===

#define MAX_PATH 16

int path_blocks[MAX_PATH];

int path_presses[MAX_PATH];

int path_len = 0;

bool path_confirmed = false;

// === IR NEC Timing ===

#define NEC_LEADER_MIN  8000

#define NEC_LEADER_MAX  10000

#define NEC_ONE_MIN     1500

int wait_for_level(int level, int timeout_us) {

timer.reset(); timer.start();

while (ir_pin.read() != level) {

if (timer.read_us() > timeout_us) { timer.stop(); return -1; }

}

int t = timer.read_us(); timer.stop();

return t;

}

uint32_t receive_nec() {

if (wait_for_level(0, 100000) < 0) return 0;

int low = wait_for_level(1, 15000);

if (low < NEC_LEADER_MIN || low > NEC_LEADER_MAX) return 0;

int high = wait_for_level(0, 6000);

if (high < 3000 || high > 6000) return 0;

uint32_t data = 0;

for (int i = 0; i < 32; i++) {

if (wait_for_level(1, 1000) < 0) return 0;

int space = wait_for_level(0, 2500);

if (space < 0) return 0;

if (space > NEC_ONE_MIN) data |= (1u << i);

}

wait_for_level(1, 1000);

return data;

}

// === Decode + Press Counter (using YOUR KEY_MAP) ===

uint8_t last_cmd = 0xFF;

int press_count = 0;

const char* decode_label(uint32_t frame, int* out_block) {

uint8_t cmd = (frame >> 16) & 0xFF;

uint8_t cmd_inv = (frame >> 24) & 0xFF;

if ((uint8_t)(cmd ^ cmd_inv) != 0xFF) return nullptr;

// Count presses if same button within 500ms

if (cmd == last_cmd && press_timer.read_ms() < 500) {

press_count++;

} else {

press_count = 1;

last_cmd = cmd;

}

press_timer.reset();

// Look up in YOUR KEY_MAP

for (int i = 0; i < KEY_COUNT; i++) {

if (KEY_MAP[i].code == cmd) {

// Convert label string to block number (if numeric)

if (out_block) {

int val = atoi(KEY_MAP[i].label);

*out_block = (val >= 1 && val <= 12) ? val : -1;

}

return KEY_MAP[i].label;

}

}

return nullptr;

}

// === Movement Functions (simple, no RTOS) ===

void climb(float target_h) {

float diff = target_h - robot.height;

pc.printf("  [CLIMB] %.1f→%.1f cm (diff:%.1f)\n", robot.height, target_h, diff);

if (diff > 0) {

pc.printf("  >>> UP %.1f cm\n", diff);

// TODO: Your climb UP mechanism

} else if (diff < 0) {

pc.printf("  >>> DOWN %.1f cm\n", -diff);

// TODO: Your climb DOWN mechanism

}

robot.height = target_h;

wait_ms(200 + (int)(abs(diff) * 5));

}

void turn_right() {

robot.direction = (robot.direction + 1) % 4;

pc.printf("  >>> TURN RIGHT (facing:%d)\n", robot.direction);

// TODO: Your turn right code

wait_ms(400);

}

void turn_left() {

robot.direction = (robot.direction + 3) % 4;

pc.printf("  >>> TURN LEFT (facing:%d)\n", robot.direction);

// TODO: Your turn left code

wait_ms(400);

}

void move_to(int block) {

pc.printf("  [MOVE] To Block %d\n", block);

// TODO: Your horizontal movement code

wait_ms(600);

robot.block = block;

}

void execute_step(int block, int presses) {

float target_h = get_height(block);

pc.printf("\n=== STEP ===\n");

pc.printf("Block:%d Presses:%d Height:%.1f cm\n", block, presses, target_h);

climb(target_h);

if (presses == 1) {

pc.printf("  Action: CLIMB ONLY\n");

} else if (presses == 2) {

pc.printf("  Action: CLIMB + TURN RIGHT\n");

move_to(block);

turn_right();

} else if (presses >= 3) {

pc.printf("  Action: CLIMB + TURN LEFT\n");

move_to(block);

turn_left();

}

pc.printf("✓ Done Block %d\n", block);

}

// === Path Management ===

void add_to_path(int block, int presses) {

if (path_len >= MAX_PATH) {

pc.printf("[WARN] Path full!\n");

return;

}

path_blocks[path_len] = block;

path_presses[path_len] = presses;

path_len++;

pc.printf("[PATH] Added:%d(%dx) | Full:", block, presses);

for (int i = 0; i < path_len; i++) {

pc.printf(" %d(%dx)", path_blocks[i], path_presses[i]);

}

pc.printf("\n");

// LED blinks = press count

for (int i = 0; i < presses; i++) {

led = 1; wait_ms(80); led = 0; wait_ms(80);

}

}

void clear_path() {

path_len = 0;

path_confirmed = false;

pc.printf("[PATH] CLEARED\n");

for (int i = 0; i < 3; i++) { led = !led; wait_ms(150); }

led = 0;

}

void confirm_path() {

if (path_len == 0) {

pc.printf("[WARN] Path empty!\n");

return;

}

path_confirmed = true;

pc.printf("\n✓ PATH CONFIRMED (%d steps)\n", path_len);

pc.printf("Press 'Ok' to execute, 'Clear' to reset\n\n");

led = 1; wait_ms(400); led = 0;

}

void run_path() {

if (!path_confirmed || path_len == 0) {

pc.printf("[WARN] No confirmed path!\n");

return;

}

pc.printf("\n================================\n");

pc.printf("   EXECUTING PATH\n");

pc.printf("================================\n");

pc.printf("Start: Block %d, H=%.1f cm, Dir=%d\n\n",

robot.block, robot.height, robot.direction);

for (int i = 0; i < path_len; i++) {

execute_step(path_blocks[i], path_presses[i]);

led = !led;

}

pc.printf("\n================================\n");

pc.printf("   PATH COMPLETE\n");

pc.printf("================================\n\n");

path_confirmed = false;

path_len = 0;

}

// === String Compare Helper ===

bool str_eq(const char* a, const char* b) {

while (*a && *b) {

if (*a != *b) return false;

a++; b++;

}

return *a == *b;

}

// === Main ===

int main() {

pc.printf("\n================================\n");

pc.printf("  IR PATH NAVIGATION\n");

pc.printf("================================\n");

pc.printf("Remote Labels → Blocks:\n");

pc.printf("  1→Block1  2→Block2  3→Block3\n");

pc.printf("  4→Block4  5→Block5  6→Block6\n");

pc.printf("  7→Block7  8→Block8  9→Block9\n");

pc.printf("  10→Block10  11→Block11  12→Block12\n");

pc.printf("\nPress Count:\n");

pc.printf("  1x = Climb to height\n");

pc.printf("  2x = Climb + Turn Right\n");

pc.printf("  3x = Climb + Turn Left\n");

pc.printf("\nCommands:\n");

pc.printf("  Clear   = Clear path\n");

pc.printf("  Confirm = Lock path\n");

pc.printf("  Ok      = Execute path\n");

pc.printf("================================\n\n");

press_timer.start();

// Startup blink

for (int i = 0; i < 3; i++) { led = !led; wait_ms(200); }

led = 0;

while (true) {

uint32_t frame = receive_nec();

if (frame == 0) continue;

// === PRINT RAW DATA (DEBUG) ===

uint8_t addr    = frame & 0xFF;

uint8_t addr_inv= (frame >> 8) & 0xFF;

uint8_t cmd     = (frame >> 16) & 0xFF;

uint8_t cmd_inv = (frame >> 24) & 0xFF;

pc.printf("[RAW] 0x%08lX | A:0x%02X/~0x%02X C:0x%02X/~0x%02X\n",

frame, addr, addr_inv, cmd, cmd_inv);

// Decode using YOUR KEY_MAP

int block = -1;

const char* label = decode_label(frame, &block);

if (!label) {

pc.printf("[ERR] Unknown button (cmd:0x%02X)\n", cmd);

continue;

}

pc.printf("[DECODED] Label:\"%s\" Block:%d Presses:%d\n",

label, block, press_count);

// === Handle Commands ===

if (str_eq(label, "Ok")) {

pc.printf("[CMD] Ok → Execute path\n");

run_path();

wait_ms(300);

continue;

}

if (str_eq(label, "Clear")) {

pc.printf("[CMD] Clear → Reset path\n");

clear_path();

wait_ms(300);

continue;

}

if (str_eq(label, "Confirm")) {

pc.printf("[CMD] Confirm → Lock path\n");

confirm_path();

wait_ms(300);

continue;

}

// === Add numeric blocks to path ===

if (block >= 1 && block <= 12) {

pc.printf("[INPUT] Block %d | Press #%d\n", block, press_count);

add_to_path(block, press_count);

} else {

pc.printf("[INFO] Label \"%s\" not a block (1-12)\n", label);

}

wait_ms(250);  // Debounce

}

}