#include "mbed.h"

#include <cstdlib>

#include <cmath>

#include "stm32f4xx_hal.h"

// ================= MOTOR & CAN CONFIG =================

#define MOTOR_ID_1      11

#define MOTOR_ID_2      12

#define MOTOR_ID_3      13

#define MOTOR_ID_4      14

#define CAN_BAUD        1000000

#define REPLY_ID_1      (MOTOR_ID_1 + 0x100)

#define REPLY_ID_2      (MOTOR_ID_2 + 0x100)

#define REPLY_ID_3      (MOTOR_ID_3 + 0x100)

#define REPLY_ID_4      (MOTOR_ID_4 + 0x100)

DigitalOut led(LED1);

Serial pc(USBTX, USBRX, 115200);

CAN can(PA_11, PA_12, CAN_BAUD);

// LCD Config

#define I2C_SDA PB_9

#define I2C_SCL PB_8

I2C i2c_lcd(I2C_SDA, I2C_SCL);

uint8_t lcd_addr = 0x27;

#define LCD_BACKLIGHT 0x08

#define LCD_EN 0x04

#define LCD_RS 0x01

#define LCD_ADDR_1 0x27

#define LCD_ADDR_2 0x3F

PwmOut     m1_pwm(PB_14);

DigitalOut m1_dira(PC_8);

DigitalOut m1_dirb(PC_6);

PwmOut     m2_pwm(PB_13);

DigitalOut m2_dira(PB_15);

DigitalOut m2_dirb(PB_1);

// M3

DigitalOut m3_ina(PA_8);

DigitalOut m3_inb(PB_10);

PwmOut     m3_pwm(PA_9);

// M4

DigitalOut m4_ina(PA_13);

DigitalOut m4_inb(PA_14);

PwmOut     m4_pwm(PB_2);

// --- CONFIGURATION ---

const float MAX_VEL = 3.0f;

const float LINEAR_SPEED = MAX_VEL;

const float DISTANCE_1 = 7.0f;

const float DISTANCE_2 = 2.0f;

const float DISTANCE_3 = 11.0f;

const float TIME_1 = DISTANCE_1 / LINEAR_SPEED;

const float TIME_2 = DISTANCE_2 / LINEAR_SPEED;

const float TIME_3 = DISTANCE_3 / LINEAR_SPEED;

const float TIME_4 = 4.0;

// Ladder Configuration

const float LADDER_SPEED = 0.5f;

const float LADDER_TIME_20CM = 4.0f;   // Time to move 20cm

const float STABILIZE_TIME = 4.0f;     // Time to move forward

const float STABILIZE_VEL = 5.0f;

const float STABILIZE_BACK = 0.1f;

// Directions for M3/M4

const int DIR_UP_M3    = -1;

const int DIR_UP_M4    = 1;

const int DIR_DOWN_M3  = 1;

const int DIR_DOWN_M4  = -1;

const int NUM_MOTORS = 5;

const int NUM_PATTERNS = 7;

// ================= STATE ENUM =================

enum State {

DISABLED = 0,

MOVE_BACK_1 = 1,

CYL1_CYCLE = 2,

MOVE_BACK_2 = 3,

CYL2_CYCLE = 4,

MOVE_FOR_1 = 5,

MOVE_DC_TO_RELEASE = 6,

TURN_TO_LADDER = 7,

FIRST_LADDER = 8,

FINISHED = 9

};

volatile State current_state = DISABLED;

volatile int rx_count[4] = {0, 0, 0, 0};

CANMessage tx_msg, rx_msg;

float velocities[4] = {0.0f, 0.0f, 0.0f, 0.0f};

const float directions[NUM_PATTERNS][NUM_MOTORS] = {

{1.0f, -1.0f, -1.0f, 1.0f},   // 0:

{1.0f, 1.0f, -1.0f, -1.0f},   // 1: hoish

{-1.0f, 1.0f, 1.0f, -1.0f},   // 2:

{-1.0f, -1.0f, 1.0f, 1.0f},   // 3: Uragsh

{0.0f, 1.0f, 0.0f, -1.0f},    // 4: Diagonal

{-1.0f, -1.0f, -1.0f, -1.0f},  // 5: Turn Arc

{1.0f, 0.0f, -1.0f, 0.0f},    // 6: Diagonal

};

// ================= SENSORS =================

DigitalIn sens_start(PC_11, PullUp);

DigitalIn sens_motor_start(PA_5, PullUp);

DigitalIn sens_field_select(PC_3, PullUp);

DigitalIn sens_motor_stop(PB_0, PullUp);

DigitalIn box_sensor_1(PA_4, PullUp);

DigitalIn box_sensor_2(PC_2, PullUp);

DigitalIn box_sensor_3(PC_5, PullUp);

// Cylinders

DigitalOut cyl1(PC_7);

DigitalOut cyl2(PB_6);

DigitalOut cyl3(PA_7);

DigitalOut cyl4(PA_6);

DigitalOut* relays[4] = {&cyl1, &cyl2, &cyl3, &cyl4};

const bool ACTIVE_LOW = false;

#define RELAY_ON  1

#define RELAY_OFF 0

// ================= FUNCTION PROTOTYPES =================

void send_cmd(uint32_t id, float vel, float kp = 0, float kd = 1.0f);

void enter_mode(uint32_t id);

void safe_stop_all();

void on_rx();

void cyl_stop_all();

int read_active_low(DigitalIn& s);

int is_pressed(DigitalIn& s);

void set_motor(int dir, float duty);

void stopAllClimb();

// Movement Functions

void moveM3M4Up();

void moveM3Down();

void moveM4Down();

void moveCANForward();

void moveCANBackward();

// LCD Helpers

void wait_us_lcd(int us);

void write_i2c(uint8_t data);

void pulse_enable(uint8_t data);

void send_nibble(uint8_t nibble, uint8_t mode);

void send_command(uint8_t cmd);

void send_char(char c);

void lcd_print(const char* str);

void lcd_print_num(float num, int decimals);

void lcd_clear();

void lcd_set_cursor(uint8_t row, uint8_t col);

bool detect_lcd();

void lcd_init();

// ================= MAIN =================

int main() {

__HAL_RCC_PWR_CLK_ENABLE();

HAL_PWR_EnableBkUpAccess();

__HAL_RCC_LSE_CONFIG(RCC_LSE_OFF);

while(__HAL_RCC_GET_FLAG(RCC_FLAG_LSERDY) != RESET);

// Init Relays Off

for (int i = 0; i < 4; i++) {

relays[i]->write(ACTIVE_LOW ? 1 : 0);

}

// Init Climbing Motors

m3_pwm.period_ms(2); m3_pwm = 0.0f;

m4_pwm.period_ms(2); m4_pwm = 0.0f;

stopAllClimb();

// Init Grip Motor

m1_pwm.period(0.0001f);

m1_pwm = 0.0f;

m1_dira = 0; m1_dirb = 0;

wait_ms(500);

pc.printf("\r\n=== System Boot ===\r\n");

pc.printf("Dir M3 Up: %d, Dir M4 Up: %d\r\n", DIR_UP_M3, DIR_UP_M4);

if (!detect_lcd()) {

led = 1;

while(true) { led = !led; thread_sleep_for(200); }

}

lcd_init();

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("ABU Robocon 2026");

lcd_set_cursor(1, 0); lcd_print("Init CAN...");

int freq = can.frequency(CAN_BAUD);

if(freq == 0) {

lcd_set_cursor(1, 0); lcd_print("CAN FAILED!   ");

while(1) { led = !led; thread_sleep_for(100); }

}

can.attach(&on_rx, CAN::RxIrq);

can.filter(0x100, 0x700, CANStandard, 0);

enter_mode(MOTOR_ID_1); enter_mode(MOTOR_ID_2);

enter_mode(MOTOR_ID_3); enter_mode(MOTOR_ID_4);

safe_stop_all();

lcd_set_cursor(1, 0); lcd_print("Ready. Wait St");

Timer state_timer, debug_timer;

State last_state = DISABLED;

int ladder_sub_step = 0;

while(1) {

int st = read_active_low(sens_start);

int field_raw = sens_field_select.read();

const char* field_str = (field_raw == 0) ? "RED" : "BLU";

if (current_state != last_state) {

state_timer.reset();

state_timer.start();

debug_timer.reset();

last_state = current_state;

pc.printf("State: %d\r\n", current_state);

if (current_state == FIRST_LADDER) {

ladder_sub_step = 0;

pc.printf("Starting Custom Ladder Sequence...\r\n");

}

if(current_state != MOVE_BACK_1 && current_state != MOVE_BACK_2 && current_state != MOVE_FOR_1 && current_state != TURN_TO_LADDER && current_state != FIRST_LADDER) {

safe_stop_all();

set_motor(0, 0);

stopAllClimb();

}

}

// ================= STATE MACHINE =================

if (current_state == DISABLED) {

safe_stop_all(); cyl_stop_all(); set_motor(0, 0); stopAllClimb();

if (read_active_low(sens_start) == 0) {

thread_sleep_for(50);

if(read_active_low(sens_start) == 0) {

current_state = MOVE_BACK_1;

lcd_clear(); lcd_set_cursor(0, 0); lcd_print("1. MOVE BACK 1");

}

} else {

lcd_set_cursor(0, 0); lcd_print("St:"); lcd_print(st ? "1" : "0");

lcd_set_cursor(1, 0); lcd_print("Fld:"); lcd_print(field_str); lcd_print(" READY");

static Timer blink;

if(blink.read() > 0.5f) { blink.reset(); led = !led; }

}

}

else if (current_state == MOVE_BACK_1) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_1) { safe_stop_all(); wait_ms(300); current_state = CYL2_CYCLE; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (current_state == CYL2_CYCLE) {

cyl2.write(RELAY_ON); wait_ms(2000); cyl1.write(RELAY_ON); wait_ms(300);

current_state = MOVE_FOR_1; thread_sleep_for(50);

}

else if (current_state == MOVE_FOR_1) {

float t = state_timer.read();

static int turn_phase = 0;

if (turn_phase == 0) {

if (t < TIME_1*2) { for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[4][i]; // Diagonal

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);  send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);  send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();  lcd_set_cursor(1, 0); lcd_print("Turn Phase 1");

}

led = 1;

} else {  turn_phase = 1; state_timer.reset();

pc.printf("Switching to Turn Phase 2...\r\n");

}

}

else if (turn_phase == 1) {

if (t < TIME_1) {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * -1 * directions[5][i]; // Arc

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);  send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);  send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();

lcd_set_cursor(1, 0); lcd_print("Turn Phase 2");

}

led = 1;

} else {

safe_stop_all(); wait_ms(300); lcd_clear();

lcd_set_cursor(0, 0); lcd_print("7. GRIP RELEASE");

current_state = MOVE_DC_TO_RELEASE;

turn_phase = 0;

cyl2.write(RELAY_ON); wait_ms(2000);

}

}

thread_sleep_for(20);

}

else if (current_state == MOVE_DC_TO_RELEASE) {

int release_hit = is_pressed(sens_motor_stop);

set_motor(1, 0.3f);

lcd_set_cursor(0, 0); lcd_print("DC: Moving");

if (release_hit) {

m1_pwm = 0.0f; m1_dira = 1; m1_dirb = 1; wait_ms(50); m1_dira = 0; m1_dirb = 0; wait_ms(100);

current_state = TURN_TO_LADDER; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("RELEASE HIT");

}

if (state_timer.read() > 3.0f) { set_motor(0, 0); current_state = TURN_TO_LADDER; wait_ms(10000);}

}

else if (current_state == TURN_TO_LADDER) {

float t = state_timer.read();

static int turn_phase = 0;

if (turn_phase == 0) {

if (t < TIME_4) { for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * -1 * directions[5][i]; // Diagonal

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);  send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);  send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();  lcd_set_cursor(1, 0); lcd_print("Turn Phase 1");

}

led = 1;

} else {  turn_phase = 1; state_timer.reset();

pc.printf("Switching to Turn Phase 2...\r\n");

}

}

else if (turn_phase == 1) {

if (t < TIME_2*2) { for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[1][i]; // Uragsh

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);  send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);  send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();  lcd_set_cursor(1, 0); lcd_print("Turn Phase 1");

}

led = 1;

} else {  turn_phase = 2; state_timer.reset();

pc.printf("Switching to Turn Phase 2...\r\n");

}

}

else if (turn_phase == 2) {

if (t < TIME_3) {   for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * -1 * directions[6][i]; // Arc

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);  send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);  send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();

lcd_set_cursor(1, 0); lcd_print("Turn Phase 2");

}

led = 1;

} else {

safe_stop_all(); wait_ms(300); lcd_clear();

lcd_set_cursor(0, 0); lcd_print("7. GRIP RELEASE");

current_state = FIRST_LADDER;

turn_phase = 0;

}

}

thread_sleep_for(20);

}

// ================= CUSTOM LADDER LOGIC =================

else if (current_state == FIRST_LADDER) {

float t = state_timer.read();

if(debug_timer.read() > 1.0f) {

debug_timer.reset();

pc.printf("Step: %d, Time: %.2f\r\n", ladder_sub_step, t);

lcd_set_cursor(1, 0); lcd_print("Step: "); lcd_print_num(ladder_sub_step, 0);

}

// Step 0: M3 UP + M4 UP (Simultaneous)

if (ladder_sub_step == 0) {

stopAllClimb(); safe_stop_all();

if (t < LADDER_TIME_20CM) {

moveM3M4Up();

if(debug_timer.read() > 0.5f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("M3+M4 UP"); }

thread_sleep_for(20);

} else {

stopAllClimb(); safe_stop_all();

pc.printf("Step 0 Done. Stabilizing...\r\n");

wait_ms(100);

ladder_sub_step = 1;

state_timer.reset();

}

}

// Step 1: CAN Forward

else if (ladder_sub_step == 1) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); ladder_sub_step = 2; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

// Step 2: M3 DOWN

else if (ladder_sub_step == 2) {

stopAllClimb(); safe_stop_all();

if (t < LADDER_TIME_20CM) {

moveM3Down();

if(debug_timer.read() > 0.6f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("M3 DOWN"); }

thread_sleep_for(20);

} else {

stopAllClimb(); safe_stop_all();

pc.printf("Step 2 Done. Stabilizing...\r\n");

wait_ms(100);

ladder_sub_step = 3;

state_timer.reset();

}

}

// Step 3: CAN Forward

else if (ladder_sub_step == 3) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); ladder_sub_step = 4; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (ladder_sub_step == 4) {

stopAllClimb(); safe_stop_all();

float t = state_timer.read();

if (t >= TIME_4) { safe_stop_all(); wait_ms(300); ladder_sub_step = 6; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("2. CYL1 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[1][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.1f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

ladder_sub_step = 5;

}

thread_sleep_for(20);

}

// Step 4: M4 DOWN

else if (ladder_sub_step == 5) {

stopAllClimb(); safe_stop_all();

if (t < LADDER_TIME_20CM) {

moveM4Down();

if(debug_timer.read() > 0.6f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("M4 DOWN"); }

thread_sleep_for(20);

} else {

stopAllClimb(); safe_stop_all();

pc.printf("Step 4 Done. Sequence Complete!\r\n");

wait_ms(500);

ladder_sub_step = 6;

state_timer.reset();

}

}

// Step 3: CAN Forward

else if (ladder_sub_step == 6) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); ladder_sub_step = 7; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (ladder_sub_step == 7) {

stopAllClimb(); safe_stop_all();

if (t < TIME_4) {

moveCANBackward();

if(debug_timer.read() > 0.1f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("CAN FWD 1"); }

thread_sleep_for(20);

} else {

safe_stop_all();

pc.printf("Step 1 Done. M3 Down...\r\n");

wait_ms(100);

ladder_sub_step = 8;

state_timer.reset();

}

}

if (ladder_sub_step == 8) {

stopAllClimb(); safe_stop_all();

if (t < LADDER_TIME_20CM) {

moveM3M4Up();

if(debug_timer.read() > 0.5f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("M3+M4 UP"); }

thread_sleep_for(20);

} else {

stopAllClimb(); safe_stop_all();

pc.printf("Step 0 Done. Stabilizing...\r\n");

wait_ms(100);

ladder_sub_step = 9;

state_timer.reset();

}

}

else if (ladder_sub_step == 9) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); ladder_sub_step = 10; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (ladder_sub_step == 10) {

stopAllClimb(); safe_stop_all();

if (t < TIME_4) {

moveCANBackward();

if(debug_timer.read() > 0.1f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("CAN FWD 1"); }

thread_sleep_for(20);

} else {

safe_stop_all();

pc.printf("Step 1 Done. M3 Down...\r\n");

wait_ms(100);

ladder_sub_step = 11;

state_timer.reset();

}

}

// Step 2: M3 DOWN

else if (ladder_sub_step == 11) {

stopAllClimb(); safe_stop_all();

if (t < LADDER_TIME_20CM) {

moveM3Down();

if(debug_timer.read() > 0.6f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("M3 DOWN"); }

thread_sleep_for(20);

} else {

stopAllClimb(); safe_stop_all();

pc.printf("Step 2 Done. Stabilizing...\r\n");

wait_ms(100);

ladder_sub_step = 12;

state_timer.reset();

}

}

else if (ladder_sub_step == 12) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); ladder_sub_step = 13; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (ladder_sub_step == 13) {

stopAllClimb(); safe_stop_all();

if (t < TIME_4) {

moveCANBackward();

if(debug_timer.read() > 0.1f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("CAN FWD 1"); }

thread_sleep_for(20);

} else {

safe_stop_all();

pc.printf("Step 1 Done. M3 Down...\r\n");

wait_ms(100);

ladder_sub_step = 14;

state_timer.reset();

}

}

else if (ladder_sub_step == 14) {

stopAllClimb(); safe_stop_all();

if (t < LADDER_TIME_20CM) {

moveM4Down();

if(debug_timer.read() > 0.6f) { debug_timer.reset(); lcd_set_cursor(0, 1); lcd_print("M4 DOWN"); }

thread_sleep_for(20);

} else {

stopAllClimb(); safe_stop_all();

pc.printf("Step 4 Done. Sequence Complete!\r\n");

wait_ms(500);

ladder_sub_step = 15;

state_timer.reset();

}

}

else if (ladder_sub_step == 15) {

float t = state_timer.read();

cyl1.write(RELAY_OFF); wait_ms(2000);

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); current_state = FINISHED; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

}

// 9. FINISHED

else if (current_state == FINISHED) {

safe_stop_all();

cyl_stop_all();

set_motor(0, 0);

stopAllClimb();

led = 0;

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("SEQUENCE DONE");

lcd_set_cursor(1, 0); lcd_print("STOPPED");

thread_sleep_for(100);

}

thread_sleep_for(20);

}

}

// ================= MOVEMENT FUNCTIONS =================

// 1. BOTH M3 and M4 UP Together

void moveM3M4Up() {

if (DIR_UP_M3 > 0) { m3_ina = 1; m3_inb = 0; }

else { m3_ina = 0; m3_inb = 1; }

m3_pwm = 0.4f;

if (DIR_UP_M4 > 0) { m4_ina = 1; m4_inb = 0; }

else { m4_ina = 0; m4_inb = 1; }

m4_pwm = 0.5f;

}

// 2. M3 DOWN Only

void moveM3Down() {

m4_pwm = 0.0f; m4_ina = 0; m4_inb = 0;

wait_us(50);

if (DIR_DOWN_M3 > 0) { m3_ina = 1; m3_inb = 0; }

else { m3_ina = 0; m3_inb = 1; }

m3_pwm = 0.42f;

}

// 3. M4 DOWN Only

void moveM4Down() {

m3_pwm = 0.0f; m3_ina = 0; m3_inb = 0;

wait_us(50);

if (DIR_DOWN_M4 > 0) { m4_ina = 1; m4_inb = 0; }

else { m4_ina = 0; m4_inb = 1; }

m4_pwm = 0.5f;

}

// 4. CAN Forward

void moveCANForward() {

m3_pwm = 0.0f; m3_ina = 0; m3_inb = 0;

m4_pwm = 0.0f; m4_ina = 0; m4_inb = 0;

for(int i=0; i<NUM_MOTORS; i++) {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);

send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);

send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

}

}

void moveCANBackward() {

m3_pwm = 0.0f; m3_ina = 0; m3_inb = 0;

m4_pwm = 0.0f; m4_ina = 0; m4_inb = 0;

for(int i=0; i<NUM_MOTORS; i++) {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[1][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);

send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);

send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

}

}

void stopAllClimb() {

m3_pwm = 0.0f; m3_ina = 0; m3_inb = 0;

m4_pwm = 0.0f; m4_ina = 0; m4_inb = 0;

wait_ms(50);

}

void cyl_stop_all() {

cyl1.write(RELAY_OFF); cyl2.write(RELAY_OFF); cyl3.write(RELAY_OFF); cyl4.write(RELAY_OFF);

}

void set_motor(int dir, float duty) {

if (duty > 1.0f) duty = 1.0f;

if (duty < 0.0f) duty = 0.0f;

if (dir == 1) { m1_dira = 0; m1_dirb = 1; m1_pwm = duty; }

else if (dir == -1) { m1_dira = 1; m1_dirb = 0; m1_pwm = duty; }

else { m1_dira = 0; m1_dirb = 0; m1_pwm = 0.0f; }

}

int read_active_low(DigitalIn& s) { return (s.read() == 0) ? 1 : 0; }

int is_pressed(DigitalIn& s) { return (s.read() == 0) ? 1 : 0; }

void send_cmd(uint32_t id, float vel, float kp, float kd) {

if(vel < -30.0f) vel = -30.0f; if(vel > 30.0f) vel = 30.0f;

int p_int = 2048;

int v_int = (int)((vel + 30.0f) * 4095.0f / 60.0f);

int kp_int = (int)(kp * 4095.0f / 500.0f);

int kd_int = (int)(kd * 4095.0f / 5.0f);

int ff_int = 2048;

tx_msg.data[0] = (p_int >> 8) & 0xFF; tx_msg.data[1] = p_int & 0xFF;

tx_msg.data[2] = (v_int >> 4) & 0xFF; tx_msg.data[3] = ((v_int & 0x0F) << 4) | ((kp_int >> 8) & 0x0F);

tx_msg.data[4] = kp_int & 0xFF; tx_msg.data[5] = (kd_int >> 4) & 0xFF;

tx_msg.data[6] = ((kd_int & 0x0F) << 4) | ((ff_int >> 8) & 0x0F); tx_msg.data[7] = ff_int & 0xFF;

tx_msg.id = id; tx_msg.type = CANData; tx_msg.format = CANStandard; tx_msg.len = 8;

can.write(tx_msg);

}

void enter_mode(uint32_t id) {

tx_msg.id = id; tx_msg.type = CANData; tx_msg.format = CANStandard; tx_msg.len = 8;

for(int i = 0; i < 7; i++) tx_msg.data[i] = 0xFF; tx_msg.data[7] = 0xFC;

can.write(tx_msg); wait_ms(50);

}

void safe_stop_all() {

for(int k=0; k<3; k++) {

for(int i=0; i<NUM_MOTORS; i++) {

send_cmd(MOTOR_ID_1+i, 0.0f, 0.0f, 10.0f); wait_us(200);

}

wait_ms(10);

}

}

void on_rx() {

if(can.read(rx_msg)) {

if(rx_msg.id == REPLY_ID_1 && rx_msg.len >= 6) rx_count[0]++;

else if(rx_msg.id == REPLY_ID_2 && rx_msg.len >= 6) rx_count[1]++;

else if(rx_msg.id == REPLY_ID_3 && rx_msg.len >= 6) rx_count[2]++;

else if(rx_msg.id == REPLY_ID_4 && rx_msg.len >= 6) rx_count[3]++;

}

}

// ================= LCD FUNCTIONS =================

void wait_us_lcd(int us) { thread_sleep_for(us / 1000); }

void write_i2c(uint8_t data) { char cmd = data | LCD_BACKLIGHT; i2c_lcd.write(lcd_addr << 1, &cmd, 1); }

void pulse_enable(uint8_t data) { write_i2c(data | LCD_EN); wait_us_lcd(100); write_i2c(data & ~LCD_EN); wait_us_lcd(100); }

void send_nibble(uint8_t nibble, uint8_t mode) { uint8_t data = (nibble & 0xF0) | mode | LCD_BACKLIGHT; pulse_enable(data); }

void send_command(uint8_t cmd) { send_nibble(cmd & 0xF0, 0); send_nibble((cmd << 4) & 0xF0, 0); thread_sleep_for(2); }

void send_char(char c) { send_nibble(c & 0xF0, LCD_RS); send_nibble((c << 4) & 0xF0, LCD_RS); }

void lcd_print(const char* str) { while (*str) send_char(*str++); }

void lcd_print_num(float num, int decimals) { char buf[16]; sprintf(buf, "%.*f", decimals, num); lcd_print(buf); }

void lcd_clear() { send_command(0x01); thread_sleep_for(2); }

void lcd_set_cursor(uint8_t row, uint8_t col) { uint8_t addr = (row == 0) ? 0x80 + col : 0xC0 + col; send_command(addr); }

bool detect_lcd() {

char dummy = 0;

if (i2c_lcd.write(LCD_ADDR_1 << 1, &dummy, 0) == 0) { lcd_addr = LCD_ADDR_1; return true; }

if (i2c_lcd.write(LCD_ADDR_2 << 1, &dummy, 0) == 0) { lcd_addr = LCD_ADDR_2; return true; }

return false;

}

void lcd_init() {

thread_sleep_for(50);

write_i2c(0x03 | LCD_BACKLIGHT); wait_us_lcd(4500);

pulse_enable(0x03 | LCD_BACKLIGHT); wait_us_lcd(4500);

pulse_enable(0x03 | LCD_BACKLIGHT); wait_us_lcd(150);

pulse_enable(0x02 | LCD_BACKLIGHT);

send_command(0x28); send_command(0x0C); send_command(0x06); send_command(0x01);

thread_sleep_for(2);

}

// GIM8008 Quad Motor + Cylinders + Gripper Motor - Full Sequence

// NUCLEO_F446RE | MCP2551 | CAN 1Mbps | LCD I2C | Limit Switches

// Includes: Quad CAN Motors, 2 Solenoid Cylinders, 1 VNH5019 Gripper Motor

#include "mbed.h"

#include <cstdlib>

#include <cmath>

#include "stm32f4xx_hal.h"

// ================= MOTOR & CAN CONFIG =================

#define MOTOR_ID_1      11

#define MOTOR_ID_2      12

#define MOTOR_ID_3      13

#define MOTOR_ID_4      14

#define CAN_BAUD        1000000

#define REPLY_ID_1      (MOTOR_ID_1 + 0x100)

#define REPLY_ID_2      (MOTOR_ID_2 + 0x100)

#define REPLY_ID_3      (MOTOR_ID_3 + 0x100)

#define REPLY_ID_4      (MOTOR_ID_4 + 0x100)

DigitalOut led(LED1);

Serial pc(USBTX, USBRX, 115200);

CAN can(PA_11, PA_12, CAN_BAUD);

// ================= GRIPPER MOTOR HARDWARE (VNH5019) =================

// Limits: Active Low (0 = Pressed)

DigitalIn lim_grip(PC_4, PullUp);      // START/HOME Position

DigitalIn lim_release(PC_1, PullUp);   // END/FAR Position

DigitalIn action_btn(PA_10, PullUp);   // Single Control Button

// Motor Driver Pins

PwmOut     pwm(PB_14);

DigitalOut dira(PC_8);

DigitalOut dirb(PC_6);

// Gripper Config

const float MOVE_DUTY = 0.4f;      // Speed for moving between limits

const float SEARCH_DUTY = 0.2f;    // Slow speed for initial finding

// ================= QUAD MOTOR CONFIG =================

const float MAX_VEL = 3.0f;

const float WHEEL_RADIUS = 0.065f;

const float DISTANCE_1 = 0.80f;     // Updated to 0.8m as per LCD text

const float DISTANCE_2 = 0.50f;     // Updated to 0.5m

const float LINEAR_SPEED = MAX_VEL * WHEEL_RADIUS;

const float TIME_1 = DISTANCE_1 / LINEAR_SPEED;

const float TIME_2 = DISTANCE_2 / LINEAR_SPEED;

const int NUM_MOTORS = 4;

const int NUM_PATTERNS = 4;

// Fixed Enum Definition

enum State {

DISABLED = 0,

MOVE_BACK_1 = 1,

CYL1_CYCLE = 2,

MOVE_BACK_2 = 3,

CYL2_CYCLE = 4,

FINISHED = 5

};

volatile State current_state = DISABLED;

volatile int rx_count[4] = {0, 0, 0, 0};

CANMessage tx_msg, rx_msg;

float velocities[4] = {0.0f, 0.0f, 0.0f, 0.0f};

const int directions[NUM_PATTERNS][NUM_MOTORS] = {

{1, -1, -1, 1},   // 0: Uragsh (Fwd)

{1, 1, -1, -1},   // 1: Baruun (Right)

{-1, 1, 1, -1},   // 2: Hoish (Back)

{-1, -1, 1, 1}    // 3: Zvvn (Left)

};

// ================= SENSOR & LCD CONFIG =================

#define I2C_SDA PB_9

#define I2C_SCL PB_8

#define LCD_ADDR_1 0x27

#define LCD_ADDR_2 0x3F

DigitalIn sens_start(PC_0, PullUp);

DigitalIn sens_motor_start(PC_1, PullUp);

DigitalIn sens_field_select(PC_3, PullUp);

DigitalIn sens_motor_stop(PC_4, PullUp);

I2C i2c_lcd(I2C_SDA, I2C_SCL);

uint8_t lcd_addr = 0;

#define LCD_BACKLIGHT 0x08

#define LCD_EN 0x04

#define LCD_RS 0x01

// ================= CYLINDER HARDWARE =================

DigitalOut cyl1_ina(PA_9);

DigitalOut cyl1_inb(PC_7);

PwmOut     cyl1_pwm(PA_8); // Changed to PA_8 to avoid conflict with INA on PA_9 if needed, adjust if your HW is specific

DigitalOut cyl2_ina(PA_13);

DigitalOut cyl2_inb(PA_14);

PwmOut     cyl2_pwm(PB_2);

// ================= FUNCTION PROTOTYPES =================

void send_cmd(uint32_t id, float vel, float kp = 0, float kd = 1.0f);

void enter_mode(uint32_t id);

void safe_stop_all();

void on_rx();

void cyl_stop_all();

int read_active_low(DigitalIn& s);

void set_gripper_motor(int dir, float duty); // New function

// LCD Functions

void wait_us_lcd(int us);

void write_i2c(uint8_t data);

void pulse_enable(uint8_t data);

void send_nibble(uint8_t nibble, uint8_t mode);

void send_command(uint8_t cmd);

void send_char(char c);

void lcd_print(const char* str);

void lcd_print_num(float num, int decimals);

void lcd_clear();

void lcd_set_cursor(uint8_t row, uint8_t col);

bool detect_lcd();

void lcd_init();

// ================= MAIN =================

int main() {

// 1. Critical: Disable LSE for PB_8/PB_9 I2C

__HAL_RCC_PWR_CLK_ENABLE();

HAL_PWR_EnableBkUpAccess();

__HAL_RCC_LSE_CONFIG(RCC_LSE_OFF);

while(__HAL_RCC_GET_FLAG(RCC_FLAG_LSERDY) != RESET);

// 2. Init Cylinders Safe State

cyl1_ina = 0; cyl1_inb = 0; cyl1_pwm = 0.0f;

cyl2_ina = 0; cyl2_inb = 0; cyl2_pwm = 0.0f;

// 3. Init Gripper Motor Safe State

dira = 0; dirb = 0; pwm = 0.0f;

pc.printf("\r\n=== System Boot ===\r\n");

// 4. Init LCD

if (!detect_lcd()) {

led = 1;

while(true) { led = !led; thread_sleep_for(200); }

}

lcd_init();

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("ABU Robocon 2026");

lcd_set_cursor(1, 0); lcd_print("Init CAN...");

// 5. Initialize CAN

int freq = can.frequency(CAN_BAUD);

if(freq == 0) {

lcd_set_cursor(1, 0); lcd_print("CAN FAILED!   ");

while(1) { led = !led; thread_sleep_for(100); }

}

can.attach(&on_rx, CAN::RxIrq);

can.filter(0x100, 0x700, CANStandard, 0);

enter_mode(MOTOR_ID_1); enter_mode(MOTOR_ID_2);

enter_mode(MOTOR_ID_3); enter_mode(MOTOR_ID_4);

safe_stop_all();

lcd_set_cursor(1, 0); lcd_print("Ready. Wait St");

Timer state_timer, debug_timer, btn_timer;

State last_state = DISABLED;

int cyl_step = 0;

// Gripper State Variables

bool btn_last = 1;

bool gripper_moving = false;

int gripper_dir = 0; // 1=Out, -1=In

while(1) {

int st = read_active_low(sens_start);

int ms = read_active_low(sens_motor_start);

int sp = read_active_low(sens_motor_stop);

int field_raw = sens_field_select.read();

const char* field_str = (field_raw == 0) ? "RED" : "BLU";

// Read Limits

int grip_hit = read_active_low(lim_grip);

int release_hit = read_active_low(lim_release);

int btn_now = read_active_low(action_btn);

// Detect Main State Change

if (current_state != last_state) {

state_timer.reset();

state_timer.start();

last_state = current_state;

cyl_step = 0;

pc.printf("Main State: %d\r\n", current_state);

if(current_state != MOVE_BACK_1 && current_state != MOVE_BACK_2) {

safe_stop_all();

}

}

// --- GRIPPER MOTOR LOGIC (Independent of Main State) ---

// Runs continuously to allow gripping anytime

if (btn_now == 0 && btn_last == 1) { // Button Pressed (Edge Detect)

btn_timer.reset();

if (!gripper_moving) {

// Start Movement based on current position

if (grip_hit) {

// At Grip -> Move Out

gripper_dir = 1;

gripper_moving = true;

lcd_set_cursor(1, 0); lcd_print("Grip: Moving OUT");

} else if (release_hit) {

// At Release -> Move In

gripper_dir = -1;

gripper_moving = true;

lcd_set_cursor(1, 0); lcd_print("Grip: Moving IN ");

} else {

// Somewhere in middle -> Default to Out? Or toggle?

// Let's default to moving towards Release if not at limit

gripper_dir = 1;

gripper_moving = true;

}

} else {

// Already moving -> Stop (Toggle behavior)

gripper_moving = false;

set_gripper_motor(0, 0);

lcd_set_cursor(1, 0); lcd_print("Grip: STOPPED   ");

}

}

if (gripper_moving) {

if (gripper_dir == 1) {

// Moving Out (Release)

if (release_hit) {

gripper_moving = false;

set_gripper_motor(0, 0);

lcd_set_cursor(1, 0); lcd_print("Grip: AT RELEASE");

} else {

set_gripper_motor(1, MOVE_DUTY);

}

} else if (gripper_dir == -1) {

// Moving In (Grip)

if (grip_hit) {

gripper_moving = false;

set_gripper_motor(0, 0);

lcd_set_cursor(1, 0); lcd_print("Grip: AT HOME   ");

} else {

set_gripper_motor(-1, MOVE_DUTY);

}

}

}

btn_last = btn_now;

// End Gripper Logic

// --- MAIN STATE MACHINE ---

if (current_state == DISABLED) {

safe_stop_all();

cyl_stop_all();

if (st == 0) { // sens_start pressed

current_state = MOVE_BACK_1;

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("MOVE BACK 0.8m");

lcd_set_cursor(1, 0); lcd_print("Dist: 0.00/0.80m");

thread_sleep_for(100);

} else {

lcd_set_cursor(0, 0);

lcd_print("St:"); lcd_print(st ? "1" : "0");

lcd_print(" Ms:"); lcd_print(ms ? "1" : "0");

lcd_set_cursor(1, 0);

lcd_print("Fld:"); lcd_print(field_str); lcd_print(" READY   ");

static Timer blink;

if(blink.read() > 0.5f) { blink.reset(); led = !led; }

}

}

// 1. MOVE BACKWARD 0.8m

else if (current_state == MOVE_BACK_1) {

float t = state_timer.read();

float dist = t * LINEAR_SPEED;

if (t >= TIME_1) {

safe_stop_all();

wait_ms(300);

current_state = CYL1_CYCLE;

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("CYL 1: CYCLE");

lcd_set_cursor(1, 0); lcd_print("Starting...   ");

cyl_step = 0;

} else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[2][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);

send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);

send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();

lcd_set_cursor(1, 0);

lcd_print("Dist:"); lcd_print_num(dist, 2); lcd_print("/0.80m ");

}

led = 1;

}

thread_sleep_for(20);

}

// 2. CYLINDER 1 CYCLE

else if (current_state == CYL1_CYCLE) {

float t = state_timer.read();

if (cyl_step == 0) {

cyl1_ina = 1; cyl1_inb = 0; cyl1_pwm = 1.0f;

cyl2_ina = 0; cyl2_inb = 0; cyl2_pwm = 0.0f;

lcd_set_cursor(1, 0); lcd_print("C1 Extend     ");

if (t >= 2.0f) {

cyl_stop_all();

wait_ms(500);

current_state = MOVE_BACK_2; // Proceed to next move

state_timer.reset();

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("MOVE BACK 0.5m");

}

}

thread_sleep_for(50);

}

// 3. MOVE BACKWARD 0.5m

else if (current_state == MOVE_BACK_2) {

float t = state_timer.read();

float dist = t * LINEAR_SPEED;

if (t >= TIME_2) {

safe_stop_all();

wait_ms(300);

current_state = CYL2_CYCLE;

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("CYL 2: CYCLE");

lcd_set_cursor(1, 0); lcd_print("Starting...   ");

cyl_step = 0;

} else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[2][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100);

send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100);

send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) {

debug_timer.reset();

lcd_set_cursor(1, 0);

lcd_print("Dist:"); lcd_print_num(dist, 2); lcd_print("/0.50m ");

}

led = 1;

}

thread_sleep_for(20);

}

// 4. CYLINDER 2 CYCLE

else if (current_state == CYL2_CYCLE) {

float t = state_timer.read();

if (cyl_step == 0) {

cyl1_ina = 0; cyl1_inb = 0; cyl1_pwm = 0.0f;

cyl2_ina = 1; cyl2_inb = 0; cyl2_pwm = 1.0f;

lcd_set_cursor(1, 0); lcd_print("C2 Extend     ");

if (t >= 2.0f) { cyl_step = 1; state_timer.reset(); }

}

else if (cyl_step == 1) {

cyl2_ina = 0; cyl2_inb = 1; cyl2_pwm = 1.0f;

lcd_set_cursor(1, 0); lcd_print("C2 Retract    ");

if (t >= 2.0f) {

cyl_stop_all();

safe_stop_all();

wait_ms(500);

current_state = FINISHED;

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("SEQUENCE DONE!");

lcd_set_cursor(1, 0); lcd_print("All Complete  ");

led = 1;

}

}

thread_sleep_for(50);

}

else if (current_state == FINISHED) {

safe_stop_all();

cyl_stop_all();

thread_sleep_for(1000);

}

thread_sleep_for(20);

}

}

// ================= IMPLEMENTATIONS =================

void set_gripper_motor(int dir, float duty) {

if (duty > 1.0f) duty = 1.0f;

if (duty < 0.0f) duty = 0.0f;

if (dir == 1) {

// Forward (Out to Release)

dira = 0;

dirb = 1;

pwm = duty;

} else if (dir == -1) {

// Reverse (In to Grip)

dira = 1;

dirb = 0;

pwm = duty;

} else {

// Stop

dira = 0;

dirb = 0;

pwm = 0.0f;

}

}

void send_cmd(uint32_t id, float vel, float kp, float kd) {

if(vel < -30.0f) vel = -30.0f;

if(vel > 30.0f) vel = 30.0f;

int p_int = 2048;

int v_int = (int)((vel + 30.0f) * 4095.0f / 60.0f);

int kp_int = (int)(kp * 4095.0f / 500.0f);

int kd_int = (int)(kd * 4095.0f / 5.0f);

int ff_int = 2048;

tx_msg.data[0] = (p_int >> 8) & 0xFF;

tx_msg.data[1] = p_int & 0xFF;

tx_msg.data[2] = (v_int >> 4) & 0xFF;

tx_msg.data[3] = ((v_int & 0x0F) << 4) | ((kp_int >> 8) & 0x0F);

tx_msg.data[4] = kp_int & 0xFF;

tx_msg.data[5] = (kd_int >> 4) & 0xFF;

tx_msg.data[6] = ((kd_int & 0x0F) << 4) | ((ff_int >> 8) & 0x0F);

tx_msg.data[7] = ff_int & 0xFF;

tx_msg.id = id;

tx_msg.type = CANData;

tx_msg.format = CANStandard;

tx_msg.len = 8;

can.write(tx_msg);

}

void enter_mode(uint32_t id) {

tx_msg.id = id; tx_msg.type = CANData; tx_msg.format = CANStandard; tx_msg.len = 8;

for(int i = 0; i < 7; i++) tx_msg.data[i] = 0xFF;

tx_msg.data[7] = 0xFC;

can.write(tx_msg);

wait_ms(50);

}

void safe_stop_all() {

for(int k=0; k<3; k++) {

for(int i=0; i<NUM_MOTORS; i++) {

send_cmd(MOTOR_ID_1+i, 0.0f, 0.0f, 10.0f);

wait_us(200);

}

wait_ms(10);

}

}

void on_rx() {

if(can.read(rx_msg)) {

if(rx_msg.id == REPLY_ID_1 && rx_msg.len >= 6) rx_count[0]++;

else if(rx_msg.id == REPLY_ID_2 && rx_msg.len >= 6) rx_count[1]++;

else if(rx_msg.id == REPLY_ID_3 && rx_msg.len >= 6) rx_count[2]++;

else if(rx_msg.id == REPLY_ID_4 && rx_msg.len >= 6) rx_count[3]++;

}

}

void cyl_stop_all() {

cyl1_ina = 0; cyl1_inb = 0; cyl1_pwm = 0.0f;

cyl2_ina = 0; cyl2_inb = 0; cyl2_pwm = 0.0f;

}

int read_active_low(DigitalIn& s) {

return (s.read() == 0) ? 1 : 0;

}

// LCD Functions

void wait_us_lcd(int us) { thread_sleep_for(us / 1000); }

void write_i2c(uint8_t data) { char cmd = data | LCD_BACKLIGHT; i2c_lcd.write(lcd_addr << 1, &cmd, 1); }

void pulse_enable(uint8_t data) { write_i2c(data | LCD_EN); wait_us_lcd(100); write_i2c(data & ~LCD_EN); wait_us_lcd(100); }

void send_nibble(uint8_t nibble, uint8_t mode) { uint8_t data = (nibble & 0xF0) | mode | LCD_BACKLIGHT; pulse_enable(data); }

void send_command(uint8_t cmd) { send_nibble(cmd & 0xF0, 0); send_nibble((cmd << 4) & 0xF0, 0); thread_sleep_for(2); }

void send_char(char c) { send_nibble(c & 0xF0, LCD_RS); send_nibble((c << 4) & 0xF0, LCD_RS); }

void lcd_print(const char* str) { while (*str) send_char(*str++); }

void lcd_print_num(float num, int decimals) { char buf[16]; sprintf(buf, "%.*f", decimals, num); lcd_print(buf); }

void lcd_clear() { send_command(0x01); thread_sleep_for(2); }

void lcd_set_cursor(uint8_t row, uint8_t col) { uint8_t addr = (row == 0) ? 0x80 + col : 0xC0 + col; send_command(addr); }

bool detect_lcd() { char dummy = 0; if (i2c_lcd.write(LCD_ADDR_1 << 1, &dummy, 0) == 0) { lcd_addr = LCD_ADDR_1; return true; } if (i2c_lcd.write(LCD_ADDR_2 << 1, &dummy, 0) == 0) { lcd_addr = LCD_ADDR_2; return true; } return false; }

void lcd_init() { thread_sleep_for(50); write_i2c(0x03 | LCD_BACKLIGHT); wait_us_lcd(4500); pulse_enable(0x03 | LCD_BACKLIGHT); wait_us_lcd(4500); pulse_enable(0x03 | LCD_BACKLIGHT); wait_us_lcd(150); pulse_enable(0x02 | LCD_BACKLIGHT); send_command(0x28); send_command(0x0C); send_command(0x06); send_command(0x01); thread_sleep_for(2); }

// GIM8008 Quad Motor + Cylinders - Custom Ladder Sequence

// Sequence: M3+M4 UP -> CAN Fwd -> M3 DOWN -> CAN Fwd -> M4 DOWN -> FINISH

#include "mbed.h"

#include <cstdio>

#include <cstdlib>

#include <cmath>

#include "stm32f4xx_hal.h"

// ================= MOTOR & CAN CONFIG =================

#define MOTOR_ID_1      11

#define MOTOR_ID_2      12

#define MOTOR_ID_3      13

#define MOTOR_ID_4      14

#define CAN_BAUD        1000000

#define REPLY_ID_1      (MOTOR_ID_1 + 0x100)

#define REPLY_ID_2      (MOTOR_ID_2 + 0x100)

#define REPLY_ID_3      (MOTOR_ID_3 + 0x100)

#define REPLY_ID_4      (MOTOR_ID_4 + 0x100)

DigitalOut led(LED1);

Serial pc(USBTX, USBRX, 115200);

CAN can(PA_11, PA_12, CAN_BAUD);

// DC Motor Limit Sensors

DigitalIn lim_grip(PC_1, PullUp);

DigitalIn lim_release(PC_4, PullUp);

// LCD Config

#define I2C_SDA PB_9

#define I2C_SCL PB_8

I2C i2c_lcd(I2C_SDA, I2C_SCL);

uint8_t lcd_addr = 0x27;

#define LCD_BACKLIGHT 0x08

#define LCD_EN 0x04

#define LCD_RS 0x01

#define LCD_ADDR_1 0x27

#define LCD_ADDR_2 0x3F

// ============================================================================

// MOTOR 1: GRIP / BOX HANDLING (VNH5019)

// ============================================================================

PwmOut     m1_pwm(PB_14);

DigitalOut m1_dira(PC_8);

DigitalOut m1_dirb(PC_6);

// ============================================================================

// ORGOGCH MOTORS (CLIMBING M3, M4)

// ============================================================================

// M3

DigitalOut m3_ina(PA_8);

DigitalOut m3_inb(PB_10);

PwmOut     m3_pwm(PA_9);

// M4

DigitalOut m4_ina(PA_13);

DigitalOut m4_inb(PA_14);

PwmOut     m4_pwm(PB_2);

// --- CONFIGURATION ---

const float MAX_VEL = 3.0f;

const float LINEAR_SPEED = MAX_VEL;

const float DISTANCE_1 = 5.0f;

const float DISTANCE_2 = 2.0f;

const float DISTANCE_3 = 10.0f;

const float TIME_1 = DISTANCE_1 / LINEAR_SPEED;

const float TIME_2 = DISTANCE_2 / LINEAR_SPEED;

const float TIME_3 = DISTANCE_3 / LINEAR_SPEED;

const float TIME_4 = TIME_2 / 4;

// Ladder Configuration

const float LADDER_SPEED = 0.5f;

const float LADDER_TIME_20CM = 4.0f;   // Time to move 20cm

const float STABILIZE_TIME = 4.0f;     // Time to move forward

const float STABILIZE_VEL = 5.0f;

const float STABILIZE_BACK = 0.1f;

// Directions for M3/M4

const int DIR_UP_M3    = -1;

const int DIR_UP_M4    = 1;

const int DIR_DOWN_M3  = 1;

const int DIR_DOWN_M4  = -1;

const int NUM_MOTORS = 4;

const int NUM_PATTERNS = 6;

// ================= STATE ENUM =================

enum State {

DISABLED = 0,

MOVE_BACK_1 = 1,

CYL1_CYCLE = 2,

MOVE_BACK_2 = 3,

CYL2_CYCLE = 4,

MOVE_FOR_1 = 5,

MOVE_DC_TO_RELEASE = 6,

TURN_TO_LADDER = 7,

FIRST_LADDER = 8,

FINISHED = 9

};

volatile State current_state = DISABLED;

volatile int rx_count[4] = {0, 0, 0, 0};

CANMessage tx_msg, rx_msg;

float velocities[4] = {0.0f, 0.0f, 0.0f, 0.0f};

const float directions[NUM_PATTERNS][NUM_MOTORS] = {

{1.0f, -1.0f, -1.0f, 1.0f},   // 0:

{1.0f, 1.0f, -1.0f, -1.0f},   // 1: hoish

{-1.0f, 1.0f, 1.0f, -1.0f},   // 2:

{-1.0f, -1.0f, 1.0f, 1.0f},   // 3: Uragsh

{0.0f, 1.0f, 0.0f, -1.0f},    // 4: Diagonal

{-1.0f, -1.0f, -1.0f, -1.0f}  // 5: Turn Arc

};

// ================= SENSORS =================

DigitalIn sens_start(PC_11, PullUp);

DigitalIn sens_motor_start(PC_4, PullUp);

DigitalIn sens_field_select(PC_3, PullUp);

DigitalIn sens_motor_stop(PC_1, PullUp);

DigitalIn box_sensor_1(PA_5, PullUp);

DigitalIn box_sensor_2(PA_4, PullUp);

DigitalIn box_sensor_3(PB_1, PullUp);

// Cylinders

DigitalOut cyl1(PC_7);

DigitalOut cyl2(PB_6);

DigitalOut cyl3(PA_7);

DigitalOut cyl4(PA_6);

DigitalOut* relays[4] = {&cyl1, &cyl2, &cyl3, &cyl4};

const bool ACTIVE_LOW = false;

#define RELAY_ON  1

#define RELAY_OFF 0

// ================= FUNCTION PROTOTYPES =================

void send_cmd(uint32_t id, float vel, float kp = 0, float kd = 1.0f);

void enter_mode(uint32_t id);

void safe_stop_all();

void on_rx();

void cyl_stop_all();

int read_active_low(DigitalIn& s);

int is_pressed(DigitalIn& s);

void set_motor(int dir, float duty);

void stopAllClimb();

// Movement Functions

void moveM3M4Up();

void moveM3Down();

void moveM4Down();

void moveCANForward();

void moveCANBackward();

// LCD Helpers

void wait_us_lcd(int us);

void write_i2c(uint8_t data);

void pulse_enable(uint8_t data);

void send_nibble(uint8_t nibble, uint8_t mode);

void send_command(uint8_t cmd);

void send_char(char c);

void lcd_print(const char* str);

void lcd_print_num(float num, int decimals);

void lcd_clear();

void lcd_set_cursor(uint8_t row, uint8_t col);

bool detect_lcd();

void lcd_init();

// ================= MAIN =================

int main() {

__HAL_RCC_PWR_CLK_ENABLE();

HAL_PWR_EnableBkUpAccess();

__HAL_RCC_LSE_CONFIG(RCC_LSE_OFF);

while(__HAL_RCC_GET_FLAG(RCC_FLAG_LSERDY) != RESET);

// Init Relays Off

for (int i = 0; i < 4; i++) {

relays[i]->write(ACTIVE_LOW ? 1 : 0);

}

// Init Climbing Motors

m3_pwm.period_ms(2); m3_pwm = 0.0f;

m4_pwm.period_ms(2); m4_pwm = 0.0f;

stopAllClimb();

// Init Grip Motor

m1_pwm.period(0.0001f);

m1_pwm = 0.0f;

m1_dira = 0; m1_dirb = 0;

wait_ms(500);

pc.printf("\r\n=== System Boot ===\r\n");

pc.printf("Dir M3 Up: %d, Dir M4 Up: %d\r\n", DIR_UP_M3, DIR_UP_M4);

if (!detect_lcd()) {

led = 1;

while(true) { led = !led; thread_sleep_for(200); }

}

lcd_init();

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("ABU Robocon 2026");

lcd_set_cursor(1, 0); lcd_print("Init CAN...");

int freq = can.frequency(CAN_BAUD);

if(freq == 0) {

lcd_set_cursor(1, 0); lcd_print("CAN FAILED!   ");

while(1) { led = !led; thread_sleep_for(100); }

}

can.attach(&on_rx, CAN::RxIrq);

can.filter(0x100, 0x700, CANStandard, 0);

enter_mode(MOTOR_ID_1); enter_mode(MOTOR_ID_2);

enter_mode(MOTOR_ID_3); enter_mode(MOTOR_ID_4);

safe_stop_all();

lcd_set_cursor(1, 0); lcd_print("Ready. Wait St");

Timer state_timer, debug_timer;

State last_state = DISABLED;

int ladder_sub_step = 0;

while(1) {

int st = read_active_low(sens_start);

int field_raw = sens_field_select.read();

const char* field_str = (field_raw == 0) ? "RED" : "BLU";

if (current_state != last_state) {

state_timer.reset();

state_timer.start();

debug_timer.reset();

last_state = current_state;

pc.printf("State: %d\r\n", current_state);

if (current_state == FIRST_LADDER) {

ladder_sub_step = 0;

pc.printf("Starting Custom Ladder Sequence...\r\n");

}

if(current_state != MOVE_BACK_1 && current_state != MOVE_BACK_2 && current_state != MOVE_FOR_1 && current_state != TURN_TO_LADDER && current_state != FIRST_LADDER) {

safe_stop_all();

set_motor(0, 0);

stopAllClimb();

}

}

// ================= STATE MACHINE =================

if (current_state == DISABLED) {

safe_stop_all(); cyl_stop_all(); set_motor(0, 0); stopAllClimb();

if (read_active_low(sens_start) == 0) {

thread_sleep_for(50);

if(read_active_low(sens_start) == 0) {

current_state = MOVE_BACK_1;

lcd_clear(); lcd_set_cursor(0, 0); lcd_print("1. MOVE BACK 1");

}

} else {

lcd_set_cursor(0, 0); lcd_print("St:"); lcd_print(st ? "1" : "0");

lcd_set_cursor(1, 0); lcd_print("Fld:"); lcd_print(field_str); lcd_print(" READY");

static Timer blink;

if(blink.read() > 0.5f) { blink.reset(); led = !led; }

}

}

else if (current_state == MOVE_BACK_1) {

float t = state_timer.read();

if (t >= TIME_1) { safe_stop_all(); wait_ms(300); current_state = CYL1_CYCLE; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("2. CYL1 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (current_state == CYL1_CYCLE) {

cyl1.write(RELAY_ON); wait_ms(2000); cyl1.write(RELAY_OFF); wait_ms(300);

current_state = MOVE_BACK_2; thread_sleep_for(50);

}

else if (current_state == MOVE_BACK_2) {

float t = state_timer.read();

if (t >= TIME_2) { safe_stop_all(); wait_ms(300); current_state = CYL2_CYCLE; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("4. CYL2 EXT"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[3][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Moving Back..."); } led = 1;

}

thread_sleep_for(20);

}

else if (current_state == CYL2_CYCLE) {

cyl2.write(RELAY_ON); wait_ms(2000); cyl2.write(RELAY_OFF); wait_ms(300);

current_state = MOVE_FOR_1; thread_sleep_for(50);

}

else if (current_state == MOVE_FOR_1) {

float t = state_timer.read();

if (t >= 6.0) { safe_stop_all(); wait_ms(300); lcd_clear(); lcd_set_cursor(0, 0); lcd_print("6. GRIP CYCLE"); current_state = MOVE_DC_TO_RELEASE; }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[4][i];

for(int i=0; i<4; i++) { send_cmd(MOTOR_ID_1+i, velocities[i]); wait_us(100); }

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Turning..."); } led = 1;

}

thread_sleep_for(20);

}

else if (current_state == MOVE_DC_TO_RELEASE) {

cyl1.write(RELAY_ON); wait_ms(1000);

int release_hit = is_pressed(lim_release);

set_motor(-1, 0.3f);

lcd_set_cursor(0, 0); lcd_print("DC: Moving");

if (release_hit) {

m1_pwm = 0.0f; m1_dira = 1; m1_dirb = 1; wait_ms(50); m1_dira = 0; m1_dirb = 0; wait_ms(100);

current_state = TURN_TO_LADDER; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("RELEASE HIT");

}

wait_ms(10000);

if (state_timer.read() > 6.0f) { set_motor(0, 0); current_state = FINISHED; }

cyl1.write(RELAY_OFF); thread_sleep_for(50);

}

else if (current_state == TURN_TO_LADDER) {

float t = state_timer.read();

if (t >= TIME_2) { safe_stop_all(); current_state = FINISHED; lcd_clear(); lcd_set_cursor(0, 0); lcd_print("8. CLIMB START"); }

else {

for(int i=0; i<NUM_MOTORS; i++) velocities[i] = MAX_VEL * directions[0][i];

send_cmd(MOTOR_ID_1, velocities[0]); wait_us(100); send_cmd(MOTOR_ID_2, velocities[1]); wait_us(100);

send_cmd(MOTOR_ID_3, velocities[2]); wait_us(100); send_cmd(MOTOR_ID_4, velocities[3]); wait_us(100);

if(debug_timer.read() > 0.2f) { debug_timer.reset(); lcd_set_cursor(1, 0); lcd_print("Approaching..."); } led = 1;

}

thread_sleep_for(20);

}

// 9. FINISHED

else if (current_state == FINISHED) {

safe_stop_all();

cyl_stop_all();

set_motor(0, 0);

stopAllClimb();

led = 0;

lcd_clear();

lcd_set_cursor(0, 0); lcd_print("SEQUENCE DONE");

lcd_set_cursor(1, 0); lcd_print("STOPPED");

thread_sleep_for(100);

}

thread_sleep_for(20);

}

}

// ================= MOVEMENT FUNCTIONS =================

void cyl_stop_all() {

cyl1.write(RELAY_OFF); cyl2.write(RELAY_OFF); cyl3.write(RELAY_OFF); cyl4.write(RELAY_OFF);

}

void set_motor(int dir, float duty) {

if (duty > 1.0f) duty = 1.0f;

if (duty < 0.0f) duty = 0.0f;

if (dir == 1) { m1_dira = 0; m1_dirb = 1; m1_pwm = duty; }

else if (dir == -1) { m1_dira = 1; m1_dirb = 0; m1_pwm = duty; }

else { m1_dira = 0; m1_dirb = 0; m1_pwm = 0.0f; }

}

int read_active_low(DigitalIn& s) { return (s.read() == 0) ? 1 : 0; }

int is_pressed(DigitalIn& s) { return (s.read() == 0) ? 1 : 0; }

void send_cmd(uint32_t id, float vel, float kp, float kd) {

if(vel < -30.0f) vel = -30.0f; if(vel > 30.0f) vel = 30.0f;

int p_int = 2048;

int v_int = (int)((vel + 30.0f) * 4095.0f / 60.0f);

int kp_int = (int)(kp * 4095.0f / 500.0f);

int kd_int = (int)(kd * 4095.0f / 5.0f);

int ff_int = 2048;

tx_msg.data[0] = (p_int >> 8) & 0xFF; tx_msg.data[1] = p_int & 0xFF;

tx_msg.data[2] = (v_int >> 4) & 0xFF; tx_msg.data[3] = ((v_int & 0x0F) << 4) | ((kp_int >> 8) & 0x0F);

tx_msg.data[4] = kp_int & 0xFF; tx_msg.data[5] = (kd_int >> 4) & 0xFF;

tx_msg.data[6] = ((kd_int & 0x0F) << 4) | ((ff_int >> 8) & 0x0F); tx_msg.data[7] = ff_int & 0xFF;

tx_msg.id = id; tx_msg.type = CANData; tx_msg.format = CANStandard; tx_msg.len = 8;

can.write(tx_msg);

}

void enter_mode(uint32_t id) {

tx_msg.id = id; tx_msg.type = CANData; tx_msg.format = CANStandard; tx_msg.len = 8;

for(int i = 0; i < 7; i++) tx_msg.data[i] = 0xFF; tx_msg.data[7] = 0xFC;

can.write(tx_msg); wait_ms(50);

}

void safe_stop_all() {

for(int k=0; k<3; k++) {

for(int i=0; i<NUM_MOTORS; i++) {

send_cmd(MOTOR_ID_1+i, 0.0f, 0.0f, 10.0f); wait_us(200);

}

wait_ms(10);

}

}

void on_rx() {

if(can.read(rx_msg)) {

if(rx_msg.id == REPLY_ID_1 && rx_msg.len >= 6) rx_count[0]++;

else if(rx_msg.id == REPLY_ID_2 && rx_msg.len >= 6) rx_count[1]++;

else if(rx_msg.id == REPLY_ID_3 && rx_msg.len >= 6) rx_count[2]++;

else if(rx_msg.id == REPLY_ID_4 && rx_msg.len >= 6) rx_count[3]++;

}

}

// ================= LCD FUNCTIONS =================

void wait_us_lcd(int us) { thread_sleep_for(us / 1000); }

void write_i2c(uint8_t data) { char cmd = data | LCD_BACKLIGHT; i2c_lcd.write(lcd_addr << 1, &cmd, 1); }

void pulse_enable(uint8_t data) { write_i2c(data | LCD_EN); wait_us_lcd(100); write_i2c(data & ~LCD_EN); wait_us_lcd(100); }

void send_nibble(uint8_t nibble, uint8_t mode) { uint8_t data = (nibble & 0xF0) | mode | LCD_BACKLIGHT; pulse_enable(data); }

void send_command(uint8_t cmd) { send_nibble(cmd & 0xF0, 0); send_nibble((cmd << 4) & 0xF0, 0); thread_sleep_for(2); }

void send_char(char c) { send_nibble(c & 0xF0, LCD_RS); send_nibble((c << 4) & 0xF0, LCD_RS); }

void lcd_print(const char* str) { while (*str) send_char(*str++); }

void lcd_print_num(float num, int decimals) { char buf[16]; sprintf(buf, "%.*f", decimals, num); lcd_print(buf); }

void lcd_clear() { send_command(0x01); thread_sleep_for(2); }

void lcd_set_cursor(uint8_t row, uint8_t col) { uint8_t addr = (row == 0) ? 0x80 + col : 0xC0 + col; send_command(addr); }

bool detect_lcd() {

char dummy = 0;

if (i2c_lcd.write(LCD_ADDR_1 << 1, &dummy, 0) == 0) { lcd_addr = LCD_ADDR_1; return true; }

if (i2c_lcd.write(LCD_ADDR_2 << 1, &dummy, 0) == 0) { lcd_addr = LCD_ADDR_2; return true; }

return false;

}

void lcd_init() {

thread_sleep_for(50);

write_i2c(0x03 | LCD_BACKLIGHT); wait_us_lcd(4500);

pulse_enable(0x03 | LCD_BACKLIGHT); wait_us_lcd(4500);

pulse_enable(0x03 | LCD_BACKLIGHT); wait_us_lcd(150);

pulse_enable(0x02 | LCD_BACKLIGHT);

send_command(0x28); send_command(0x0C); send_command(0x06); send_command(0x01);

thread_sleep_for(2);

}